| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0219v4 |
| --- | --- |

| Title: | Changing cu_qp_delta parsing to enable CU-level processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution claims that the existing syntax definition for cu_qp_delta, in which QP delta transmission is delayed until the first CU with coefficients, inhibits CU-level decode processing.  It notes that a QP value is necessary for filtering operations, and therefore earlier CUs in the same CU quantization group cannot be filtered until cu_qp_delta is received.  It claims that this is the only syntax element in the entire standard which has this property.  It presents two possible solutions.  The first recommends changing the parsing syntax such that cu_qp_delta is always sent in the first CU of a quantization group, unless that CU occupies the entire group and has no coefficients.  The second changes the definition of QP within a quantization group.

# Introduction

HEVC in-picture QP adjustment is accomplished by sending a delta QP (cu_qp_delta) at a specified level of CU granularity.  The level of granularity defines a CU quantization group, and all CUs in that group share the same QP.  However, the delta QP value for the group is sent in the first CU that has coefficients, which may be many CUs away from the start of the group.  The lack of a QP value means that those earlier CUs cannot be processed to completion, because QP values are needed for in-loop filtering;  deblocking uses them for edge strength, and all filters use QP = 0 to disable filtering when lossless mode is enabled.

Consider the example LCU shown below.  It consists of 16 CUs, 15 of which are either skip CUs or CUs with no coefficients.  Because of this, the QP for these blocks cannot be determined until the 16th block is decoded.  A CU-pipelined decoder could not do any filtering on these earlier CUs.  In the worst case, 63 CUs can be waiting with unknown QP values until the delta QP is parsed!

Figure 1 - Delta QP Transmission in a large CU quantization group

This is a serious flaw in the standard, as it hobbles decoder architectures that process at the CU level. No other syntax element causes such a processing delay.  A preferable algorithm would allow for the QP value to always be known at the time a CU is parsed.  This contribution presents two ways of achieving that goal.

# Proposed Solution #1: Change Parsing Syntax

The first solution is to change the syntax such that the quantization group’s QP value is always determined in the first CU of the group.  .  A simple rule determines the need to insert delta QP.

Delta QP is inserted at the first CU if:

- CU size is less than the quantization group size OR

- CU has coefficients

This covers most of the cases where cu_qp_delta would not be sent with the existing rules, while allowing it to be included at the start of the quantization group when it is necessary.  The only case in which a new cu_qp_delta is sent under these rules (and would not have been sent previously) is when the entire group is split into smaller blocks, all of which have no coefficients.  This difference produces a small loss when the QP group size is small, as shown in section 4.

In HM 6 the delta QP is inserted at the transform unit level as soon as a CBF is seen. Here it is inserted in one of 2 places in the first CU of the group: at the start of the prediction unit syntax if the CU is smaller than the quantization group size (rule 1), or at the start of the transform tree syntax if the block has coefficients (rule 2).  The existing flag (IsCuQpDeltaCoded) is still used to ensure only one delta QP per quantization group.

Figure 2 - Necessary Parsing Changes

# Proposed Solution #2: Change the definition of CU quantization group

The alternative solution is to leave the parsing as is, and simply change the definition of the CU quantization group such that the delta QP only applies to the CU(s) containing the cu_qp_delta syntax element, and those that come after.  Any earlier CUs simply use the predicted QP for the group.  In the previous example, the initial 15 CUs would use the group’s predicted QP, and the last CU which contains coefficients would use the QP adjusted by cu_qp_delta.

Figure 3 - Different QPs within a quantization group

This means that there can be two different QPs inside a CU quantization group, as shown.  This method has the advantage of optimal coding efficiency, since cu_qp_delta is only sent when at least one of the sub-blocks has coefficients (as today).

Figure 2 shows how the QP quantization group works with this change.  The prediction method (average of up and left) is unchanged.  The only difference is that the QP of the neighboring CU is used, rather than the QP of the CU quantization group.

Figure 4 – QP Group Prediction (Colors represent different QP values)

Figure 1 also shows how QPs are assigned to blocks before and after the cu_delta_qp (two different colors).  The different QP applies only to the deblocking operation, because the light blue blocks don’t have any transform coefficients.

# Test Results for Solution #1

The common test conditions cannot be used for delta QP testing, since they specify a constant QP.  The test conditions of San Jose’s CE4 Subtest 1: QP Coding were used instead [2].    This test specified QP adaptation with the options –aq 1 –aqr12.  An LCU size of 64x64 was used, and two quantization group sizes were tested: 8x8 (MaxCuDQPDepth = 3), and 16x16 (MaxCuDQPDepth = 2).  The larger quantization groups insert delta QP so infrequently that the effect of any change is small.  The 8x8 quantization group produced essentially no change to the coding efficiency, since the number of delta QPs sent was identical.  Likewise, the results for all-intra sequences are unchanged.

The results for the 16x16 quantization group are:

|  | Random Access Main |  |  | Random Access HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |
| Class A | 0.4% | 0.5% | 0.3% | 0.2% | 0.4% | 0.0% |  |
| Class B | 0.3% | 0.2% | 0.2% | 0.3% | 0.5% | 0.1% |  |
| Class C | 0.3% | 0.3% | 0.3% | 0.2% | 0.1% | 0.2% |  |
| Class D | 0.3% | 0.0% | 0.3% | 0.2% | -0.1% | 0.1% |  |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.3% | 0.2% | 0.3% | 0.2% | 0.2% | 0.1% |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |  |
|  | Y | U | V | Y | U | V |  |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.4% | 0.2% | 0.4% | 0.3% | 0.2% | 0.4% |  |
| Class C | 0.4% | 0.3% | 0.5% | 0.2% | 0.2% | 0.3% |  |
| Class D | 0.3% | 0.1% | -0.2% | 0.4% | 0.2% | 0.6% |  |
| Class E | 0.5% | -0.4% | 0.3% | 0.1% | 0.0% | 1.5% |  |
| Overall | 0.4% | 0.1% | 0.2% | 0.3% | 0.2% | 0.6% |  |
| Enc Time[%] | 99% |  |  | 95% |  |  |  |
| Dec Time[%] | 98% |  |  | 91% |  |  |  |

# Conclusions

Either proposed solution fixes the delta QP problem.  The second solution represents a smaller change to the existing draft, but gives the encoder less control, and makes the definition of a quantization group somewhat confusing.  The first method does have a small amount of coding loss in some cases, but it gives the encoder the most control over all QP values, and makes lossless signaling easier to do.  This contribution strongly recommends solving this problem, either with one of these solutions or through some equivalent means.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003_dK, Feb. 2012.

- K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li, “CE4: Summary report of Core Experiment on Quantization”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0034, Feb. 2012

- K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li, “Description of Core Experiment 4: Quantization”, Document of Joint Collaborative Team on Video Coding, JCTVC-G1204, Nov. 2011

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes (solution #2)

## 7.3.4 – Slice Data Syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| CUQPDelta = 0 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |

## 7.3.5 – Coding Tree Syntax

| AlfCuFlagIdx++ |  |
| --- | --- |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CUQPDelta = 0 |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |

## 7.3.9 – Transform Unit

| if( (max_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) { |  |
| --- | --- |
| cu_qp_delta | ae(v) |
| CUQPDelta = cu_qp_delta |  |
| IsCuQpDeltaCoded = 1 |  |
| } |  |

## 7.4.9 – Transform coefficient semantics

cu_qp_delta specifies the difference between a luma quantization parameter for the coding unit containing cu_qp_delta and the prediction for the coding unit quantization group.    The coding unit quantization group is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CbSize is greater than or equal to log2MinCUDQPSize, the coding unit quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CbSize is equal to log2MinCUDQPSize, the coding unit quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta is inferred to be equal to 0 when it is not present for any coding unit quantization group.

The luma quantization parameter QP’Y and the chroma quantization parameters QP’Cb and QP’Cr are derived by invoking the process specified in subclause 8.4.1 for every coding unit with ( x0 ,y0 ) specifying the top-left luma sample of the current coding unit quantization group relative to the top-left luma sample of the current sample as inputs.

### Derivation process for quantization parameters

Inputs of this process are:

–	luma location ( xB, yB ) specifying the top-left luma sample of the current coding unit quantization group relative to the top-left luma sample of the current picture,

Outputs of this process are:

–	luma quantization parameter QP’Y for the coding unit,

–	chroma quantization parameters QP’Cb and QP’Cr for the coding unit

Let a variable QPY_A specifying the luma quantization parameter of the coding unit quantization group covering ( xB−1,  yB ), a variable tbAddrA specifying the treeblock address of treeblock containing the coding unit quantization group covering ( xB−1,  yB ), a variable QPY_B specifying the luma quantization parameter of the coding unit quantization group covering ( xB,  yB−1 ), a variable , tbAddrB specifying the treeblock address of treeblock containing the coding unit quantization covering ( xB,  yB−1 ), and a variable QPY_PREV specifying the luma quantization parameter of the previous coding unit quantization group in decoding order, respectively.

When at least one of the following conditions is true, QPY_PREV is initially set equal to SliceQPY at the start of each slice for the first coding unit quantization group in the slice.

–	A quantization group of coding units is the first quantization group of coding units in a slice.

–	A quantization group of coding units is the first quantization group of coding units in a tile.

–	A quantization group of coding units is the first quantization group of coding units in a treeblock row and tiles_or_entry_coding_sync_idc is equal to 2.

The prediction of luma quantization parameter QPY_PRED is derived as the following ordered steps:

- If the coding unit quantization group covering ( xB−1,  yB ) in the current slice is not available or tbAddrA is not equal to CurrTbAddr, then QPY_A is set equal to QPY_PREV.

- If the coding unit quantization group covering ( xB,  yB−1 ) in the current slice is not available or tbAddrB is not equal to CurrTbAddr, then QPY_B is set equal to QPY_PREV.

- The prediction of luma quantization paramaeter QPY_PRED is derived as:

QPY_PRED =  (QPY_A + QPY_B + 1) >> 1		(8-1)

The value of QPY is derived as

QPY = ( ( ( QPY_PRED + CUQPDelta cu_qp_delta +52+

2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY	(8-2)

The value of luma quantization parameter QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-3)

The values of chroma quantization parameters QP’Cb and QP’Cr are derived as

QP’Cb = Clip3( −QpBdOffsetC, 51, QPY + cb_qp_offset) + QpBdOffsetC	(8-4)

QP’Cr = Clip3( −QpBdOffsetC, 51, QPY + cr_qp_offset) + QpBdOffsetC	(8-5)

# Draft Text Changes (solution #1)

## 7.3.4 – Slice Data Syntax

}

IsCuQpDeltaCoded = 1

moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 )

CtbAddrTS++

## 7.3.7 – Prediction Unit Syntax

prediction_unit( … ) {

if (!IsCuQpDeltaCoded && log2CbSize < log2MinCUQPSize) {

cu_qp_delta

IsCuQpDeltaCoded = 1

}

if( skip_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else if( PredMode = = MODE_INTRA ) {

## 7.3.8 – Transform Tree Syntax

transform_tree( … ) {

if( … )

no_residual_data_flag

if (!IsCUQpDeltaCoded && !no_residual_data_flag) {

cu_qp_delta

IsCUQpDeltaCoded = 1

}

if( !no_residual_data_flag ) {

## 7.3.9 – Transform Unit Syntax

transform_unit( … ) {

if( … any cbf … ) {

if( (max_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded ) {

cu_qp_delta

IsCuQpDeltaCoded = 1

}

log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 )