| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0225_r2 |
| --- | --- |

| Title: | AHG6: Deblocking of IPCM-coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Semih Esenlik Thomas Wedi Steffen Kamp Panasonic R&D Center Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

HM6.0 allows to code blocks in the IPCM mode. For the deblocking of such IPCM coded blocks, HM6.0 assigns the fixed quantization parameter QP=0 to these blocks, which may result in degraded picture quality, e.g. in the case that IPCM coded blocks contain quantization noise. In order to overcome this problem, this proposal presents two solutions exploiting the knowledge of the encoder about the quantization noise of PCM coded blocks. Additional QP information is transmitted from the encoder to the decoder side to control the deblocking appropriately. The first solution assigns QP=qp_pcm to all PCM coded blocks of a slice. The parameter qp_pcm is coded in the slice header. The second solution follows the suggestion of the JCTVC-meeting in Geneva, November 2011. For each IPCM region, CABAC is terminated, the number n of successive PCM coded blocks is coded followed by n QP values using fixed length coding with the minimum number of bits needed for the QP range. It is proposed to adopt one of the two provided solutions

# Introduction

HM6.0 [2][1] allows to code blocks in the IPCM mode. For the deblocking of such IPCM coded blocks, HM6.0 assigns the fixed quantization parameter QP=0 to these blocks, whereas the QP controls the quantization step size used in the quantization process of the prediction error coefficients. This QP and the QP of the neighboring block are averaged. The averaged QP value is used to control the deblocking. In the case that IPCM coded blocks contain quantization noise, this deblocking control may result in degraded picture, e.g. in the case that IPCM coded blocks contain quantization noise. In order to overcome this problem, this proposal presents two solutions exploiting the knowledge of the encoder about the quantization noise of PCM coded blocks. Additional QP information is transmitted from the encoder to the decoder side to control the deblocking appropriately.

## Solution 1: Transmission of an additional slice-level QP for deblocking control of IPCM coded blocks

This first solution assigns QP=qp_pcm to all PCM coded blocks of a slice. The parameter qp_pcm is coded in the slice header. This has been proposed previously to the JCTVC-meeting in Geneva, November 2011 [3].

### Committee Draft modifications for solution 1

The required changes relative to the latest Committee Draft [1] are highlighted in yellow.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| … |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( pcm_enabled_flag ) { |  |
| qp_pcm | ue(v) |
| } |  |
| … |  |

qp_pcm specifies a luma quantization parameter used for deblocking of PCM coded blocks.

8.7.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on the pcm_flag of the coding unit containing the sample p0,0, a variable QPP is derived as follows:

–	If pcm_flag is equal to 0, the variable QPP is set equal to the value of QPY of the coding unit containing the sample p0,0.

–	Otherwise, the variable QPP is set equal to qp_pcm.

Depending on the pcm_flag of the coding unit containing the sample q0,0, a variable QPQ is derived as follows:

–	If pcm_flag is equal to 0, the variable QPQ is set equal to the value of QPY of the coding unit containing the sample q0,0.

–	Otherwise, the variable QPQ is set equal to qp_pcm.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

qPL = ( ( QPQ + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

The equivalent change is done in the sections

- 8.7.1.4.2: Filtering process for luma block edge

- 8.7.1.4.3: Filtering process for chroma block edge

## Solution 2:

The second solution follows the suggestion of the JCTVC-meeting in Geneva, November 2011 [4], which is also described in JCTVC-I0035 [5]. For each IPCM region, an individual QP value is coded and transmitted in the following way: CABAC is terminated, the number n of successive PCM coded blocks is coded followed by n QP values using fixed length coding with the minimum number of bits needed for the QP range. The only difference to JCTVC-I0035 is that in the fixed length coding of the QP values, each bit is inverted. This is done in order to avoid insertions of emulation_prevention_three_bytes in the case of successive fixed length coding of QP=0. In the CD, the negation of each bit is described by adding the negative QP-value to the maximum allowed value and coding of the resulting sum.

### Committee Draft modifications for solution 2

The required changes relative to the latest Committee Draft [1] are highlighted in yellow.

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| for( i = num_subsequent_pcm; i >= 0; i−− ) { |  |
| max_value_minus_cu_qp_minus_qp_bd_offset_y [ i ] | u(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_lc_flag[ x0 ][ y0 ] | ae(v) |
| } else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

B.2. Changes in 7.3.7.2 PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| QPY[ x0 ][ y0 ]= cu_qp_plus_qp_bd_offset_y [ NumPCMBlock − 1 ] − QpBdOffsetY |  |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 2; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock−− |  |
| } |  |

B.3. Addition of minus_cu_qp_plus_qp_bd_offset_y_plus_max_value semantics to 7.4.7 Prediction unit semantics

max_value_minus_cu_qp_minus_qp_bd_offset_y specifies the sum of the negative value of the luma quantization parameter, the negative value of QpBdOffsetY and the maximum value ((1<<Number of bits)-1). The value Number of bits is specified in Table X-xx.

The variable cu_qp_plus_qp_bd_offset_y specifying the luma quantization parameter plus QpBdOffsetY is derived

as follows:

cu_qp_plus_qp_bd_offset_y = (1<<Number of bits)-1- max_value_minus_cu_qp_minus_qp_bd_offset_y.

The decoded value of cu_qp_plus_qp_bd_offset_y shall be in the range of 0 to 51+ QpBdOffsetY, inclusive. The number of bits representing max_value_minus_cu_qp_minus_qp_bd_offset_y is specified in Table X-xx based on BitDepthY.

The luma quantization parameter QP’Y and the chroma quantization parameters QP’Cb and QP’Cr are derived by invoking the process specified in subclause Error: Reference source not found with ( x0 ,y0 ) as inputs.

- Table X-xx – Specification of the number of bits of cu_qp_plus_qp_bd_offset_y as a function of BitDepthY

| BitDepthY | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of bits | 6 | 6 | 6 | 7 | 7 | 7 | 7 |

## Software

Solution 1 and solution 2 have both been implemented into the software package of JCTVC-I0035, which is based on HM-6.0-dev rev. 2170. Software is provided together with this contribution.

## Experiments and results

The same experiments as described in JCTVC-I0035 have been conducted. For these experiments, the sequence Sandstorms is generated using the software provided together with JCTVC-E057 [6]. This sequence is coded using the RA-Main coder configuration [7], with QP values set to 2, 6, 10, and 14, and pcm_loop_filter_disable=0. In the following details about the source code configurations are given:

### Configuration for the anchor:

#define AHG06_IPCM                  1

#define IPCM_BUG_FIX                1

#define IPCM_DBF_USE_QPY_REC        0

#define IPCM_CODE_QPY               0

#define IPCM_SEND_ZERO_QP           0

#define PANASONIC_SLICE_LEVEL_QP_PCM 0

#define PANASONIC_BLOCK_LEVEL_QP_PCM 0

#define PANASONIC_SLICE_QP_PCM       0

### Configuration for solution 1

#define AHG06_IPCM                  1

#define IPCM_BUG_FIX                1

#define IPCM_DBF_USE_QPY_REC        0

#define IPCM_CODE_QPY               0

#define IPCM_SEND_ZERO_QP           0

#define PANASONIC_SLICE_LEVEL_QP_PCM 1

#define PANASONIC_BLOCK_LEVEL_QP_PCM 0

Two experiments are conducted for solution 1. In a first experiment, qp_pcm equal to zero is used:

#define PANASONIC_SLICE_QP_PCM       0

In a second experiment, qp_pcm equal to the slice QP is used:

#define PANASONIC_SLICE_QP_PCM       1

### Configuration for solution 2

#define AHG06_IPCM                  1

#define IPCM_BUG_FIX                1

#define IPCM_DBF_USE_QPY_REC        1

#define IPCM_CODE_QPY               1

#define PANASONIC_SLICE_LEVEL_QP_PCM 0

#define PANASONIC_BLOCK_LEVEL_QP_PCM 1

#define PANASONIC_SLICE_QP_PCM       0

Two experiments are conducted for solution 2. In a first experiment, qp_pcm equal to zero is used:

#define IPCM_SEND_ZERO_QP           1

In the second experiment, qp_pcm equal to the slice QP is used:

#define IPCM_SEND_ZERO_QP           0

### Results

The following table shows the results of the experiments described above. The 2nd column shows the average absolute bit rate of the transmitted QP values, the 3rd column the corresponding average relative data rate, the 4th, 5th, and 6th columns show the averaged difference in Y

It can be seen the relative data rate for the coding of the QP values is small for both solutions. It is 0% for solution1 and 0.04% for solution 2. PSNR differences occur in the case of using the Slice-QP for the deblocking of PCM coded blocks. They disappear in the case of using a QP equal to zero. No difference in run-times have been observed.

# Conclusion

For the deblocking of IPCM coded blocks, this proposal presents two solutions exploiting the knowledge of the encoder about the quantization noise of PCM coded blocks. Additional QP information is transmitted from the encoder to the decoder side to control the deblocking appropriately. The first solution assigns QP=qp_pcm to all PCM coded blocks of a slice. The parameter qp_pcm is coded in the slice header. The second solution follows the suggestion of the JCTVC-meeting in Geneva, November 2011 as well as the proposal JCTVC-I0035. QP values are coded for each IPCM block avoiding the insertion of emulation_prevention_three_bytes. The relative data rate for the coding of the QP values is small for both solutions. It is proposed to adopt one of the two provided solutions.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, San José, February, 2012.

- HM6, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0.

- A. Kotra, M. Narroschke, T. Wedi, “Deblocking bug fix for CU-Varying QP’s and IPCM blocks”, JCTVC-G640, Geneva, November 2011.

- Meeting notes of the JCTVC-meeting in Geneva, February 2012, http://wftp3.itu.int/av-arch/jctvc-site/2012_02_H_SanJose/JCTVC-H_Notes_dE.doc.

- K. Chono, G. Van der Auwera, M. Karczewicz, „AHG6: Report on unified QP derivation process in deblocking of I_PCM regions“, JCTVC-I0035, Geneva, February 2012.

- K. Chono, H. Aoki, Y. Senda, “Pulse code modulation mode for HEVC”, JCTVC-E057, Geneva, March, 2011.

- F. Bossen, “Common test conditions”, JCTC-H1100, San José, February, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).