| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0230 adopted text |
| --- | --- |

| Title: | Parameter sets modifications for temporal scalability and extension hooks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ, 07601 USA | Tel: Email: | +1 201 478 6145 {jill, danny, wonkap} @vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Adopted syntax changes to the base specification

## NAL unit type

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 15 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 16-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

## Video Parameter Set

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies a video parameter set. The value of view_parameter_set_id shall be in the range of 0 to 15, inclusive.

vps_max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream.

vps_max_layers_minus1 plus 1 specifies the maximum number of layers that may be present in the coded video sequence.  vps_num_layers_minus1 shall be equal to 0.

vps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on vps_temporal_id_nesting_flag, the following applies.

–	If vps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (vps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB and tIdB  less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC and tIdC less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM > tIdN, is always possible.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

- vps_max_dec_pic_buffering[ i ] specifies the required size of the HRD decoded picture buffer (DPB) in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, vps_max_dec_pic_buffering[ i ] ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of vps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive.

- vps_num_reorder_pics[ i ] indicates the maximum number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The value of vps_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_dec_pic_buffering[ i ], inclusive.

vps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by MaxLatencyPictures[ i ] = vps_num_reorder_pics[ i ] + vps_max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. When vps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. When vps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of vps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive.

## Sequence Parameter Set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |

| pcm_bit_depth_luma_minus1 | u(4) |
| --- | --- |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| non_square_quadtree_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| … |  |

video_parameter_set_id identifies the video parameter set that is referred to by the sequence parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_max_temporal_layers_minus1 + 1 specifies the maximum number of temporal layers present in the sequence. The value of sps_max_temporal_layers_minus1 shall be in the range of 0 to 7, inclusive.

sps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on sps_temporal_id_nesting_flag, the following applies.

–	If sps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (sps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB and tIdB  less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC and tIdC less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM > tIdN, is always possible.

- sps_max_dec_pic_buffering[ i ] specifies the required size of the HRD decoded picture buffer (DPB) in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, sps_max_dec_pic_buffering[ i ] ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive. sps_max_dec_pic_buffering[ i ] shall be equal to vps_max_dec_pic_buffering[ i ].

- sps_num_reorder_pics[ i ] indicates the maximum number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The value of sps_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering[ i ], inclusive. sps_num_reorder_pics[ i ] shall be equal to vps_num_reorder_pics[ i ].

sps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by MaxLatencyPictures[ i ] = sps_num_reorder_pics[ i ] + sps_max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. When sps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. When sps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of sps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive. sps_max_latency_increase[ i ] shall be equal to vps_max_latency_increase[ i ].

Video Parameter Set activation

##### 7.4.1.2.1 Order of sequence, picture and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of picture, sequence, video and adaptation parameter sets for coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found.

- NOTE 1 – The sequence, picture and adaptation parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence, picture and adaptation parameter sets may, in some applications, be conveyed "out-of-band".

- An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one adaptation parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular adaptation parameter set RBSP results in the deactivation of the previously-active adaptation parameter set RBSP (if any).

- When an adaptation parameter set RBSP (with a particular value of aps_id) is not active and it is referred to by a coded slice NAL unit (using that value of aps_id), it is activated. This adaptation parameter set RBSP is called the active adaptation parameter set RBSP until it is deactivated by the activation of another adaptation parameter set RBSP. An adaptation parameter set RBSP, with that particular value of aps_id, shall be available to the decoding process prior to its activation.

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units or coded slice data partition A NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP results in the deactivation of the previously-active picture parameter set RBSP (if any).

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active and it is referred to by a coded slice NAL unit or coded slice data partition A NAL unit (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated by the activation of another picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id and contained within an access unit with temporal_id equal to 0, shall be available to the decoding process prior to its activation. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

- NOTE 3 – If picture parameter set RBSP, sequence parameter set RBSP, video parameter set RBSP, or adaptation parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the picture parameter set RBSP or sequence parameter set RBSP, respectively. Otherwise (picture parameter set RBSP, sequence parameter set RBSP, video parameter set RBSP, or adaptation parameter set RBSP are conveyed by other means not specified in this Recommendation | International Standard), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

- When a video parameter set RBSP (with a particular value of video_parameter_set_id) is not already active and it is referred to by activation of a sequence parameter set RBSP (using that value of video_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message, it is activated. This video parameter set RBSP is called the active video parameter set RBSP until it is deactivated by the activation of another video parameter set RBSP. A video parameter set RBSP, with that particular value of video_parameter_set_id and contained within an access unit with temporal_id equal to 0, shall be available to the decoding process prior to its activation. An activated video parameter set RBSP shall remain active for the entire coded video sequence.

- Any video parameter set NAL unit containing the value of video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

All constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in sequence parameter sets, picture parameter sets, video parameter sets, and adaptation parameter sets and other syntax elements are expressions of constraints that apply only to the active sequence parameter set, the video parameter set, the active picture parameter set and the active adaptation parameter set. If any sequence parameter set RBSP or video parameter set RBSP is present that is not activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any picture parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any adaptation parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

- During operation of the decoding process (see clause Error: Reference source not found), the values of parameters of the active picture parameter set and the active sequence parameter set shall be considered in effect. For interpretation of SEI messages, the values of the parameters of the picture parameter set,sequence parameter set, and video parameter set that are active for the operation of the decoding process for the VCL NAL units of the coded picture in the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).