| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0230v3 |
| --- | --- |

| Title: | Parameter sets modifications for temporal scalability and extension hooks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ, 07601 USA | Tel: Email: | +1 201 478 6145 {jill, danny, wonkap} @vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

Changes are proposed to parameter set syntax in the base specification, related to support of temporal scalability, and for hooks to enable future extensions.  The addition of a Video Parameter Set (VPS) is proposed, which includes temporal scalability related parameters applicable to the entire sequence, as well as hooks for future extensions.  A modification to the Sequence Parameter Set (SPS) is also proposed, including the addition of a vps_id, and moving of the temporal scalability related syntax elements to the VPS.

In this v3 revision, an error in the syntax table for the proposed VPS extension is corrected, and based on discussion in the initial presentation, also propose moving max_dec_pic_buffering[ i ], num_reorder_pics[ i ], max_latency_increase[ i ] from the SPS to the VPS.

# Proposal

As proposed in JCTVC-H0388, a Video Parameter Set (VPS) can contain parameters applicable to an entire coded video sequence.  We propose inclusion of a VPS in the base specification, for better support of future extensibility.  The burden on a non-scalable codec of supporting a VPS is very small.

We propose that the temporal scalability related syntax elements currently in the SPS, max_temporal_layers_minus1, temporal_id_nesting_flag, max_dec_pic_buffering[ i ], num_reorder_pics[ i ], max_latency_increase[ i ] be moved to the VPS in the base specification.

The semantics of temporal_id require that all NAL units in an access unit have the same value of temporal_id.

“temporal_id specifies a temporal identifier for the NAL unit. The value of temporal_id shall be the same for all NAL units of an access unit.”

In future scalable and multiview extensions, that same restriction will apply to all layers/views, as it did in MVC and SVC.  Hence, it is more appropriate that temporal scalability information reside in the VPS than in the SPS, as all layers/views in a coded video sequence refer to the same VPS, while separate layers/views may refer to a different SPS.

In SVC, the temporal_id_nesting_flag is in the Scalability Information SEI message, which applies to the entire coded video sequence.  The Scalability Information SEI message also contains information about the number of temporal layers.

Moving the temporal scalability parameters to the VPS avoids the need for duplication of these parameters in each SPS, and aids a middle box performing bitstream extraction in having access to the temporal scalability information earlier.  Contribution JCTVC-I0231 which proposes an SEI message containing sub-bitstream profile & level information also benefits from temporal scalability parameters being present in the VPS rather than each SPS.

In addition to the temporal scalability parameters, we propose that the VPS in the base spec include a fixed length max_layers_minus1 syntax element, which will be restricted to be equal to 0 in the base spec.  We also propose that some of the VUI parameters be moved from the SPS to the VPS, but postpone the decision on which parameters until the VUI design is more fully explored.  Additionally an extension flag is proposed.

Section 2 contains the syntax of the proposed changes to the base specification.  Section 3 contains information of envisioned future definition of the VPS syntax for the scalability and multiview extensions.

# Proposed syntax changes to the base specification

## Video Parameter Set

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| max_temporal_layers_minus1 | u(3) |
| max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies a video parameter set. The value of view_parameter_set_id shall be in the range of 0 to 7, inclusive.

max_temporal_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream.

max_layers_minus1 plus 1 specifies the maximum number of layers that may be present in the coded video sequence.  num_layers_minus1 shall be equal to 0. [Ed. This could be renamed reserved_zero_5bits.]

temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on temporal_id_nesting_flag, the following applies.

–	If temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB and tIdB  less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC and tIdC less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM > tIdN, is always possible.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

## Sequence Parameter Set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |

| pcm_bit_depth_luma_minus1 | u(4) |
| --- | --- |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| non_square_quadtree_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| … |  |

video_parameter_set_id identifies the video parameter set that is referred to by the sequence parameter set. The value of video_parameter_set_id shall be in the range of 0 to 7, inclusive.

# Information: envisioned syntax for extensions

For informational purposes, envisioned modifications to the NAL unit header and VPS for future scalability and multiview extensions are described.

## NAL unit header

We propose renaming the reserved_one_5bits syntax element to be layer_id_plus1.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 1 |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| layer_id_plus1 | u(5) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

## Video Parameter Set

Similar to the proposal in JCTVC-H0386, each view and/or spatial or SNR scalable layer is considered to be a layer.  Each layer refers separately to an SPS.  Temporal layers are not considered to be layers, instead there are one or more temporal sub-layers contained within a layer.  Each layer is uniquely identified by its layer_id.  In the VPS, for each possible layer_id value, values are specified of dependency_id, quality_id, view_id, and depth_flag for the layer.

The VPS provides information about the dependency relationships of the layers in a bitstream, as well as other information applicable to all slices across all (scalability or view) layers in the entire coded video sequence.  In MVC, the view dependency information was included in the SPS, and repeated for each view.  Because this information is applicable to more than one view/layer, it is more appropriate that it be included in the VPS rather than the SPS, and is more bitrate efficient because duplication is avoided.

The VPS also contains some of the information that was present in the SVC Scalability Information SEI message, but unlike the SEI message, the VPS is normative and required.

For each layer_id value, the VPS also contains a dependent_flag.  If the value is equal to 1, the directly dependent reference layer is signaled using delta_reference_layer_id_plus1.  A variable ReferenceLayer_id is calculated to indicate the directly dependent reference layer.

At most one VPS may be active at a time.  A VPS is activated when it is not already active and it is referred by a coded slice NAL unit in a particular layer in an IDR access unit being decoded, though through the active PPS for that particular layer, which references an SPS.  The VPS must be available before the NAL unit that activates the VPS.  Once activated, the VPS applies to the entire coded video sequence.  In other words, the active VPS can only change at an IDR access unit.

Changing the contents of the VPS frequently is not recommended.  A redefined access unit delimiter is used to change which layers are active without requiring a change in the contents of the VPS or the use of IDR access units.  The VPS itself contains a maximum number of layers and layer dependencies for the sequence.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| max_temporal_layers_minus1 | u(3) |
| max_layers_minus1 | u(5) |
| temporal_id_nesting_flag | u(1) |
| video_parameter_set_id | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| bit_equal_to_one /* equal to 1 */ | u(1) |
| if( max_layers_minus1 > 0) { |  |
| extension_type | ue(v) |
| for( i = 1; i < max_layers_minus1; i++ ) { |  |
| dependent_flag[ i ] | u(1) |
| if ( dependent_flag[ i ] ) { |  |
| delta_reference_layer_id_minus1[ i ] | ue(v) |
| if( extension_type = = 0 ¦ ¦ extension_type = = 2 ) { |  |
| dependency_id[ i ] | ue(v) |
| quality_id[ i ] | ue(v) |
| } |  |
| if( extension_type = = 1 ¦ ¦ extension_type = = 2 ) { |  |
| view_id[ i ] | ue(v) |
| depth_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

extention_type equal to 0 specifies that multiple dependency and/or quality layers may be present in the bitstream.  extension_type equal to 1 specifies that multiple view layers may be present in the bitstream. extension_type equal to 2 specifies that multiple dependency and/or quality layers as well as multiple view layers may be present in the bitstream. The value of extension_type shall be in the range of 0 to 2, inclusive.

dependent_flag[ i ] equal to 0 specifies that the layer representation with layer_id equal to i is an independent layer, which can be decoded independently of other layers.  dependent_flag[ i ] equal to 1 specifies that the layer with layer_id equal to i is dependent on a reference layer.  dependent_flag[ 0 ] shall be inferred to be 0.

delta_reference_layer_id_plus1[ i ] specifies the reference layer that that the layer representation with layer_id equal to i is directly dependent upon. The value of delta_reference_layer_id[ i ] shall be in the range of 0 to i-1, inclusive.

ReferenceLayer_id[ i ] = i −1 − delta_reference_layer_id_plus1[ i ].

dependency_id[ i ] specifies a dependency identifier for the layer with layer_id equal to i.  dependency_id[ i ] shall be in the range of 0 to X inclusive.  When not present, dependency_id[ i ] shall be inferred to be 0.  When num_directly_dependent_layers[ i ] is greater than 0, dependency_id[ i ] shall be equal to or greater than the dependency identifier of any layer the layer with layer_id equal to i depends on.

quality_id[ i ] specifies a equality identifier for the layer with layer_id equal to i. quality_id[ i ] shall be in the range of 0 to X inclusive. When not present, quality_id[ i ] shall be inferred to be 0. When num_directly_dependent_layers[ i ] is greater than 0, quality_id[ i ] shall be equal to or greater than the quality identifier of any layer that the layer with layer_id equal to i depends on and that has dependency identifier equal to dependency_id[ i ].

view_id[ i ] specifies a view identifier for the layer with layer_id equal to i. view_id[ i ] shall be in the range of 0 to X inclusive. When not present, view_id[ i ] shall be inferred to be 0.

depth_flag[ i ] equal to 1 specifies that the layer with layer_id equal to i represents a depth map.  depth_flag[ i ] equal to 0 specifies that the layer with layer_id equal to i represents texture. When not present, depth_flag[ i ] shall be inferred to be 0.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).