| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0231 |
| --- | --- |

| Title: | SEI message for sub-bitstream profile & level indicators |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ, 07601 USA | Tel: Email: | +1 201 478 6145 {jill, danny, wonkap} @vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

An SEI message is proposed for the base HEVC specification, to optionally indicate sub-bitstream profiles and levels.  The proposed SEI message applies to both temporal-sub layers and to scalability or multiview layers.   For the future scalable and multiview extensions, a re-definition of the maximum pixel throughput limit level constraint is proposed, such that the constraint applies only to the individual layer, and not the full sub-bitstream corresponding to that target layer_id, enabling the same SPS to be referred to by multiple layers.

# Proposed SEI message for base specification

A sub-bitstream extraction process is defined in the HEVC design for an input target temporal_id value, outputting a sub-bitstream that is itself a compliant bitstream.  The current design does not provide means for indicating the profile and level of the sub-bitstream separately from the full bitstream.

An SEI message is proposed that allows optional indication of the profile and level indication for each possible sub-bitstream.  The proposed syntax is applicable to both temporal sub-layers and future scalability/multiview layers.  The syntax could be restricted to consider only temporal scalability, reflecting the base specification design, but that would complicate the syntax of the SEI message for the future extensions.  In SVC, the scalability info SEI message enabled optional indication of profile and level for the sub-bitstream associated with each target layer.  The MVC view scalability info SEI message similarly allowed optional indication of profile and level for each operation point.

The proposed SEI message loops through the possible sub-bitstreams, corresponding to particular target values of temporal_id and layer_id, and provides separate flags indicating the presence of profile info and level info.  Assuming that the level is more likely to change for temporal sub-layers than the profile, having separate flags for profile and level are more efficient.  The profile presence flag indicates the presence of both the sub-bitstream profile and reserved_zero_8bits, which is expected to be used for some sort of constraint set.  The level presence flag indicates the presence of the sub-bitstream level.    Proposed syntax of the SEI message is below.

If the target temporal_id value is equal to the max_temporal_layers_minus1 and the bitstream contains only a single spatial/multiview layer, the extracted sub-bitstream is identical to the full bitstream.  There is no requirement that the profile and level for the sub-bitstream indicated in this SEI message be identical to the profile_idc and level_idc in the SPS.  Rather, the bitstream must not violate either of the two constraints.

## Proposed syntax and semantics

| sub_bitstream_profile_level_info( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | ue(v) |
| for ( i = 0; i <= max_layers_minus1; i++ ) { |  |
| for ( j = 0; j <= max_temporal_layers_minus1; j ++ ) { |  |
| sub_bitstream_profile_info_present_flag[ i ][ j ] | u(1) |
| if (sub_bitstream_profile_info_present_flag[ i ][ j ]) { |  |
| sub_bitstream_profile_idc[ i ][ j ] | u(8) |
| sub_bitstream_reserved_zero_8bits[ i ][ j ] /* equal to 0 */ | u(8) |
| } |  |
| sub_bitstream_level_info_present_flag[ i ][ j ] | u(1) |
| if (sub_bitstream_level_info_present_flag[ i ][ j ]) |  |
| sub_bitstream_level_idc[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |
| } |  |

video_parameter_set_id identifies a video parameter set. The value of view_parameter_set_id shall be in the range of 0 to 7, inclusive.

sub_bitstream_profile_info_present_flag[ i ][ j ] equal to 0 specifies the syntax elements sub_bitstream_profile_idc[ i ][ j ] and sub_bitstream_reserved_zero_8bits[ i ][ j ]  are not present in the bitstream,  equal to 1 specifies they are present.

sub_bitstream_level_info_present_flag[ i ][ j ] equal to 0 specifies the syntax elements sub_bitstream_level_idc[ i ][ j ] is not present in the bitstream,  equal to 1 specifies it is present.

sub_bitstream_profile_idc[ i ][ j ] and sub_bitstream_level_idc[ i ][ j ]  indicates the profile and level to which the sub-bitstream extracted with target layer i and target temporal layer j conforms.

## SVC and MVC comparison

The SVC scalability information SEI message provides information about the number of layers present in the coded video sequence, and loops through those layers to provide a mapping of a layer id value to the priority, dependency, quality and temporal id values, as well as many other parameters describing each layer, some of which are optional, such as profile and level information, and average bitrate.  The profile and level information in the SSEI message indicate decoding capability for the sub-bitstream associated with the target layer representation identified by the layer_id value.  The level limits are interpreted in the same manner as if the same level indicator value were included in a sequence parameter set.

Similarly, the MVC view scalability information SEI message provides information about the number of views present in the coded video sequence, and optionally provides profile and level information for the sub-bitstream associated with a target view representation.

The relevant portions of the two SEI messages are copied below.

| scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| temporal_id_nesting_flag | 5 | u(1) |
| priority_layer_info_present_flag | 5 | u(1) |
| priority_id_setting_flag | 5 | u(1) |
| num_layers_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_layers_minus1; i++ ) { |  |  |
| layer_id[ i ] | 5 | ue(v) |
| priority_id[ i ] | 5 | u(6) |
| discardable_flag[ i ] | 5 | u(1) |
| dependency_id[ i ] | 5 | u(3) |
| quality_id[ i ] | 5 | u(4) |
| temporal_id[ i ] | 5 | u(3) |
| … |  |  |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| … |  |  |
| if( profile_level_info_present_flag[ i ] ) |  |  |
| layer_profile_level_idc[ i ] | 5 | u(24) |
| if( bitrate_info_present_flag[ i ] ) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate_layer[ i ] | 5 | u(16) |
| max_bitrate_layer_representation[ i ] | 5 | u(16) |
| max_bitrate_calc_window[ i ] | 5 | u(16) |
| … |  |  |

| view_scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_operation_points_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_operation_points_minus1; i++ ) { |  |  |
| operation_point_id[ i ] | 5 | ue(v) |
| priority_id[ i ] | 5 | u(5) |
| temporal_id[ i ] | 5 | u(3) |
| num_target_output_views_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_target_output_views_minus1[ i ]; j++ ) |  |  |
| view_id[ i ][ j ] | 5 | ue(v) |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| bitrate_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| if( !num_target_output_views_minus1[ i ] ) |  |  |
| view_dependency_info_present_flag[ i ] | 5 | u(1) |
| parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| bitstream_restriction_info_present_flag[ i ] | 5 | u(1) |
| if ( profile_level_info_present_flag[ i ] ) |  |  |
| op_profile_level_idc[ i ] | 5 | u(24) |
| if( bitrate_info_present_flag[ i ] ) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate[ i ] | 5 | u(16) |
| max_bitrate_calc_window[ i ] | 5 | u(16) |
| } |  |  |
| … |  |  |

## Interaction with contribution JCTVC-I0230

The proposed SEI message syntax assumes that temporal scalability information is present in the VPS rather than the SPS, as proposed in contribution JCTVC-I0230.  This modification is unnecessary for single layer HEVC, but provides advantages for the future scalability and multiview extensions, as it requires only a single vps_id, and not multiple sps_id’s in the SEI message, in order to access the relevant values of max_layers_minus1 and max_temporal_layers_minus1.  It also allows the proposed SEI message to be present in the bitstream in advance of the SPS’s of individual layers, although following the VPS.

# Information for extensions: proposed level constraint redefinition

It is proposed to redefine the level constraint on maximum pixel throughput for the scalable and multiview extension, such that only the individual layer is constrained, and not its reference layers.  No action is required for the base specification.

## Background on base AVC, SVC, MVC, and HEVC level constraints

Background on the level constraints for base AVC, SVC, MVC, and HEVC are described, to provide context for the proposal.

For the single layer AVC profiles, including those used as an SVC base layer or MVC base view, assuming a fixed frame rate, FrameRate, the maximum MB throughput is restricted such that level limit simplifies to MaxMBPS >=  PicSizeInMbs * FrameRate.  The description of the constraint of the level limit does not assume a fixed frame rate, and is expressed as a limit on the minimum output time between frames, which simplifies to the above equation for fixed frame rates.

For the profiles associated with SVC and MVC, where multiple scalable layers or views are to be decoded, the interpretation of the max MB throughput is modified, based upon the number of layers or views.

In the SVC extension, in a subset sequence parameter set  for a non-base layer, the level limit expresses a constraint of the maximum MB throughput MaxMBPS >= svcPicSizeInMbs * FrameRate, where the value of svcPicSizeInMbs is based on the number of layers and the picture size of the active layer and its reference layers, with the assumption of a fixed frame rate.

In the MVC extension, in a subset sequence parameter set  for a non-base view, the level limit expresses a constraint of the maximum MB throughput MaxMBPS >=  (NumViews / 2) * PicSizeInMbs  * FrameRate, where NumViews refers to the number of views required for decoding the target output view, with the assumption of a fixed frame rate.

In MVC, because the MB throughput limit is based upon the number of views, multiple view layers whose SPS parameter values are otherwise identical, e.g. with the same image resolution and VUI data, differ in their level indicator value.   This means that they must refer to different SPS ids, and separate PPS ids.  This is inefficient in that multiple nearly identical sequence and picture parameter sets must be transmitted, plus an increase in the number of bits in each slice header to refer different picture parameter set id values, which in turn refer to the sequence parameter set id.

Similar to single layer AVC, the HEVC design has profile and level indicator syntax elements in the SPS.  The level limits are based directly upon the pixel rates rather than the MB rates, but otherwise are similar.  Table 1 shows the maximum pixel rate and picture size for each allowable level in the current HEVC design.  Again assuming a fixed frame rate, there is a restriction on pixel throughput, such that the level limit pixel throughput simplifies to MaxLumaPR >=  PicSizeLuma * FrameRate.

## Proposal for HEVC extension

Unlike the level limit constraint on macroblock throughput for SVC and MVC, which is calculated considering the number of dependent layers or views, we propose that the pixel throughput limitation for the HEVC extension specifies a constraint only on the individual layer.  This per layer level indicator in the SPS can be renamed the layer level indicator to clarify that it applies only to the individual layer.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| layer_level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| … |  |

layer_level_idc indicates the level to which layer conforms.

The proposed constraint definition will improve coding efficiency vs. the MVC and SVC methods by reducing the number of SPSs required to be transmitted.  Additionally, the number of bits can be reduced for the signaling of the PPS identifier in the coded slice header, because fewer different SPS id values must be signaled, and hence fewer different PPS id values.  The PPS value in the slice header is variable length entropy coded, such that higher values require more bits than smaller values.   Sub-bitstream level indicators which apply additional constraints to a sub-bitstream rather to an individual layer are optionally sent in the proposed sub-bitstream profile level information SEI message.

The HEVC table which maps each allowable level id value to a pixel throughput limit, will be applicable to both layer levels specified in the SPS, and to sub-bitstream levels specified in the proposed SEI message.   The interpretation of the pixel throughput level limit constraint imposed upon a compliant bitstream through the layer level indicator in the SPS will not be based upon the requirements for decoding the sub-bitstream associated with the target layer, but instead be based upon processing the particular individual layer.  Assuming a fixed frame rate, FrameRate, for simplicity, the constraint on the maximum pixel throughput layer level limit simplifies to MaxLumaPR >=  PicSizeLuma * FrameRate , both for the base layer and for all non-base layers.

A decoder or MANE processing a scalable or multview bitstream may check its capability to process a coded bitstream by considering the level limit constraint for each layer individually, and also considering the number of dependent layers required to decode the bitstream.  The number of dependent layers may be determined by information in the VPS.

## Example

Table 2 provides an example for a 3-view multiview bitstream, with 3 temporal sub-layers.  Each layer has a resolution of 1920x1080.  Temporal layer 0 is 15 fps, temporal layer 1 is 30 fps, and temporal layer 2 is 60 fps, for all 3 layers.   The pixel rate for each individual layer is calculated in the table.   Considering the HEVC constraints of Table 1, the level limit for each layer is calculated, which because they are of the same resolution, have the same level, which is signaled in the SPS.  Layers 1 and 2 may refer to the same SPS.  Additional columns show the pixel rate for the sub-bitsteams associated with each possible target layer_id and temporal_id, and the associated sub-bitstream levels.

This example would save bitrate by avoiding sending of almost duplicate SPS’s and PPS’s for layers 1 and 2, and could also save bits in each slice header by having smaller values of pps_id.

Table 1.  HEVC Level Limits

| Level | Pixel rate | picture size |
| --- | --- | --- |
| 1 | 552,960 | 36,864 |
| 2 | 3,686,400 | 122,880 |
| 3 | 13,762,560 | 458,752 |
| 3.1 | 33,177,600 | 983,040 |
| 4 | 62,668,800 | 2,088,960 |
| 4.1 | 62,668,800 | 2,088,960 |
| 4.2 | 133,693,440 | 2,228,224 |
| 4.3 | 133,693,440 | 2,228,224 |
| 5 | 267,386,880 | 8,912,896 |
| 5.1 | 267,386,880 | 8,912,896 |
| 5.2 | 534,773,760 | 8,912,896 |
| 6 | 1,002,700,800 | 33,423,360 |
| 6.1 | 2,005,401,600 | 33,423,360 |
| 6.2 | 4,010,803,200 | 33,423,360 |

Table 2. Sub-bitstream Level Example

|  | Individual Layer |  | Sub Bitstream |  |  |
| --- | --- | --- | --- | --- | --- |
| layer _id | Pixel Rate | Level | temporal _id | Pixel Rate | Level |
| 0 | 124,416,000 | 4.2 | 0 | 31,104,000 | 4 |
|  |  |  | 1 | 62,208,000 | 4 |
|  |  |  | 2 | 124,416,000 | 4.2 |
| 1 | 124,416,000 | 4.2 | 0 | 31,104,000 | 4 |
|  |  |  | 1 | 62,208,000 | 4.2 |
|  |  |  | 2 | 248,832,000 | 5 |
| 2 | 124,416,000 | 4.2 | 0 | 31,104,000 | 4.2 |
|  |  |  | 1 | 62,208,000 | 5 |
|  |  |  | 2 | 373,248,000 | 5.2 |

--

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).