| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0232 WG 11Number :_m24474 |
| --- | --- |

| Title: | On secondary transforms for intra/inter prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

It was previously shown by Han, Saxena & Rose in ICASSP 2010, that following intra prediction, the optimal transform is not DCT, but DST Type-7 with performance close to KLT, along the direction of prediction, for the horizontal and vertical modes. The 4x4 DST by Saxena & Fernandes: JCTVC-E125 was adopted in the HEVC Geneva meeting in March 2011. Secondary transforms for intra and inter residuals were previously proposed in JCTVC-H0125 and JCTVC-H0126. This contribution presents the experimental results for the secondary transform scheme for intra/inter prediction residues. The proposed 4x4 and 8x8 secondary transform scheme is applied following the DCT output for block sizes 8x8 and higher. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. The 4x4 secondary transform does not have any addition latency in the transform pipeline. Experimental results are provided with HM 6.0 as anchor for the test conditions as stipulated in common test conditions. Average BD Rate gains of upto 0.8 and 0.5 % respectively are obtained for the 8x8 and 4x4 secondary transform schemes respectively with almost negligible run-time increase in encoder or decoder run-times.

# Introduction

In HM 6.0, mode-dependent DCT/DST scheme from JCTVC-E125 [1] is used at block size 4x4. The mode-dependent DCT/DST scheme was first published in literature in [2]. In this contribution, we propose a secondary transform scheme for intra and inter residues. The proposed 4x4 and 8x8 secondary transforms are derived analytically [3] without any training, and do not require any additional R-D search during encoding. Further as shown in [3], the additional latency of the 4x4 secondary transform is zero extra cycles with an appropriate hardware implementation. The proposed secondary transform scheme is for Luma TU’s only. We next give a high level overview of how the proposed transform scheme is applied in HM 6.0 in Table 1. Following that, we present the details of how the proposed transforms are applied for Intra TU’s as mode-dependent transform or for Inter TU’s as boundary-dependent transform.

Table 1: Overview of how the proposed transforms are applied

|  | Block Size | Intra | Inter |
| --- | --- | --- | --- |
| HM 6.0 | 4 | Mode Dependent Primary DCT/DST Type 7 | DCT |
|  | 8 | DCT | DCT |
|  | 16 | DCT | DCT |
|  | 32 | DCT | DCT |
| Proposed | 4 | No Change to HM 6.0 | Boundary Dep. Primary DCT/DST-Type 7 |
|  | 8 | DCT/DCT+Mode Dep. Secondary | Boundary Dep. DCT/DCT+Secondary |
|  | 16 | DCT/DCT+Mode Dep. Secondary | Boundary Dep. DCT/ DCT+Secondary |
|  | 32 | DCT/DCT+Mode Dep. Secondary | Boundary Dep. DCT/DCT+Secondary |

# Secondary Transforms for Intra Residues

## Application of Secondary Transform Based On Intra-Prediction Modes

Fig. 1 shows the decoder operations when a K-point mode-dependent secondary transform is applied as a row or column transform depending on the intra prediction mode for a block of size NxN. Note that N is typically greater than or equal to K. Extension to rectangular blocks of size N1 x N (for example in Short Distance Intra Prediction) is straightforward [5].

The trigger conditions when the secondary transform is used are shown in the right column in Fig. 1 and depend on the categorization of the intra prediction modes.

Depending on whether the prediction for a block is performed from only from one of the top row or left column, a prediction mode is termed as Category 1 intra prediction mode. In case, the prediction for the block is performed from both the top row and left columns, it is termed as Category 2 prediction mode. More details are in JCTVC-C108 [6], and JCTVC-E125 [1], where the optimality of mode-dependent DCT/DST for 4x4 blocks is shown.

In Fig. 1, when the intra prediction mode is DC mode (a non-directional mode), no secondary transform is applied in horizontal and vertical directions. For category 1 intra prediction modes, if the prediction was performed from pixels only from the top row, i.e., in vertical direction, and the intra prediction modes are one of the VER, VER+1, VER+2, … VER+8, then the secondary transform is used only in the vertical direction (along columns). In case, the prediction was performed from pixels only from the left column, i.e., in horizontal direction, and intra prediction modes are HOR, HOR+1, HOR+2, … HOR+8, then the secondary transform is used only in the horizontal direction (along rows).

Finally for Category 2 intra prediction modes, if the prediction is performed using both the left column and the top row, i.e., intra prediction modes are VER-1, VER-2, … VER-8 or HOR-1, HOR-2, … HOR-7, or if the intra prediction mode is Planar (a non-directional mode), then the secondary transform is applied in both the horizontal and vertical directions. The encoder instantiation is a straightforward inverse of the decoder implementation.

Note that the trigger conditions re-use the existing mapping table in HM6.0 for DCT/DST at 4x4 blocks. For example, for Vertical mode, at size 4x4 DST is used in Vertical direction, and DCT in Horizontal. In the proposed scheme at size 8x8 (or higher), for Vertical mode, before 2-d DCT, a secondary transform is applied only in Vertical Direction (maintaining a correspondence with DCT/DST scheme at 4x4 size).

Trigger Conditions

Decoder Operations

NxN

Inverse DCT

Inverse Quantizer

K-Point Inverse Row

(Horizontal)

Secondary Transform

K-Point

Inverse Column (Vertical)

Secondary

Transform

NxN Transform Unit

when IntraPredMode

= Planar or VER -1,

VER -2, …, VER-8 , or HOR-1,  HOR-2, ….,

HOR-7.

NxN

Inverse DCT

Inverse Quantizer

NxN Transform Unit

when IntraPredMode

= DC Mode

NxN

Inverse DCT

Inverse Quantizer

K-Point Inverse Row

(Horizontal)

Secondary Transform

NxN Transform Unit

when IntraPredMode =

= HOR, HOR+1,

HOR+2,…, HOR+8

NxN

Inverse DCT

Inverse Quantizer

K-Point

Inverse Column (Vertical)

Secondary Transform

NxN Transform Unit

when IntraPredMode

VER, VER+1,

VER +2 , …, VER+8

Fig 1: Example decoder operations for mode-dependent secondary transform

## Mode-Dependent Secondary Transform (MD-ST)Matrices

We derived secondary transform matrices for sizes 4x4 and 8x8 using an analytical model in JCTVC-G108. They are given in Appendix 1. The secondary transform matrices operate only on the KxK (K=4 or 8) low-frequency coefficients of the 8x8 (or larger) DCT output. Further details about the model used for deriving secondary transform matrices are in JCTVC-G108.

Note that in order to improve the compression efficiency, primary alternate transforms other than conventional DCT can be applied at block sizes 8x8, 16x16 and 32x32. However, these primary transforms will have the same size as the block size. In general, these alternate transforms at higher block sizes such as 32x32 may have marginal gains that may not justify the enormous cost of supporting additional 32x32 transform in the hardware.

Also, since most of the energy in the DCT coefficients is concentrated in the low-frequency coefficients, it should be adequate to perform operations only on the first few coefficients (typically 4 to 8) of DCT output. These operations can be performed by simply using a MD-ST of size KxK (K<=8) as presented above. Furthermore, the same secondary transform derived for a block size (such as 8x8) can be applied at higher block sizes (such as 16x16 or 32x32), while this is not true for alternate primary transform. This re-utilization of MD-ST at higher block sizes is one of the most important advantages of secondary transform.

# Secondary Transforms for Inter Residues

In [7], alternate primary transforms at sizes 4x4, 8x8 and 16x16 are proposed for the inter prediction residuals. In this proposal, we retain the transform scheme in [7] at size 4x4. However, instead of applying DST Type-4 via full matrix multiplication at sizes 8x8 and 16x16 as in [7], which may have high computational cost, we propose to use secondary transforms, which can be applied following the DCT at block sizes 8x8 and higher, including 32x32.

## Applying secondary transform for inter residues

We present the steps for applying a secondary transform (for TU1 in Fig. 3) at encoder and decoder: Encoder

1. Flip the input data. i.e., for N-point input vector x, with entries xi, i=1..N, define y with elements yi = xN+1-i

2. Take the DCT of y. Denote the output as z.

3. Apply a K-point secondary transform on the first K-elements of z. Let the output be w, where we copy the remaining “N-K” high-frequency elements from z, on which the secondary transform was not applied.

Decoder

Similarly at the decoder, let the input for transform module be v, which is a quantized version of w. Following steps can be performed for taking the inverse transform:

4. Apply inverse secondary transform on the first K-elements of v. Let the output be b (where the “N-K” high frequency coefficients are identical to that of v).

5. Take the inverse DCT of b. Denote the output as d.

6. Flip the data in d. i.e., define f with elements fi = dN+1-i

Then f is the reconstructed values for the pixels x.

For TU1 in Fig. 3, the flipping operations need not be required, and DCT followed by secondary transform only needs to be taken at the encoder. At the decoder, one just needs to take inverse secondary transform followed by inverse DCT.

Figure 2 shows how to apply the secondary transform at size 8x8 and larger. For size 4x4, the scheme is identical to [7], and more details can be found therein. Appendix 3 shows an example why a DST (or similarly a DCT+ secondary transform) would be a better transform than DCT via some figures.

Conditions for

Hor Trans. and Vert Trans.

Figure 2. Decoder operations for boundary-dependent secondary transforms at size 8 and larger. When the Horizontal secondary transform is applied such as in Case 3 or 4 above, the data is also flipped afterwards if the left TU boundary is that of PU. By symmetry, in Case 2 or 4, when the vertical transform is applied, the data is flipped afterwards if the top TU boundary is that of PU.

Decoder Operations

HM NxN Inverse DCT

Inverse Quant

K-Point Inv. Row Transforms

K-Point Inv. Col. Transforms

HM NxN Inverse DCT

Inverse Quant

TU Hor boundary

(Left and Right) are

both non-PU boundaries; or both are PU boundaries; or TU is same as PU.

HM NxN Inverse DCT

Inverse Quant

K-Point Inv. Row Transforms

HM NxN Inverse DCT

Inverse Quant

K-Point Inv. Col. Transforms

TU Ver boundary

(Top and Down) are

both non-PU boundaries; or both are PU boundaries; or TU is same as PU

TU Hor boundary:

(Left and Right): one is non-PU boundary; and

other is PU boundary

TU Hor boundary:

(Left and Right): one is non-PU boundary; and

other is PU boundary

TU Ver boundary

(Top and Down): one is non-PU boundary; and other is PU boundary

TU Ver boundary

(Top and Down) are both non-PU boundaries; or both are PU boundaries

TU Ver boundary (Top and Down): one is non-PU boundary; and

other is PU boundary

TU Hor boundary

(Left and Right) are

both non-PU boundaries;

or both are PU  boundaries

# Simulation Results

The test conditions as stipulated in common test conditions were followed for conducting the experiments. Here the anchor was HM 6.0. The performance of the proposed 4x4 and 8x8 secondary transform schemes is evaluated for all the 8 settings: Intra High Efficiency (HE)-10 bits, Intra Main, Random Access HE-10 bits, Random Access Main, Low Delay HE-10bits, Low Delay Main, Low Delay P HE-10 bits, and Low Delay Main.

Tables 2 and 3 show the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 6.0 anchor) for all the settings when the proposed 8x8 and 4x4 secondary transform schemes are used respectively. Detailed results are available in the attached Excel sheets.

Table 2: Performance Summary of 8x8 secondary transform scheme

|  |  | All Intra Main |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -1.0% | -0.6% | -0.6% | -0.9 % | -0.4 % | -0.6 % |  |
|  | Class B | -0.9% | -0.6% | -0.7% | -0.7 % | -0.7 % | -0.7 % |  |
|  | Class C | -0.5% | -0.5% | -0.5% | -0.5 % | -0.5 % | -0.5 % |  |
|  | Class D | -0.4% | -0.3% | -0.3% | -0.3 % | -0.3 % | -0.3 % |  |
|  | Class E | -1.4% | -1.0% | -0.9% | -1.2 % | -1.0 % | -1.0 % |  |
|  | Overall | -0.8% | -0.6% | -0.6% | -0.7 % | -0.6 % | -0.6 % |  |
|  |  | -0.8% | -0.6% | -0.6% | -0.7 % | -0.6 % | -0.6 % |  |
|  | Class F | 0.0% | -0.3% | -0.3% | 0.0 % | -0.1 % | -0.2 % |  |
|  | Enc Time[%] | 104% |  |  | 104% |  |  |  |
|  | Dec Time[%] | 104% |  |  | 102% |  |  |  |
|  |  | Random Access Main |  |  | Random Access HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7% | 0.1% | 0.1% | -0.6 % | 0.4 % | -0.2 % |  |
|  | Class B | -0.8% | -0.3% | -0.2% | -0.6 % | -0.5 % | -0.5 % |  |
|  | Class C | -0.5% | -0.4% | -0.3% | -0.5 % | -0.4 % | -0.5 % |  |
|  | Class D | -0.5% | -0.3% | 0.0% | -0.4 % | -0.1 % | -0.4 % |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Overall | -0.6% | -0.2% | -0.1% | -0.5 % | -0.2 % | -0.4 % |  |
|  |  | -0.6% | -0.2% | -0.1% | -0.5 % | -0.2 % | -0.4 % |  |
|  | Class F | 0.0% | 0.0% | 0.0% | -0.1 % | -0.4 % | -0.1 % |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 100% |  |  |  |
|  |  | Low delay B Main |  |  | Low delay B HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.6% | -0.7% | -0.5% | -0.6 % | -0.4 % | -0.3 % |  |
|  | Class C | -0.5% | -0.3% | -0.4% | -0.5 % | -0.3 % | -0.2 % |  |
|  | Class D | -0.4% | -0.3% | -0.2% | -0.5 % | -0.1 % | -0.1 % |  |
|  | Class E | -0.2% | -0.8% | 1.0% | -0.3 % | -0.5 % | -0.8 % |  |
|  | Overall | -0.5% | -0.5% | -0.1% | -0.5 % | -0.3 % | -0.3 % |  |
|  |  | -0.5% | -0.5% | 0.0% | -0.5 % | -0.3 % | -0.3 % |  |
|  | Class F | -0.3% | -0.3% | -0.5% | -0.2 % | -0.8 % | 1.9 % |  |
|  | Enc Time[%] | 101% |  |  | 101 % |  |  |  |
|  | Dec Time[%] | 101% |  |  | 99 % |  |  |  |
|  |  | Low delay P Main |  |  | Low delay P HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.8% | -0.5% | -1.0% | -0.6 % | -0.6 % | -0.2 % |  |
|  | Class C | -0.6% | -0.9% | -0.5% | -0.6 % | -0.8 % | -0.4 % |  |
|  | Class D | -0.5% | -0.2% | -0.9% | -0.5 % | 0.0 % | 0.2 % |  |
|  | Class E | -0.6% | -0.2% | 1.3% | -0.5 % | 0.1 % | -0.5 % |  |
|  | Overall | -0.6% | -0.5% | -0.5% | -0.6 % | -0.3 % | -0.2 % |  |
|  |  | -0.6% | -0.4% | -0.3% | -0.6 % | -0.3 % | -0.1 % |  |
|  | Class F | -0.3% | 0.4% | -0.3% | -0.3 % | 0.2 % | -1.1 % |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 99% |  |  |  |

Table 3: Performance Summary of 4x4 secondary transform scheme

|  |  | All Intra Main |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.6% | -0.1% | -0.1% | -0.6 % | 0.0 % | 0.0 % |  |
|  | Class B | -0.6% | -0.2% | -0.2% | -0.5 % | -0.2 % | -0.2 % |  |
|  | Class C | -0.4% | -0.4% | -0.3% | -0.3 % | -0.3 % | -0.3 % |  |
|  | Class D | -0.2% | -0.2% | -0.2% | -0.2 % | -0.2 % | -0.2 % |  |
|  | Class E | -0.9% | -0.6% | -0.5% | -0.9 % | -0.6 % | -0.5 % |  |
|  | Overall | -0.5% | -0.3% | -0.3% | -0.5 % | -0.2 % | -0.2 % |  |
|  |  | -0.5% | -0.3% | -0.3% | -0.5 % | -0.2 % | -0.2 % |  |
|  | Class F | 0.0% | -0.3% | -0.2% | 0.0 % | -0.1 % | -0.2 % |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |  |
|  |  | Random Access Main |  |  | Random Access HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.5% | 0.3% | 0.5% | -0.5 % | 0.3 % | 0.3 % |  |
|  | Class B | -0.5% | -0.2% | -0.1% | -0.5 % | -0.1 % | -0.1 % |  |
|  | Class C | -0.4% | -0.4% | -0.4% | -0.4 % | -0.3 % | -0.3 % |  |
|  | Class D | -0.3% | -0.1% | 0.0% | -0.3 % | -0.3 % | -0.2 % |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Overall | -0.5% | -0.1% | 0.0% | -0.4 % | -0.1 % | -0.1 % |  |
|  |  | -0.5% | -0.1% | 0.0% | -0.4 % | -0.1 % | -0.1 % |  |
|  | Class F | -0.1% | -0.2% | 0.0% | -0.1 % | -0.1 % | -0.1 % |  |
|  | Enc Time[%] | 101% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 100% |  |  |  |
|  |  | Low delay B Main |  |  | Low delay B HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.5% | -0.6% | -0.4% | -0.5 % | -0.5 % | -0.2 % |  |
|  | Class C | -0.4% | -0.1% | -0.2% | -0.5 % | -0.4 % | -0.2 % |  |
|  | Class D | -0.3% | 0.1% | -0.2% | -0.4 % | -0.4 % | -0.2 % |  |
|  | Class E | -0.3% | -1.0% | 0.1% | -0.3 % | -0.8 % | 0.2 % |  |
|  | Overall | -0.4% | -0.4% | -0.2% | -0.4 % | -0.5 % | -0.1 % |  |
|  |  | -0.4% | -0.3% | -0.2% | -0.4 % | -0.4 % | -0.1 % |  |
|  | Class F | -0.4% | -0.1% | 0.0% | -0.1 % | 0.0 % | 0.4 % |  |
|  | Enc Time[%] | 101% |  |  | 101 % |  |  |  |
|  | Dec Time[%] | 102% |  |  | 101 % |  |  |  |
|  |  | Low delay P Main |  |  | Low delay P HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.6% | -0.6% | -0.9% | -0.5 % | -0.6 % | -0.6 % |  |
|  | Class C | -0.6% | -0.6% | -0.5% | -0.5 % | -0.4 % | -0.5 % |  |
|  | Class D | -0.5% | 0.2% | -0.3% | -0.5 % | -0.3 % | -0.2 % |  |
|  | Class E | -0.3% | -1.2% | 0.2% | -0.3 % | -0.1 % | 0.9 % |  |
|  | Overall | -0.5% | -0.5% | -0.5% | -0.5 % | -0.4 % | -0.2 % |  |
|  |  | -0.5% | -0.5% | -0.4% | -0.5 % | -0.3 % | 0.0 % |  |
|  | Class F | -0.2% | 0.0% | 0.5% | -0.3 % | 1.2 % | -1.5 % |  |
|  | Enc Time[%] | 100% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |  |

From the above experimental results, average BD rate gains of upto 0.8% and 0.5 % are obtained for the proposed 8x8 and 4x4 secondary transform scheme. The additional encoding/decoding time due to the introduction of secondary transforms is negligible.

## Further Results

We also present results when the proposed 8x8 secondary transform is used at block size 8x8 only, and not at 16x16 and 32x32. Further, we multiply the 8x8 DCT and 8x8 secondary transforms to effectively have a primary KLT matrix as follows:

const short g_as_KLT_MAT_8 [8][8]=

{

{    28,    42,    52,    62,    70,    76,    80,    82,},

{    70,    86,    78,    53,    13,   -28,   -62,   -82,},

{   -85,   -67,     2,    66,    88,    47,   -26,   -80,},

{    83,    13,   -81,   -65,    35,    89,    24,   -72,},

{   -76,    43,    76,   -51,   -71,    57,    68,   -62,},

{    63,   -81,     0,    83,   -70,   -24,    89,   -48,},

{   -47,    86,   -75,    23,    44,   -87,    82,   -34,},

{    30,   -58,    78,   -89,    85,   -70,    48,   -16,},

};

Note that the above matrix was also presented in JCTVC-G632.

Table 4.1.1: Performance Summary of 4x4 secondary transform scheme

|  |  | All Intra Main |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7% | 0.1% | 0.3% | -0.6% | 0.4% | 0.2% |  |
|  | Class B | -0.4% | 0.2% | 0.1% | -0.3% | 0.2% | 0.1% |  |
|  | Class C | -0.4% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% |  |
|  | Class D | -0.3% | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% |  |
|  | Class E | -1.0% | 0.1% | 0.2% | -0.8% | 0.0% | 0.1% |  |
|  | Overall | -0.5% | 0.0% | 0.1% | -0.4% | 0.1% | 0.0% |  |
|  |  | -0.5% | 0.0% | 0.1% | -0.4% | 0.1% | 0.0% |  |
|  | Class F | 0.0% | 0.0% | 0.0% | 0.1 % | 0.0 % | 0.0 % |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 100% |  |  |  |
|  |  | Random Access Main |  |  | Random Access HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.5% | 0.2% | 0.4% | -0.4% | 0.5% | 0.1% |  |
|  | Class B | -0.4% | 0.2% | 0.3% | -0.3% | 0.3% | 0.1% |  |
|  | Class C | -0.4% | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% |  |
|  | Class D | -0.4% | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Overall | -0.4% | 0.1% | 0.1% | -0.3% | 0.1% | 0.0% |  |
|  |  | -0.4% | 0.0% | 0.1% | -0.3% | 0.2% | 0.0% |  |
|  | Class F | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |
|  | Enc Time[%] | 101% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 99% |  |  | 99% |  |  |  |
|  |  | Low delay B Main |  |  | Low delay B HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.4% | -0.2% | -0.2% | -0.3% | 0.0% | 0.3% |  |
|  | Class C | -0.4% | 0.1% | -0.1% | -0.4% | 0.0% | 0.0% |  |
|  | Class D | -0.3% | -0.2% | 0.1% | -0.4% | 0.3% | -0.2% |  |
|  | Class E | -0.2% | -0.3% | 1.2% | -0.2% | 0.5% | 0.3% |  |
|  | Overall | -0.4% | -0.2% | 0.1% | -0.3% | 0.1% | 0.1% |  |
|  |  | -0.3% | -0.1% | 0.1% | -0.3% | 0.2% | 0.1% |  |
|  | Class F | -0.2% | 0.9% | -0.4% | -0.2% | -0.8% | 1.5% |  |
|  | Enc Time[%] | 101% |  |  | 101 % |  |  |  |
|  | Dec Time[%] | 99% |  |  | 99 % |  |  |  |
|  |  | Low delay P Main |  |  | Low delay P HE10 |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.5% | -0.1% | -0.4% | -0.3 % | 0.0 % | 0.5 % |  |
|  | Class C | -0.5% | -0.4% | -0.3% | -0.4 % | -0.1 % | -0.4 % |  |
|  | Class D | -0.5% | 0.3% | -0.5% | -0.4 % | 0.8 % | -0.2 % |  |
|  | Class E | -0.4% | -0.1% | 1.5% | -0.2 % | 0.8 % | 0.2 % |  |
|  | Overall | -0.5% | -0.1% | -0.1% | -0.4 % | 0.4 % | 0.0 % |  |
|  |  | -0.5% | -0.1% | 0.0% | -0.4 % | 0.4 % | 0.1 % |  |
|  | Class F | -0.2% | 0.9% | -0.3% | -0.2 % | 2.2 % | 1.3 % |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 98% |  |  | 98% |  |  |  |

From the above results, we note that the 8x8 DCT/KLT gives upto 0.5 % BD-Rate gain.

Table 4.1.2: Overview of how the proposed transforms are applied

|  | Block Size | Intra | Inter |
| --- | --- | --- | --- |
| HM 6.0 | 4 | Mode Dependent Primary DCT/DST Type 7 | DCT |
|  | 8 | DCT | DCT |
|  | 16 | DCT | DCT |
|  | 32 | DCT | DCT |
| Proposed | 4 | No Change to HM 6.0 | Boundary Dep. Primary DCT/DST-Type 7 |
|  | 8 | Mode Dep. Primary DCT/KLT | Boundary Dep. Primary DCT/KLT |
|  | 16 | No Change to HM 6.0 | No Change to HM 6.0 |
|  | 32 | No Change to HM 6.0 | No Change to HM 6.0 |

Operation Counts : The matrix multiplication for 8x8 KLT in our proposal requires 1024 multiplications, 1024 additions, 128 shifts and 128 clips. The 8x8 DCT has 352 mults, 576 adds and 128 shifts and 128 clips.

- Complexity Considerations

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed secondary transforms were derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 6.0.

Transform Matrices: For a KxK secondary transform scheme (K=4 or 8), we only require the storage of one additional KxK matrix.

Bit Shift at Intermediate Levels: In our implementation, we shift the output of secondary transform by 7 bits after vertical or horizontal transform. The bit-precision for all the transforms is 16 bits, similar to the transforms HM 6.0.

Scanning: In this proposal we have retained the conventional HM 6.0 scans.

Operation Counts: Tables 4 and 5 show the worst-case and average case operations when the secondary transform scheme is applied for Intra TU’s. Similarly, Tables 6 and 7 show the worst-case and average case operations when the secondary transform scheme is applied to Inter TU’s. For more details how to obtain these operation counts, we refer the reader to [3] and [4].

Table 4: Worst-Case Inverse transform operations count summary for the secondary transform schemes for Intra TU’s

| Transform Unit size |  | HM 6.0 DCT | HM 6.0 DST | 4x4 Secondary Transform | 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT | Percentage increase for 4x4 Secondary Transform Over HM DCT | Percentage increase for 8x8 Secondary Transform Over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 64 | 0 | 0 | 33.3 | N/A | N/A |
|  | Adds | 96 | 120 | 0 | 0 | 25 | N/A | N/A |
|  | Shifts | 32 | 32 | 0 | 0 | 0 | N/A | N/A |
|  | Clips | 32 | 32 | 0 | 0 | 0 | N/A | N/A |
| 8x8 | Mults | 352 | N/A | +128 | +1024 | N/A | 36.4 | 290.9 |
|  | Adds | 576 | N/A | +128 | +1024 | N/A | 22.2 | 177.8 |
|  | Shifts | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
|  | Clips | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
| 16x16 | Mults | 2752 | N/A | +128 | +1024 | N/A | 4.7 | 37.2 |
|  | Adds | 3712 | N/A | +128 | +1024 | N/A | 3.4 | 27.6 |
|  | Shifts | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
|  | Clips | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
| 32x32 | Mults | 21888 | N/A | +128 | +1024 | N/A | 0.6 | 4.7 |
|  | Adds | 25856 | N/A | +128 | +1024 | N/A | 0.5 | 4.0 |
|  | Shifts | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |
|  | Clips | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |

Table 5: Average Case Inverse transform operations count summary for the secondary transform schemes for Intra TU’s

| Transform Unit size |  | HM 6.0 DCT | HM 6.0 Average for 4x4 blocks which use DCT and DST | 4x4 Secondary Transform | 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT | Percentage increase for 4x4 Secondary Transform Over HM DCT | Percentage increase for 8x8 Secondary Transform Over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 60 | 0 | 0 | 24.8 | N/A | N/A |
|  | Adds | 96 | 114 | 0 | 0 | 18.6 | N/A | N/A |
|  | Shifts | 32 | 32 | 0 | 0 | 0.0 | N/A | N/A |
|  | Clips | 32 | 32 | 0 | 0 | 0.0 | N/A | N/A |
| 8x8 | Mults | 352 | N/A | +91 | +731 | N/A | 26.0 | 207.8 |
|  | Adds | 576 | N/A | +91 | +731 | N/A | 15.9 | 127.0 |
|  | Shifts | 128 | N/A | +23 | +91 | N/A | 17.9 | 71.4 |
|  | Clips | 128 | N/A | +23 | +91 | N/A | 17.9 | 71.4 |
| 16x16 | Mults | 2752 | N/A | +91 | +731 | N/A | 3.3 | 26.6 |
|  | Adds | 3712 | N/A | +91 | +731 | N/A | 2.5 | 19.7 |
|  | Shifts | 512 | N/A | +23 | +91 | N/A | 4.5 | 17.9 |
|  | Clips | 512 | N/A | +23 | +91 | N/A | 4.5 | 17.9 |
| 32x32 | Mults | 21888 | N/A | +91 | +731 | N/A | 0.4 | 3.3 |
|  | Adds | 25856 | N/A | +91 | +731 | N/A | 0.4 | 2.8 |
|  | Shifts | 2048 | N/A | +23 | +91 | N/A | 1.1 | 4.5 |
|  | Clips | 2048 | N/A | +23 | +91 | N/A | 1.1 | 4.5 |

Remark: From the above table, we see that the worst-case increase in the complexity of the 4x4 and 8x8 secondary transform is 36.4 % and 290.9 %. Note that the gains for Random Access case, the 4x4 and 8x8 secondary transform schemes in JCTVC-H0126 are providing upto 0.7 % and 0.8 % gain respectively on average for the High Efficiency and Low Complexity settings in HM 6.0, while the 4x4 DST adopted in March 2011 provided 0.3 % on average gain in Random Access High Efficiency case at an almost additional worst-case complexity of 33.3 % (RA-LC setting results for 4x4 DST from E-125 are not reported because CAVLC was used).

Note that the worst-case complexity metrics should not be directly compares with average case gains, which are actually for the whole video codec. In fact, the transforms take a very nominal part of the whole video codec, especially for the inter case, and the additional complexity for the whole video codec by the introduction of new transform techniques could be gauged by the increase in encoding/decoding times.

Table 6: Worst-Case Inverse transform operations count summary for the proposed secondary transform scheme for Inter TU’s

| Transform Unit size |  | HM 6.0 DCT | HM 6.0 DST(currently Used for Intra TU’s only in HM) | Proposed scheme with 4x4 Secondary Transform | Proposed scheme with 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT in Intra TU’s | Percentage increase for proposed scheme with 4x4 secondary transform Over HM DCT | Percentage increase for proposed scheme with 8x8 secondary transform over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 64 | 64 | 0 | 33.3 | 33.3 | 33.3 |
|  | Adds | 96 | 120 | 120 | 0 | 25 | 25 | 25 |
|  | Shifts | 32 | 32 | 32 | 0 | 0 | 0 | 0 |
|  | Clips | 32 | 32 | 32 | 0 | 0 | 0 | 0 |
| 8x8 | Mults | 352 | N/A | +128 | +1024 | N/A | 36.4 | 290.9 |
|  | Adds | 576 | N/A | +128 | +1024 | N/A | 22.2 | 177.8 |
|  | Shifts | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
|  | Clips | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
| 16x16 | Mults | 2752 | N/A | +128 | +1024 | N/A | 4.7 | 37.2 |
|  | Adds | 3712 | N/A | +128 | +1024 | N/A | 3.4 | 27.6 |
|  | Shifts | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
|  | Clips | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
| 32x32 | Mults | 21888 | N/A | +128 | +1024 | N/A | 0.6 | 4.7 |
|  | Adds | 25856 | N/A | +128 | +1024 | N/A | 0.5 | 4.0 |
|  | Shifts | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |
|  | Clips | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |

Table 7: Average-Case Inverse transform operations count summary for the secondary transform scheme for Inter TU’s

| Transform Unit size |  | HM 6.0 DCT | HM 6.0 Average for 4x4 blocks which use DCT and DST (for Intra TU’s only) | Proposed scheme with 4x4 Secondary Transform | Proposed scheme with 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT in Intra TU’s | Percentage increase for proposed scheme with 4x4 secondary transform Over HM DCT | Percentage increase for proposed scheme with 8x8 secondary transform over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 60 | 56 | 56 | 24.8 | 16.7 | 16.7 |
|  | Adds | 96 | 114 | 108 | 108 | 18.6 | 12.5 | 12.5 |
|  | Shifts | 32 | 32 | 32 | 32 | 0.0 | 0.0 | 0.0 |
|  | Clips | 32 | 32 | 32 | 32 | 0.0 | 0.0 | 0.0 |
| 8x8 | Mults | 352 | N/A | +64 | +512 | N/A | 18.2 | 145.5 |
|  | Adds | 576 | N/A | +64 | +512 | N/A | 11.1 | 88.9 |
|  | Shifts | 128 | N/A | +16 | +64 | N/A | 12.5 | 50.0 |
|  | Clips | 128 | N/A | +16 | +64 | N/A | 12.5 | 50.0 |
| 16x16 | Mults | 2752 | N/A | +64 | +512 | N/A | 2.3 | 18.6 |
|  | Adds | 3712 | N/A | +64 | +512 | N/A | 1.7 | 13.8 |
|  | Shifts | 512 | N/A | +16 | +64 | N/A | 3.1 | 12.5 |
|  | Clips | 512 | N/A | +16 | +64 | N/A | 3.1 | 12.5 |
| 32x32 | Mults | 21888 | N/A | +85 | +683 | N/A | 0.4 | 3.1 |
|  | Adds | 25856 | N/A | +85 | +683 | N/A | 0.3 | 2.6 |
|  | Shifts | 2048 | N/A | +21 | +85 | N/A | 1.0 | 4.2 |
|  | Clips | 2048 | N/A | +21 | +85 | N/A | 1.0 | 4.2 |

# Techniques for reducing latency for secondary transform in hardware

In general, a secondary transform after a primary transform (e.g., DCT) would require some additional cycles for execution. This can pose latency overhead in the encoder/decoder, and throughput may decrease. However, this latency can be reduced if an appropriate hardware implementation is performed as we have shown in an earlier contribution JCTVC-H0125. For the 4x4 secondary transform, the additional latency of zero cycles can be obtained. We refer the reader to [3] for more details.

# Conclusions

In this contribution, we have presented various secondary transforms for intra/inter prediction residual coding. The proposed secondary transform scheme requires the storage of only one additional transform matrix, and does not require any explicit signaling. Simulation results show significant gains in compression performance as compared to HM 6.0 anchors. The proposed 4x4 secondary transform does not have any additional latency in hardware as well. We recommend adopting this proposal into HM and Committee Draft.

# References

1. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” JCTVC-E125, Geneva, Switzerland, March 2011.

2. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

3. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “On secondary transforms for intra prediction residual,” JCTVC-H0125, San Jose, USA, Feb 2012.

4. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “On secondary transforms for inter prediction residual,” JCTVC-H0126, San Jose, USA, Feb 2012.

5. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “Harmonization of SDIP and mode-dependent secondary transforms,” JCTVC-G629, Geneva, Switzerland, Nov 2011.

6.  A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” JCTVC-C108, Guangzhou, China, October 2010.

7. J. An, X. Zhao, X. Guo, S. Lei, “Non-CE7: Boundary dependent transform for inter predicted residue”, JCTVC-G281, Geneva, Switzerland, Nov 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix 1

The 4x4 and 8x8 secondary transform matrices are given as follows, where the basis vectors are along rows, and the variable SEC_TR_SIZE denotes the size of secondary transform.

SEC_TR_SIZE ==4

{   123,   -35,    -8,    -3,},

{   -32,  -120,    30,    10,},

{    14,    25,   123,   -22,},

{     8,    13,    19,   126,},

SEC_TR_SIZE ==8

{   123,   -35,    -8,    -3,    -2,    -1,    -1,    -1},

{    32,   120,   -29,   -10,    -5,    -3,    -2,    -1},

{   -14,   -24,  -123,    21,     8,     4,     3,     2},

{     7,    11,    17,   125,   -16,    -7,    -4,    -2},

{    -4,    -7,    -8,   -13,  -126,    13,     5,     3},

{     3,     4,     5,     6,    11,   127,   -10,    -5},

{    -2,    -3,    -3,    -4,    -5,    -9,  -127,     9},

{     2,     2,     3,     3,     3,     5,     8,   128},

# Appendix 2

Fig. 3 Transform Units in a Prediction Unit. The numbers in 4x4 TU4 denote the energy of the prediction error.

(a)                                     (b)

Fig. 4

(a) DCT basis vectors: First basis vector is flat.

(b)DST basis vectors: First basis vector is zero at one boundary, and maximum at the other boundary. This mimics the prediction error residue behavior in TU4 more efficiently as compared to the DCT basis vector.