Change of Woking Darft for proposal

JCTVC-I0232: On secondary transforms for intra/inter prediction residual

The document is based on JCTVC-H1003_d22.doc and change is colored yellow.

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

- a luma location ( xB, yB ) specifying top-left luma sample of the current TU relative to top-left luma sample of the current CU,

- a variable nCS specifying size of current coding unit.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,   qP = QP’Y															    (8-1)

–	Otherwise if cIdx is equal to 1,

qP = QP’C	b		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP’C	r		(8-3)

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, (nW)x(nH) array r is set equal to (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the width of the transform unit nW, the height of the transform unit nH, the (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ], the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nW)x(nH) array d.

- The transformation process for scaled transform coefficients as specified in subclause  is invoked with the width of the transform unit nW, the height of the transform unit nH, the scaled transform coefficient (nW)x(nH) array d and the chroma component variable cIdx, luma location (xB, yB), and coding unit size nCS as the inputs and the output is a residual samples (nW)x(nH) array r.

### 8.6.4   Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

- a luma location ( xB, yB ) specifying top-left luma sample of the current TU relative to the top-left luma sample of the current CU,

- a variable nCS specifying size of current coding unit.

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

Depending on PredMode, PartMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, log2(nW*nH) is equal to 4 or 8, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Error: Reference source not found with IntraPredMode as input.

–	Otherwise, if PredMode is not equal to MODE_INTRA, and cIdx is equal to 0,	then the variables horizTrType and vertTrType are derived by invoking the process as specified in subclause 8.6.4.2 with the PartMode, xB,yB, nW, nH, and nCS as the inputs and the output are horizTrType and vertTrType.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.    .......

8.6.4.1 Transformation process

Inputs of this process are:

–	a variable nS specifying the sample size of scaled transform coefficients,

–	an array of scaled transform coefficients x with elements xi, with i=0..nS−1.

–	a transform type variable trType

Output of this process is an array of samples of the residual samples y with elements yi, with i = 0..nS-1.

Depending on trType, the following applies:

–	If nS is equal to 4 and trType is equal to 1 or 2 , the following ordered steps apply:

- A set of intermediate values c0, c1, c2 and c3 is calculated as follows:c0 = x0 + x2		(8-4)
c1 = x2 + x3		(8-5)
c2 = x0 – x3		(8-6)
c3 = 74 * x1		(8-7)

- If trType = = 1, the output values yi with i = 0..3 are then specified as follows:

y0 = Clip3( -32768, 32767,  (29*c0 + 55*c1 + c3 + 64 )  >> 7)	
y1 = Clip3( -32768, 32767,  (55*c2 – 29*c1 + c3 + 64 )  >> 7)	
y2 = Clip3( -32768, 32767,  (74*( x0 - x2 + x3 ) + 64 )    >> 7)
y3 = Clip3( -32768, 32767,  (55*c0 + 29 * c2 – c3 + 64 ) >> 7)

–	else trType is equal to 2, and the output values yi with i = 0..3 are then specified as follows:

y3 = Clip3( -32768, 32767,  (29*c0 + 55*c1 + c3 + 64 )  >> 7)	
y2 = Clip3( -32768, 32767,  (55*c2 – 29*c1 + c3 + 64 )  >> 7)	
y1 = Clip3( -32768, 32767,  (74*( x0 - x2 + x3 ) + 64 )    >> 7)
y0 = Clip3( -32768, 32767,  (55*c0 + 29 * c2 – c3 + 64 ) >> 7)

–	If nS is equal to 8 and trType is equal to 1, the following ordered steps apply:

yi = ∑j( mkj * xj ) with i=0..nS-1, j=0..nS-1		(8-8)

–	Otherwise ( (nS is equal to 8, and trType is equal to 2)

yj = ∑j( mjk * xj ) with i=0..nS-1, j=0..nS-1		(8-9)

where k = 1 << ( 5 – log2( nS ) ) * i and the transform coefficient matrix m is the proposed KLT

m =  [28,    42,    52,    62,    70,    76,    80,    82;

70,    86,    78,    53,    13,   -28,   -62,   -82;

-85,   -67,     2,    66,    88,    47,   -26,   -80;

83,    13,   -81,   -65,    35,    89,    24,   -72;

-76,    43,    76,   -51,   -71,    57,    68,   -62;

63,   -81,     0,    83,   -70,   -24,    89,   -48;

-47,    86,   -75,    23,    44,   -87,    82,   -34;

30,   -58,    78,   -89,    85,   -70,    48,   -16;]

–	[Delete this: Otherwise (nS is not equal to 4 or trType is not equal to 1), the following applies:]

–	If ( (nS is equal to 4, and trType is equal to 0) or (nS is equal to 8 and trType is equal to 0 or 1) ), the following applies:

yi = ∑j( ckj * xj ) with i=0..nS-1, j=0..nS-1		(8-10)

–	Otherwise ( (nS is equal to 8, and trType is equal to 2)

yi = ∑j( cjk * xj ) with i=0..nS-1, j=0..nS-1			(8-11)

where k = 1 << ( 5 – log2( nS ) ) * i and the transform coefficient matrix c is DCT in Working Draft.

8.6.4.2 Transformation type derivation process

Inputs of this process are: a luma location ( xB, yB ) specifying the top-left luma sample of the current TU relative to the top-left luma sample of the current CU, variables nW, nH specifying the width, height of the current TU, a variable PartMode specifying prediction partition mode of current CU, a variable nCS specifying size of current CU.

Output of this process are: Variables horizTrType and vertTrType.

Set intermediate variable leftBoundaryType to 1 if one of the following conditions is true; else set it to 0.

- xB is equal to 0

- xB is equal to nCS>>2, and PartMode is equal to PART_nLx2N

- xB is equal to nCS>>1, and PartMode is equal to PART_Nx2N or PART_NxN

- xB is equal to 3*nCS>>2, and PartMode is equal to PART_nRx2N

Set intermediate variable rightBoundaryType to 1 if one of the following conditions is true; else set it to 0.

- xB+nW-1 is equal to (nCS>>2)-1, and PartMode is equal to PART_nLx2N

- xB+nW-1 is equal to (nCS>>1)-1, and PartMode is equal to PART_Nx2N or PART_NxN

- xB+nW-1 is equal to (3*nCS>>2)-1, and PartMode is equal to PART_nRx2N

- xB+nW-1 is equal to nCS-1

If  nW is larger than 32, set horizTrType to 0, else derive horizTrType as follows:

- If leftBoundaryType == 1 and rightBoundaryType == 0, set horizTrType to 2,

- Else if leftBoundaryType == 0 and rightBoundaryType == 1, set horizTrType is set to 1,

- Else set horizTrType to 0.

Set topBoundaryType to 1 if one of the following conditions is true; else set it to 0

- yB is equal to 0

- yB is equal to nCS>>2, and PartMode is equal to PART_2NxnU

- yB is equal to nCS>>1, and PartMode is equal to PART_2NxN or PART_NxN

- yB is equal to 3*nCS>>2, and PartMode is equal to PART_2NxnD

Set bottomBoundaryType to 1 if one of the following conditions is true; else set it to 0.

- yB+nH-1 is equal to (nCS>>2)-1, and PartMode is equal to PART_2NxnU

- yB+nH-1 is equal to (nCS>>1)-1, and PartMode is equal to PART_2NxN or PART_NxN

- yB+nH-1 is equal to (3*nCS>>2)-1, and PartMode is equal to PART_2NxnD

- yB+nH-1 is equal to nCS-1

If nH is larger than 32, set vertTrType to 0, else derive variable vertTrType as follows:

- If topBoundaryType == 1 and bottomBoundaryType == 0, set variable vertTrType to 2,

- Else if the topBoundaryType == 0 and bottomBoundaryType == 1, set vertTrType to 1,

- Else set variable vertTrType to 0.