| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0233_r2 m24475 |
| --- | --- |

| Title: | AHG4: Enabling decoder parallelism with tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Jack Enhorn Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution identifies a number of problems regarding tiles: There is currently no mechanism for an encoder to guarantee that a coded video sequence can be decoded in parallel, the tile syntax is replicated in both SPS and PPS, there is no semantics for the PPS tile syntax, there is a dependency between SPS and PPS, no tile index is signaled when entry point offsets are used for tiles, the semantics for tile_idx_minus_1 is incomplete, and the tile parameter derivation text is currently in the tile semantics section. A revision 1 (r1) version of this document was uploaded late. The r1 changes consist of changes to the abstract and editorial corrections to the proposed WD semantics for use_tile_info_from_pps_flag. A revision 2 (r2) version of this document was uploaded late. The r2 changes consist of proposed WD text changes reflecting the decision at the 9th JCT-VC meeting to adopt proposal 4 as a VUI flag. The WD text change can be found in section 4 of this proposal.

This proposal claims to address these tile problems by proposing the following changes:

1) To make a separate tile_info syntax table that is shared between SPS and PPS

2) To merge the two PPS flags, tile_info_present_flag and tile_control_present_flag into one flag: tile_info_present_in_pps_flag

3) To add a flag in the slice header, use_tile_info_from_pps_flag, to control whether the tile info from the SPS or the PPS shall be used. The flag is only present if there is both SPS and PPS tile info.

4) To add an SPS flag, tiles_fixed_structure_flag, to indicate that the tile info from the SPS is always used. If set to one, we do not parse use_tile_info_from_pps_flag.

5) To add two SPS flags to indicate that all tiles do have entry point offsets or entry point markers and to include tile id with entry point offsets and markers only if the corresponding flag is set equal to 0.

6) To only send tile_idx_minus_1 for entry point markers if tiles are used (not send them in case of WPP) and change its name to tile_id_marker_minus1

7) To specify the length and value of tile_idx_minus_1

8) To add a tile id syntax element, tile_id_offset_minus1, for every tile entry point offset

9) To move tile parameters derivation text, currently in the semantics section, to a new subclause in the decoding process

10) To clarify the semantics for entry_point_offset

# Introduction

Tiles can be used in HEVC to enable parallel encoding and decoding. It defines and breaks the prediction between picture segments such that each segment can be processed in parallel.

To enable parallel decoding with tiles, there are entry points that optionally can be inserted into the bitstream to indicate where in the bitstream the segments start. There are two different types of entry points: Entry point offsets and entry point markers.

## Entry point offsets

Entry point offsets are fixed length coded offsets that are optionally present in the slice header according to the syntax shown in Table 1.

Table 1 - Existing slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

num_entry_point_offsets denote how many entry points offsets there are in a slice. The maximum number of tile entry point offsets is the number of tiles minus 1 since the first tile in the slice does not need any index. Note that there is no restriction that each tile needs to have an entry point. For example, there may be four tiles in a picture but only one entry point offset.

Fig. 1 shows an example of an entry point offset. The slice header contains a list of bitstream pointers as shown in Table 1 above. The entry point offset is a byte offset which indicates to a decoder where tile 1 starts. The tile 0 data is byte-aligned in order for the entry point offset to be a byte pointer and not a bit pointer.

Fig. 1 - A tile entry point offset

## Entry point markers

Entry point markers provide an independent way of indicating tile boundaries. The current syntax shows that there may optionally be an entry_point_marker between two tiles. If there is a marker, there will be a tile_idx_minus_1 codeword to indicate the tile id of the next tile. This codeword is needed for a decoder in order to know the spatial position of the tile that follow the marker.

Table 2 - Existing slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

Fig. 2 shows an example of an entry point marker. The tile 0 and tile 1 data is separated by a three-byte startcode-like code word called entry point marker that due to the startcode emulation prevention mechanism in HEVC cannot occur anywhere else in the slice. A decoder can find out where tile 1 starts by scanning the slice data for the entry point marker. Similar to the entry point offset case, the tile 0 data is byte-aligned.

Fig. 2 - A tile entry point marker

# Proposal

## SPS and PPS tile syntax

The tile syntax is currently replicated in both SPS and PPS using the same syntax elements (with exactly the same names), which, in our opinion, seems very confusing and imprecise. Especially since there is no semantics for the PPS elements. Further, it is also not clearly specified which one to use when both the active SPS and the active PPS contains tile syntax (there is an editorial note in the CD about this). Finally, the SPS codeword tiles_or_entropy_coding_sync_idc is currently used in the PPS, causing a dependency between SPS and PPS. There is also an editorial note about this.

We propose to remove the replication by breaking out the tile syntax into one syntax table that is called from both the SPS and PPS. We also propose to merge the two PPS flags, tile_info_present_flag and tile_control_present_flag into one flag: tile_info_present_in_pps_flag since we see no real use case for applying loop_filter_across_tiles_enabled_flag from the PPS while applying the other tile paramters from the SPS, or the other way around. On the contrary, this flexibility seems to only complicate decoder implementations at the cost of one additional bit.

The current PPS syntax looks like this:

Table 3 - From the current PPS syntax

| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| --- | --- |
| tile_info_present_in_pps_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_in_pps_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |

We propose to replace the current PPS syntax shown in Table 3 with the syntax in Table 4:

Table 4 - Proposed PPS syntax

| tile_info_present_in_pps_flag | u(1) |
| --- | --- |
| if( tile_info_present_in_pps_flag ) |  |
| tile_info( 1 ) |  |

We propose the following semantics:

tile_info_present_in_pps_flag equal to 1 specifies that tile_info() is present in the picture parameter set. tile_info_present_in_pps_flag equal to 0 specifies that tile_info() is not present in the picture parameter set.

loop_filter_across_tiles_enabled_flag.is included in the proposed tile_info syntax shown in Table 5.

Table 5 - Proposed tile_info syntax

| tile_info( InPpsFlag ) { | Descriptor |
| --- | --- |
| num_tile_columns_minus1[ InPpsFlag ] | ue(v) |
| num_tile_rows_minus1[ InPpsFlag ] | ue(v) |
| uniform_spacing_flag[ InPpsFlag ] | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ InPpsFlag ][ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ InPpsFlag ][ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag[ InPpsFlag ] | u(1) |
| } |  |

Correspondingly we propose to change the syntax from the SPS shown in Table 6 to the syntax shown in Table 7.

Table 6 – From the SPS syntax

| num_tile_columns_minus1 | ue(v) |
| --- | --- |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |

Table 7 - Proposed SPS syntax

| tile_info( 0 ) |  |
| --- | --- |

In the slice header we propose to add a flag, use_tile_info_from_pps_flag, that indicates if tile info from the PPS is used or not (if tile info from the PPS is not used, tile info from the SPS is used). The flag is only present in the slice header if there is tile info both in SPS and in PPS (i.e. tile_info_present_in_pps_flag is equal to 1) otherwise it is inferred to be equal to 0. The flag is shown, together with the other proposed changes in the slice header in Table 9. The SPS/PPS dependency problem is solved since there is no dependency on tiles_or_entropy_coding_sync_idc in the proposed PPS syntax in Table 4.

## Decoder parallelism

A problem with decoder parallelism with tiles is that a decoder will not know whether it can decode a stream in parallel. In order for parallel decoding to work, there needs to be tiles with entry points present in every picture. In the current design, there is no way for an encoder to signal a guarantee to the decoder that there are tiles and entry points present in every picture of the coded video sequence.

We propose to include a flag in the SPS, tiles_fixed_structure_flag, to indicate that the tile info from the SPS is used for all pictures in the coded video sequence. This means that when tiles_fixed_structure_flag is equal to 1, tile info from PPS cannot be used, thus there is no need to signal the flag tile_info_present_in_pps_flag, instead it could be inferred to be equal to 0 (its conditional presence on tiles_fixed_structure_flag  is shown in Table 9).

We further propose indicators in the SPS to indicate whether all tiles do have entry point offsets or entry point markers. It is not until the presence of entry points is guaranteed that a decoder can really make use of (and depend on) the tile structure for parallelism, especially for the case when tiles_fixed_structure_flag is equal to 1. With a fixed tile structure and entry points for all tiles, a decoder can divide the workload of decoding each frame on multiple threads without risking that all of a sudden there are no entry points or e.g. for some tiles there are 16 tiles while others only have two.

We propose the addition of the following three SPS flags:

Table 8 - Proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tiles_fixed_structure_flag | u(1) |
| entry_point_offsets_for_all_tiles_flag | u(1) |
| entry_point_markers_for_all_tiles_flag | u(1) |

tiles_fixed_structure_flag equal to 1 specifies that tile_info() shall not be present in any picture parameter set with a seq_parameter_set_id that references this sequence parameter set. tiles_fixed_structure_flag equal to 0 indicates that tile_info() may be present in a picture parameter set with a seq_parameter_set_id that references this sequence parameter set.

entry_point_offsets_for_all_tiles_flag equal to 1 indicates that the codeword entry_point_offset[ i ] is present for every tile in the coded video sequence except for tiles that are the first tile in a slice.

entry_point_markers_for_all_tiles_flag equal to 1 indicates that there is an entry_point_marker_two_3bytes codeword indicating the start of every tile in the coded video sequence except for tiles that are the first tile in a slice.

We further propose to include tile id code words together with the entry points if tiles_or_entropy_coding_sync_idc  is equal to 1 and entry points are not present for every tile in the coded video sequence. Note that the current CD text specifies that a tile id code word is present for entry point markers if WPP is used, this is clearly a bug in the text.

## Entry point offsets

No tile index is signaled when entry point offsets are used. This is a problem in the case where not every tile has an entry point. Assume that there are three tiles in a slice, tile A, B, and C but there is only one entry point. The decoder may try to use the entry point in order to decode the picture in two parallel threads, but that is not possible since the decoder will not know whether tile B or C is indicated and therefore not the spatial location of the tile.

We propose to mandate a codeword for each entry point offset so that a parallel decoder can determine the spatial location of the tile and decode the stream in parallel.

We propose the following syntax and semantics:

Table 9 - Proposed slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| if( !tiles_fixed_structure_flag && tile_info_present_in_pps_flag ) |  |
| use_tile_info_from_pps_flag | u(1) |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) { |  |
| if( tiles_or_entropy_coding_sync_idc == 1 && !entry_point_offsets_for_all_tiles_flag ) |  |
| tile_id_offset_minus1[ i ] | u(v) |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

tile_id_offset_minus1[ i ] specifies an ID of a tile in raster scan order and shall be represented by Ceil( Log2( NumTilesInPic − 1 ) ) bits. When tile_id_offset_minus1 is not present it shall be inferred to be equal to i.

Currently the entry point offset syntax is shared between tiles and WPP. Tile id should not be included for WPP.

## Entry point markers

The current specification contains tile ID when markers are used, the codeword tile_idx_minus_1 is u(v) but the length is undefined. We propose to change the name to tile_id_minus1 to align it with other codewords. We propose to replace the following current text

tile_idx_minus_1 specifies the TileID in raster scan order.  The first tile in the picture shall have a TileID of 0.  The value of tile_id_minus1 shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1.

by

tile_id_marker_minus1 specifies an ID of a tile in raster scan order and shall be represented by Ceil( Log2( NumTilesInPic − 1 ) ) bits.  tile_id_marker_minus1 shall be equal to TileId[ CtbAddrTS ] – 1.

## Tile process currently specified in semantics section

There is some text in the tile semantics that fits better in the process section of the specification (there is an editorial note in the CD about this).

We propose that the variables ColumnWidth, ColumnWidthInLumaSamples, ColBD, RowHeight and RowBd, currently specified in the semantics section, are moved to a new section "Decoding process for tiles" as shown in Section 3 of this proposal

# Proposed syntax and semantics

- 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tiles_fixed_structure_flag | u(1) |
| entry_point_offsets_for_all_tiles_flag | u(1) |
| entry_point_markers_for_all_tiles_flag | u(1) |
| tile_info( 0 ) |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

- 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { [Ed. (JB): PPS parsing dependency on syntax element in SPS.] |  |
| tile_info_present_in_pps_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_in_pps_flag ) { |  |
| tile_info( 1 ) |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else … |  |
| … |  |
| } |  |

- 7.3.2.3 Tile info syntax

| tile_info( InPpsFlag ) { | Descriptor |
| --- | --- |
| num_tile_columns_minus1[ InPpsFlag ] | ue(v) |
| num_tile_rows_minus1[ InPpsFlag ] | ue(v) |
| uniform_spacing_flag[ InPpsFlag ] | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ InPpsFlag ][ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ InPpsFlag ][ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag[ InPpsFlag ] | u(1) |
| } |  |

- 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| if( !tiles_fixed_structure_flag && tile_info_present_in_pps_flag ) |  |
| use_tile_info_from_pps_flag | u(1) |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) { |  |
| if( tiles_or_entropy_coding_sync_idc == 1 && !entry_point_offsets_for_all_tiles_flag ) |  |
| tile_id_offset_minus1[ i ] | u(v) |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

- 7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| … |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| if(tiles_or_entropy_coding_sync_idc == 1 && !entry_point_markers_for_all_tiles_flag ) |  |
| tile_idx_marker_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

- 7.4.2 Raw byte sequence payloads and RBSP trailing bits semantics

- 7.4.2.1 Sequence parameter set RBSP semantics

tiles_fixed_structure_flag equal to 1 specifies that tile_info() shall not be present in any picture parameter set with a seq_parameter_set_id that references this sequence parameter set. tiles_fixed_structure_flag equal to 0 indicates that tile_info() may be present in a picture parameter set with a seq_parameter_set_id that references this sequence parameter set.

entry_point_offsets_for_all_tiles_flag equal to 1 indicates that the codeword entry_point_offset[ i ] is present for every tile in the coded video sequence except for tiles that are the first tile in a slice.

entry_point_markers_for_all_tiles_flag equal to 1 indicates that there is an entry_point_marker_two_3bytes codeword indicating the start of every tile in the coded video sequence except for tiles that are the first tile in a slice.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

One or both of the following conditions shall be fulfilled for each slice and tile:

–	All coded blocks in a slice belong to the same tile.

–	All coded blocks in a tile belong to the same slice.

NOTE 2 – Within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices.

uniform_spacing_flag equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width[ i ] and row_height[ i ].

column_width[ i ] specifies the width of the i-th tile column in units of coding treeblocks.

row_height[ i ] specifies the height of the i-th tile row in units of coding treeblocks.

Values of ColumnWidth[ i ], specifying the width of the i-th tile column in units of coding treeblocks, and the values of ColumnWidthInLumaSamples[ i ], specifying the width of the i-th tile column in units of luma samples, are derived as follows:

for( i = 0; i <= num_tile_columns_minus1; i++ ) {
	if( uniform_spacing_flag )
		ColumnWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbs ) / ( num_tile_columns_minus1 + 1 ) – 	(1)
						( i * PicWidthInCtbs ) / ( num_tile_columns_minus1 + 1 )
	else
		ColumnWidth[ i ] = column_width[ i ]
	ColumnWidthInLumaSamples[ i ] = ColumnWidth[ i ] << Log2CtbSize
}

Values of RowHeight[ i ], specifying the height of the i-th tile row in units of coding treeblocks, are derived as follows:

for( i = 0; i <= num_tile_rows_minus1; i++ )
	if( uniform_spacing_flag )
		RowHeight[ i ]  = ( ( i + 1 ) * PicHeightInCtbs ) / ( num_tile_rows_minus1 + 1 ) – 	(2)
						( i * PicHeightInCtbs ) / ( num_tile_rows_minus1 + 1) 
	else
		RowHeight[ i ] = row_height[ i ]

Values of ColBd[ i ], specifying the location of the left column boundary of the i-th tile column in units of coding treeblocks, are derived as follows:

for( ColBd[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )	(3)
		ColBd[ i + 1 ] = ColBd[ i ] + ColumnWidth[ i ]

Values of RowBd[ i ], specifying the location of the top row boundary of the i-th tile row in units of coding treeblocks, are derived as follows:

for( RowBd[ 0 ] = 0, i = 0; i <= num_tile_rows_minus1; i++ )	(4)
	RowBd[ i + 1 ] = RowBd[ i ] + RowHeight[ i ]

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across tile boundaries. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

- 7.4.2.2 Picture parameter set RBSP semantics

tile_info_present_in_pps_flag equal to 1 specifies that tile_info() is present in the picture parameter set. tile_info_present_in_pps_flag equal to 0 specifies that tile_info() is not present in the picture parameter set.

tile_info_present_flag equal to 1 specifies that the syntax elements num_tile_columns_minus1, num_tile_rows_minus1 are present and the syntax elements uniform_spacing_flag, column_width[ i ], and row_height[ i ] may be present in the picture parameter set. tile_info_present_flag equal to 0 specifies that syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], and row_height[ i ] are not present in the picture parameter set.

tile_control_present_flag equal to 1 specifies that the syntax element loop_filter_across_tiles_enabled_flag is present in the picture parameter set.  tile_control_present_flag equal to 0 specifies that syntax element loop_filter_across_tiles_enabled_flag is not present in the picture parameter set.

7.4.2.3 Tile info semantics

Tile info may be present in a sequence parameter set or in a picture parameter set. If tile info is present in a sequence parameter set InPpsFlag shall be equal to 0, otherwise it shall be equal to 1.

num_tile_columns_minus1[ InPpsFlag ] plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1[ InPpsFlag ] plus 1 specifies the number of tile rows partitioning the picture.

uniform_spacing_flag[ InPpsFlag ] equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width[ InPpsFlag ][ i ] and row_height[ InPpsFlag ][ i ].

column_width[ InPpsFlag ][ i ] specifies the width of the i-th tile column in units of coding treeblocks.

row_height[ InPpsFlag ][ i ] specifies the height of the i-th tile row in units of coding treeblocks.

loop_filter_across_tiles_enabled_flag[ InPpsFlag ] equal to 1 specifies that in-loop filtering operations are performed across tile boundaries. loop_filter_across_tiles_enabled_flag[ InPpsFlag ] equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

- 7.4.3 Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, idr_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, and num_long_term_pics and use_tile_info_from_pps_flag shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

…

use_tile_info_from_pps_flag equal to 1 specifies that tile info from the picture parameter set is used for the current picture. use_tile_info_from_pps_flag equal to 0 specifies that tile info from the sequence parameter set is used for the current picture. When use_tile_info_from_pps_flag is not present it shall be inferred to be equal to 0.

The variable NumTilesInPic is derived as follows.

NumTilesInPic = ( num_tile_columns_minus1[ use_tile_info_from_pps_flag ] + 1) * 
                                           ( num_tile_rows_minus1[ use_tile_info_from_pps_flag ] + 1)

tile_id_offset_minus1[ i ] specifies an ID of a tile in raster scan order and shall be represented by Ceil( Log2( NumTilesInPic − 1 ) ) bits. When tile_id_offset_minus1 is not present it shall be inferred to be equal to i.

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice NAL unit consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice NAL unit, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice NAL unit, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice NAL unit. [Ed. (JB): Perhaps better to introduce variables for the start and end points of the slice.  Can BitstreamTable[] be used? There seems to be a problem with the existing test for the last subset in the tile, because entropy_point_offset[k] does not exist, and need instead to refer to the coded slice length.]

NOTE – The NAL unit header and the slice header of a coded slice NAL unit are always included in subset 0.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of one or multiple complete tiles, and the number of subsets shall be equal to or less than the number of tiles in the slice entry_point_offset[ i ] shall be equal to the offset in bytes, relative to the start of the slice header, for the first treeblock, with address CtbAddrTS, such that TileId[ CtbAddrTS ] = tile_id_offset_minus1[ i ] + 1.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, subset k, for each of all the possible k values, shall contain all bits to be used during the initialization process for the current bitstream pointer k.

- 7.4.4 Slice data semantics

entry_point_marker_two_3bytes is a fixed-value sequence of 3 bytes equal to 0x000002.  This syntax element is called an entry marker prefix.

tile_idx_marker_minus_1 specifies the TileID an ID of a tile in raster scan order and shall be represented by Ceil( Log2( NumTilesInPic − 1 ) ) bits. The first tile in the picture shall have a TileID of 0.  The value of tile_idx_minus_1 shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1. tile_id_marker_minus1 shall be equal to TileId[ CtbAddrTS ] – 1.

8.3.4 Decoding process for tiles

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 8.3.5.

The variable X is set equal to use_tile_info_from_pps_flag.

Values of ColumnWidth[ i ], specifying the width of the i-th tile column in units of coding treeblocks, and the values of ColumnWidthInLumaSamples[ i ], specifying the width of the i-th tile column in units of luma samples, are derived as follows:

for( i = 0; i <= num_tile_columns_minus1; i++ ) {
	if( uniform_spacing_flag )
		ColumnWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbs ) / ( num_tile_columns_minus1[ X ] + 1 ) – 	(5)
						( i * PicWidthInCtbs ) / ( num_tile_columns_minus1[ X ] + 1 )
	else
		ColumnWidth[ i ] = column_width[ X ] [ i ]
	ColumnWidthInLumaSamples[ i ] = ColumnWidth[ i ] << Log2CtbSize
}

Values of RowHeight[ i ], specifying the height of the i-th tile row in units of coding treeblocks, are derived as follows:

for( i = 0; i <= num_tile_rows_minus1; i++ )
	if( uniform_spacing_flag )
		RowHeight[ i ]  = ( ( i + 1 ) * PicHeightInCtbs ) / ( num_tile_rows_minus1[ X ] + 1 ) – 	(6)
						( i * PicHeightInCtbs ) / ( num_tile_rows_minus1[ X ] + 1) 
	else
		RowHeight[ i ] = row_height[ X ] [ i ]

Values of ColBd[ i ], specifying the location of the left column boundary of the i-th tile column in units of coding treeblocks, are derived as follows:

for( ColBd[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1[ X ]; i++ )	(7)
		ColBd[ i + 1 ] = ColBd[ i ] + ColumnWidth[ i ]

Values of RowBd[ i ], specifying the location of the top row boundary of the i-th tile row in units of coding treeblocks, are derived as follows:

for( RowBd[ 0 ] = 0, i = 0; i <= num_tile_rows_minus1[ X ]; i++ )	(8)
	RowBd[ i + 1 ] = RowBd[ i ] + RowHeight[ i ]

# Proposed text for tiles_fixed_structure_flag as a VUI flag

VUI parameters syntax:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| low_delay_hrd_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

tiles_fixed_structure_flag equal to 1 indicates that each picture parameter set that is active in the coded video sequence has the same value of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present. tiles_fixed_structure_flag equal to 0 indicates that tiles syntax elements in different picture parameter sets may or may not have the same value. When the tiles_fixed_structure_flag syntax element is not present, it is inferred to be equal to 0.

NOTE – The signalling of tiles_fixed_structure_flag equal to 1 is a guarantee to a decoder that each picture in the coded video sequence has the same number of tiles distributed in the same way which might be useful for workload allocation in the case of multi-threaded decoding.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).