| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0235 m24477 |
| --- | --- |

| Title: | AHG12: Slice header extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

An extension field in the slice header is proposed to enable extension of the slice header in future profiles (or extensions) of HEVC. The extension field is preceded by a length parameter in order to provide a legacy decoder with the information needed to find the start of the slice data. The presence of the extension field in the slice header is conditioned on a flag in the SPS in order to not add any bit overhead in the slice headers for the current profile(s) of HEVC.

# Introduction

In the CD of HEVC, all parameter sets have an extension field at the end of their syntax table. The presence of the extension fields are controlled by extension data flags. It is specified that a decoder shall ignore the extension data (“Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard”). It is further specified that an encoder has to set the flag to 0 (“shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard”).

The extension fields provide HEVC with the possibility to add new syntax elements to the parameter sets in the future while still being backwards compatible with legacy decoders as they will simply ignore the new syntax elements. Since the NumBytesInRBSP is known for all parameter sets there is no problem for a legacy decoder to calculate the length of an extension field (or detect the end of it).

Example from sequence parameter set:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Problem

The slice header of HEVC does not contain an extension field.  In future extensions of HEVC (scalability extension, multiview extension etc.) it might be useful to be able to add syntax elements to the slice header that a legacy decoder ignores but a decoder that conforms to the extension makes use of. It would be an advantage to be able to add such syntax elements as straight-forward as possible (e.g. without having to add prefix NAL units, new NAL unit types or SEI messages).

One problem is that it is not possible to simply add an extension field in the same manner as in the parameter sets since the length of the slice header is not known. A slice consists of slice header and slice data and it is only the total length of the slice that is known (through NumBytesInRBSP) not the individual lengths of slice header and slice data respectively. Thus, if new slice header syntax elements were added in an extension to HEVC a legacy decoder would not be able to conclude where the slice header ends and the slice data begins.

Slice layer RBSP syntax:

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

Another problem with extensibility in the slice header is the bit cost. Having to spend even a single bit can be somewhat costly if it has to be done in all slice headers in all HEVC bitstreams.

# Proposed solution

We propose to add a slice header extension field which is conditioned on a flag in the SPS and which includes a length parameter representing the length of the actual slice header extension data.

Proposed additional syntax element in SPS:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_header_extension_present_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Proposed slice header extension:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_flag | u(1) |
| } |  |
| } |  |

The length field provides the legacy decoder with sufficient information for finding the start of the slice data. Putting the slice header extension present flag in the SPS makes the bit cost for the extensibility very small. For bitstreams that conform to the current profile(s) of HEVC the cost will be a single bit per SPS.

The semantics for the three new syntax elements will be in line with the semantics of the extension fields of the parameter sets:

slice_header_extension_present_flag equal to 0 specifies that no slice header extension syntax elements are present in the slice headers of the coded video sequence. slice_header_extension_present_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for slice_header_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

slice_header_extension_length specifies the length of the slice header extension data in bits, not including the bits used for signalling slice_header_extension_length itself. When present, the slice_header_extension_length also specifies the start position of the slice_data( ) as being equal to the bit position of the first bit after the slice_header_extension_length syntax element + slice_header_extension_length number of bits.

slice_header_extension_data_flag may have any value. Decoders shall ignore the value of slice_header_extension_data_flag. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).