| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0236 m24478 |
| --- | --- |

| Title: | TLA picture restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that the current CD text allows using the TLA picture type even though performing a temporal layer switching at the TLA picture may result in a non-conforming bitstream. This means that a media-aware network element (MANE) currently cannot only look at the NAL unit header to decide where temporal switching is possible, it must also decode reference picture sets. This contribution proposes to add a bitstream conformance requirement for temporal layer access pictures to ensure that TLA-only based temporal layer switching does not break bitstream conformance.

# Problem Statement

At the eighth JCT-VC meeting in San Jose, the temporal layer access (TLA) picture type was included into the CD of HEVC. A corresponding NAL unit type (3) was also included. The TLA picture type can be used to indicate temporal layer switching points. Currently the switching point property of the TLA picture is only enforced by the definition of the TLA picture:

temporal layer access (TLA) picture: A coded picture for which each slice has nal_unit_type equal to 3; the TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order.

With this definition as the only TLA restriction, it is allowed to include a reference picture from the same or higher temporal layer in the reference picture set of a TLA picture. It is also allowed to include such a reference picture in a reference picture list of the TLA picture (by setting used_by_curr_pic_flag of the picture to 1) as long as it is not actually used for inter prediction. However if a media-aware network element (MANE) performs temporal layer switching at that point the reference picture in the RPS from the same or higher temporal layer as the TLA picture would not be in the DPB. Thus the third RPS bitstream conformance requirement would be broken:

- When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture".

The result is that it is possible to signal the TLA picture type even though a temporal layer switching at the TLA picture results in a non-conforming bitstream. This means that a media-aware network element (MANE) currently cannot only look at the NAL unit header to decide where temporal switching is possible, it must also decode reference picture set. This was never the intension.

Note that the decoding process does not specify what a decoder shall do if an entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr is equal to "no reference picture" (since it is prohibited).

# Proposal

We propose to solve this problem by strengthening the requirement on TLA pictures by adding the following bitstream conformance requirement on the reference picture set (RPS):

- When the current picture is a TLA picture, there shall be no reference picture included in the reference picture set with temporal_id greater than or equal to the temporal_id of the current picture.

The proposed requirement would solve the problem by prohibiting any picture with the same or higher temporal_id to be included in the reference picture set of a TLA picture. With the requirement, a MANE would only have to inspect the NAL unit headers in order to know where temporal layer switching can be done.

The proposed requirement would be analog to the situation for the CRA picture type where the current CD definition states:

clean random access (CRA) picture: A coded picture containing only I slices and for which each slice has nal_unit_type equal to 4; all coded pictures that follow the CRA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the CRA picture either in decoding order or output order; and any picture that precedes the CRA picture in decoding order also precedes the CRA picture in output order.

And one of the RPS requirements from the current CD text is:

- There shall be no reference picture included in the reference picture set that precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

# Proposed text

8.3.2	Decoding process for reference picture set

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr.

- When the current picture is a TLA picture, there shall be no reference picture included in the reference picture set with temporal_id greater than or equal to the temporal_id of the current picture.

- There shall be no reference picture included in the reference picture set that precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A reference picture cannot be included in more than one of the five reference picture set lists.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).