| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0237 |
| --- | --- |

| Title: | Specifying entry points to facilitate different decoder implementations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Peisong Chen Broadcom Corporation 16340 West Bernardo Dr San Diego, CA 92127 USA | Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-858-521-4835 peisongc@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends mandating the entry point of every tile and every wavefront substream be signaled instead of the definition in the present draft of the standard which allows an encoder to selectively choose which entry points to transmit.  It claims that different decoder implementations may expect or require the entry points of every tile or wavefront substream to facilitate efficient decoding in their architecture.  An example is given where a single core decoder performing raster scan decoding of tiles would need every entry point to facilitate efficient decoding.  Another example is provided where a multi-core decoder may have difficulties decoding a stream generated with a number of entry points that is not well matched to the number of cores it has available for decoding.  Changes to the text are provided to mandate transmission of every entry point as well as general cleanup of tile processing syntax and semantics.

# Introduction

A harmonized method for signaling entry points of tiles and wavefront substreams was presented in JCTVC-H0556 [1] and adopted after some refinements in JCTVC-H0737 [2].

The number of entry points is specified in Section 7.4.3 (“Slice header semantics”) of the present draft of the standard [3]

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

Note that this definition allows an encoder to selectively choose to transmit entry points of every tile or wavefront substream.  That is, an encoder may choose to transmit none, some or all of the entry points of tiles or wavefront substreams in a bitstream.

# Discussion

The definition in the present draft of the standard was designed to attempt to gain some coding efficiency in the case where an encoder using N cores could provide M entry points (where M<N) for a decoder using M cores.  This may be feasible for unicast applications with back channel initialization and communication.  However, this is problematic for many other applications including multicast environments and those where the encoder does not have a priori knowledge of a decoder’s architecture.  In these scenarios, there is a significant possibility of complicating interoperability by allowing this flexibility.

For example, a single core decoder attempting to support tiles while performing raster scan decoding would require all entry points to facilitate efficient decoding.  Otherwise, substantial buffering, CABAC decoding and delay would be needed to determine the starting locations of subsequent tiles in a tile row).

Another example is a stream generated with four wavefronts but only two entry points.  Such a stream would be problematic for a four-core decoder to decode if it was not given the proper entry points.

# Proposal

## Changes to num_entry_point_offsets semantics

It is proposed that is  mandatory to signal the entry point of every tile and every wavefront substream in a bitstream.  This can be accomplished by the following changes to the text:

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

## Changes to entry_point_offset semantics

It is also recommended to make the following changes to the definition of entry_point_offset to make its definition consistent with the above changes:

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice NAL unit consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice NAL unit, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice NAL unit, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice NAL unit.

NOTE – The NAL unit header and the slice header of a coded slice NAL unit are always included in subset 0.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of a one or multiple complete tiles, and the number of subsets shall be equal to or less than the number of tiles in the slice.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, subset k, for each of all the possible k values, shall contain all bits to be used during the initialization process for the current bitstream pointer k.

- The removal of entry_point_offset[k-1] appears to be a typo in the text.

- Changing the term “of one or multiple complete tiles” to “of a complete tile” is done to reflect that each tile must have an associated entry point regardless of whether it is part of a slice consisting of multiple complete tiles.

## Clarification and changes to slice header syntax

The current semantics for entry points appears to be sufficient when there is only one slice in a picture.  However, it is unclear what the behavior should be when there are multiple slices in a picture.  It is assumed that the underlying idea is to transmit all the entry points at the beginning of a picture to facilitate a decoder being able to quickly determine entry points, e.g. to send the appropriate bits to parallel processing units.  In that case, transmitting all the entry points with the very first slice header seems appropriate but it is not necessary to repeat this information for subsequent slice headers.  The current text does not distinguish whether the entry point syntax is transmitted only with the first slice header.  Therefore, the following additions to the syntax are proposed:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| … |  |
| … |  |
| if ((tiles_or_entropy_coding_sync_idc > 0 ) && (first_slice_in_pic_flag = = 0) ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

# References

- Hendry, S. Jeong, S. W. Park, B. M. Jeon, A. Segall, K. Misra, “AHG4: Harmonized Method for Signalling Entry Points of tiles and WPP Substreams”, JCTVC-H0566, Feb. 2012.

- Y.-K. Wang, A. Segall, M. Horowitz, Hendry, W. Wan, F. Henry, T. Lee, “Text for tiles, WPP and entropy slices”, JCTVC-H0737

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK, Feb. 2012.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).