| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0238 |
| --- | --- |

| Title: | Specifying a maximum bound on slices per picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com +1-949-926-5388 bheng@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends reducing the maximum number of slices allowed per picture as the present draft of the standard appears to use a similar formula from the H.264/AVC specification but the H.264/AVC calculation is performed using macroblocks while the HEVC calculation is performed using pixels.  Thus, a significantly higher number of slices compared to H.264/AVC is now possible.  This can be corrected by introducing a scale factor to convert the pixel based calculation to a LCU based calculation similar to H.264/AVC.  However, this proposal also notes that because the formula for the maximum number of slices is directly proportional to pixel resolution, it scales too quickly for implementation looking towards supporting resolutions like 4K and 8K video.  To address this issue, this contribution suggests modifying the SliceRate parameter because supporting the worst-case number of slices is known to be problematic and rarely used in practice for H.264/AVC, thus the formula for HEVC is modified to achieve a better cost/functionality tradeoff.

# Background

The maximum number of slices in the present draft of the standard [1] is constrained for Main profile in section A.4.2.  In particular, note clauses (a) and (b):

a) In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeLuma, fR * MaxLumaPR ) + MaxLumaPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture 0 and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

b) In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture n.

For comparison, the maximum number of slices in the H.264/AVC specification [2] are specified using similar clauses in section A.3.3 of that standard as follows:

a) In bitstreams conforming to the Main, High, High 10, High 4:2:2, High 4:4:4 Predictive, High 10 Intra, High 4:2:2 Intra, High 4:4:4 Intra, or CAVLC 4:4:4 Intra profiles, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables A-1 and A-4, respectively, that apply to picture 0 and PicSizeInMbs is the number of macroblocks in picture 0.

b) In bitstreams conforming to the Main, High, High 10, High 4:2:2, High 4:4:4 Predictive, High 10 Intra, High 4:2:2 Intra, High 4:4:4 Intra, or CAVLC 4:4:4 Intra profiles, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables A-1 and A-4, respectively, that apply to picture n.

Note the similar formulas with HEVC using PicSizeLuma and maxLumaPR which are pixel based values and H.264/AVC using PicSizeInMbs and MaxMBPS which are macroblock based values.

As an example, consider the slice limits for access units n and n-1 with n>0 for a fixed frame-rate 720p60 video sequence coded using 16x16 CTBs:

- HEVC (Level 4.0)

  - MaxLumaPR = 62,668,800 		(from Table A-1)

  - tr( n ) − tr( n – 1) = 1/60			(fixed frame-rate of 60 frames/sec)

  - SliceRate = 60 				(from Table A-3)

  - Maximum number of slices 	= 	MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate

= 	62,668,800*(1/60)/60

= 	17,408

- H.264/AVC (Level 4.0)

  - MaxMBPS = 245,760	 		(from Table A-1)

  - tr( n ) − tr( n – 1) = 1/60			(fixed frame-rate of 60 frames/sec)

  - SliceRate = 60 				(from Table A-4)

  - Maximum number of slices 	= 	MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate

= 	245,760*(1/60)/60

= 	68.27

# Proposal

As the example in the previous section demonstrated, it appears to be problematic to use pixel rate in the formula to bound the maximum number of slices.  It is not straightforward to extend the usage of macroblocks per second from H.264/AVC to HEVC due to the fact that the LCU size can be chosen by the encoder.  If one were to assume the largest LCU size (64x64) to bound the worst-case scenario for a decoder, then the resulting value would be too small for some applications.  More specifically, replacing MaxLumaPR with (MaxLumaPR/(64*64)) in the above formula would yield a maximum of 4.25 slices.  Note that replacing MaxLumaPR with MaxLumaPR/256 where 256 represents a 16x16 LCU comparable to the 16x16 AVC macroblock  results in the same number (68) of maximum slices as AVC for this example.

A high maximum number of slices for H.264/AVC is problematic as it results in significant complexity added to an implementation to handle processing slice headers in the worst-case.  The use of a high number of slices is also rarely used in practice in many applications, thus the formula for HEVC could be refined further to achieve a better cost/functionality tradeoff.

The formula for the maximum number of slices is directly proportional to resolution and as higher resolutions like 4K and 8K video become more prevalent, it may be difficult to imagine that supporting a large number of slices is really necessary so a more gradual increase in the maximum number of slices rather than an increase that scales directly according to the resolution.

It is proposed that the issues discussed above can be remedied by increasing the values of SliceRate in Table A-3 (after the correction factor of 256 is applied above) according to Table 1 below.

Table 1: Comparison of different approaches to bound the maximum number of slices

In Table 1:

- Column D represents the current maximum number of slices for a 60 fps second sequence at the appropriate level.  As shown in the discussion above, an example is a maximum of 17,408 slices for a 720p60 sequence (Level 4)

- Column E represents the maximum number of slices for a 60 fps second sequence at the appropriate level if the pixel based calculations are adjusted assuming a 16x16 LCU.  That is, Column E = Column D / 256.  So a maximum of 68 slices is allowed for a 720p sequence

  - Note the maximum number of slices scales very quickly beyond Level 4 in a linear fashion to the pixel rate.

- Column F represents the proposed new values of slice rate

  - The same value is used for Level 4.0 and Level 4.1 since they have the same pixel rate.

  - SliceRate is increased instead of being held fixed at 24 for levels above Level 4 to provide a more gradual increase in slice rate as resolution increases.

- Column G represents the maximum number of slices for a 60 fps second sequence at the appropriate level if the pixel based calculations are adjusted assuming a 16x16 LCU and the proposed SliceRate values are used instead of the existing values.

  - These values still provide an increasing number of maximum slices as resolution increase but avoid specifying the very large values specified before to achieve a better cost/functionality tradeoff.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK, Feb. 2012.

- ISOIEC 14496-10 | ITU-T Rec. H.264, “Advanced video coding for generic audiovisual services”, Mar. 2010.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).