| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0238r3 |
| --- | --- |

| Title: | Specifying a maximum bound on slices per picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com +1-949-926-5388 bheng@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends reducing the maximum number of slices allowed per picture as the present draft of the standard appears to use a similar formula from the H.264/AVC specification but the H.264/AVC calculation is performed using macroblocks while the HEVC calculation is performed using samples.  Thus, a significantly higher number of slices compared to H.264/AVC is now possible.  This can be corrected by introducing a scale factor to convert the sample based calculation to a LCU based calculation similar to H.264/AVC.  However, this proposal also notes that because the formula for the maximum number of slices is directly proportional to picture resolution, it scales too quickly for implementation looking towards supporting resolutions like 4K and 8K video.  To address this issue, this contribution suggests modifying the SliceRate parameter because supporting the worst-case number of slices is known to be problematic and rarely used in practice for H.264/AVC, thus the formula for HEVC is modified to achieve a better cost/functionality tradeoff.  The last part of this contribution is a proposal to remove the dependency of the formula for maximum number of slices per picture on frame rate.  It is suggested that this is not well suited to the wide variety of possible frame rates and a formula based only on picture resolution is proposed.

# Background

The maximum number of slices in the present draft of the standard [1] is constrained for Main profile in section A.4.2.  In particular, note clauses (a) and (b):

a) In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeLuma, fR * MaxLumaPR ) + MaxLumaPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture 0 and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

b) In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture n.

For comparison, the maximum number of slices in the H.264/AVC specification [2] are specified using similar clauses in section A.3.3 of that standard as follows:

a) In bitstreams conforming to the Main, High, High 10, High 4:2:2, High 4:4:4 Predictive, High 10 Intra, High 4:2:2 Intra, High 4:4:4 Intra, or CAVLC 4:4:4 Intra profiles, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables A-1 and A-4, respectively, that apply to picture 0 and PicSizeInMbs is the number of macroblocks in picture 0.

b) In bitstreams conforming to the Main, High, High 10, High 4:2:2, High 4:4:4 Predictive, High 10 Intra, High 4:2:2 Intra, High 4:4:4 Intra, or CAVLC 4:4:4 Intra profiles, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables A-1 and A-4, respectively, that apply to picture n.

Note the similar formulas with HEVC using PicSizeLuma and maxLumaPR which are sample based values and H.264/AVC using PicSizeInMbs and MaxMBPS which are macroblock based values.

As an example, consider the slice limits for access units n and n-1 with n>0 for a fixed frame-rate 720p60 video sequence coded using 16x16 CTBs:

- HEVC (Level 4.0)

  - MaxLumaPR = 62,668,800 		(from Table A-1)

  - tr( n ) − tr( n – 1) = 1/60			(fixed frame-rate of 60 frames/sec)

  - SliceRate = 60 				(from Table A-3)

  - Maximum number of slices 	= 	MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate

= 	62,668,800*(1/60)/60

= 	17,408

- H.264/AVC (Level 4.0)

  - MaxMBPS = 245,760	 		(from Table A-1)

  - tr( n ) − tr( n – 1) = 1/60			(fixed frame-rate of 60 frames/sec)

  - SliceRate = 60 				(from Table A-4)

  - Maximum number of slices 	= 	MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate

= 	245,760*(1/60)/60

= 	68.27

# Proposal

As the example in the previous section demonstrated, it appears to be problematic to use sample rate in the formula to bound the maximum number of slices.  It is not straightforward to extend the usage of macroblocks per second from H.264/AVC to HEVC due to the fact that the LCU size can be chosen by the encoder.  If one were to assume the largest LCU size (64x64) to bound the worst-case scenario for a decoder, then the resulting value would be too small for some applications.  More specifically, replacing MaxLumaPR with (MaxLumaPR/(64*64)) in the above formula would yield a maximum of 4.25 slices.  Note that replacing MaxLumaPR with MaxLumaPR/256 where 256 represents a 16x16 LCU comparable to the 16x16 AVC macroblock  results in the same number (68) of maximum slices as AVC for this example.

A high maximum number of slices for H.264/AVC is problematic as it results in significant complexity added to an implementation to handle processing slice headers in the worst-case.  The use of a high number of slices is also rarely used in practice in many applications, thus the formula for HEVC could be refined further to achieve a better cost/functionality tradeoff.

The formula for the maximum number of slices is directly proportional to resolution and as higher resolutions like 4K and 8K video become more prevalent, it may be difficult to imagine that supporting a large number of slices is really necessary so a more gradual increase in the maximum number of slices rather than an increase that scales directly according to the resolution.

It is proposed that the issues discussed above can be remedied by increasing the values of SliceRate in Table A-3 (after the correction factor of 256 is applied above) according to Table 1 below.

Table 1: Comparison of different approaches to bound the maximum number of slices

In Table 1:

- Column D represents the current maximum number of slices for a 60 fps second sequence at the appropriate level.  As shown in the discussion above, an example is a maximum of 17,408 slices for a 720p60 sequence (Level 4)

- Column E represents the maximum number of slices for a 60 fps second sequence at the appropriate level if the sample based calculations are adjusted assuming a 16x16 LCU.  That is, Column E = Column D / 256.  So a maximum of 68 slices is allowed for a 720p sequence

  - Note the maximum number of slices scales very quickly beyond Level 4 in a linear fashion to the sample rate.

- Column F represents the proposed new values of slice rate

  - The same value is used for Level 4.0 and Level 4.1 since they have the same sample rate.

  - SliceRate is increased instead of being held fixed at 24 for levels above Level 4 to provide a more gradual increase in slice rate as resolution increases.

- Column G represents the maximum number of slices for a 60 fps second sequence at the appropriate level if the sample based calculations are adjusted assuming a 16x16 LCU and the proposed SliceRate values are used instead of the existing values.

  - These values still provide an increasing number of maximum slices as resolution increase but avoid specifying the very large values specified before to achieve a better cost/functionality tradeoff.

It is also suggested that the formula to calculate maximum number of slices not depend on frame rate.  This is because it is not well suited to the wide variety of possible frame rates with examples including\surveillance video at 1 frame per second to high fidelity sports programming at 60 frames per second and other applications going to even higher bitrates , e.g. 120 frames per second.

For example, assuming all the proposed recommendations listed above were accepted, a  fixed-frame rate 720p60 sequence would only be allowed to have up to 68 slices per picture but the same 720p sequence at 1 frame per second would be allowed to have a maximum of 68*60 = 4080 slices!  The formula is also confusing for variable frame-rate video sequences causing decoders to have difficulty claiming conformance across a wide variety of frame rates.

Therefore, it is proposed that the term “( tr( n ) − tr( n − 1 ) )” be replaced by 1/60 so that the maximum number of slices per picture is determined strictly by the resolution of the picture and that only one equation be used for all acccess units instead of having one for access unit 0 and another for access units n where n>0.  Note that this 1/60 factor can be combined with the 1/256 factor proposed to correct the problem described above with the formula scaling based on picture resolution instead of “macroblock resolution” as in H.264/AVC.  The resulting factor would be 1/60 * 1/256 = 1/15360.

# Proposed text changes

## Section A.4.2

Current text:

- In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeLuma, fR * MaxLumaPR ) + MaxLumaPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture 0 and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

- In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture n.

Proposed text:

- In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeLuma, fR * MaxLumaPR ) + MaxLumaPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture 0 and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

- In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in a picture n is less than or equal to MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ (15360*SliceRate), where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture n.

## Table A.3S

Current text:

| Level number | SliceRate |
| --- | --- |
| 1 | - |
| 2 | - |
| 3 | 22 |
| 3.1 | 60 |
| 4 | 60 |
| 4.1 | 24 |
| 4.2 | 24 |
| 4.3 | 24 |
| 5 | 24 |
| 5.1 | 24 |
| 5.2 | 24 |
| 6 | 24 |
| 6.1 | 24 |
| 6.2 | 24 |

Proposed text:

| Level number | SliceRate |
| --- | --- |
| 1 | - |
| 2 | - |
| 3 | 2230 |
| 3.1 | 60 |
| 4 | 60 |
| 4.1 | 2460 |
| 4.2 | 2490 |
| 4.3 | 2490 |
| 5 | 24150 |
| 5.1 | 24150 |
| 5.2 | 24210 |
| 6 | 24300 |
| 6.1 | 24480 |
| 6.2 | 24640 |

# Proposal Refinement

This contribution has been refined after it was presented in the meeting.  It was suggested to basically adopt this in concept with a revisit on the exact formula and numbers.  This section documents the refined proposal after discussion with other experts (S. Wenger, T.K. Tan, M. Zhou).

One suggestion was that since the removal of frame rate from the formula was generally accepted that the formula should use MaxLumaFS (max luma picture size in samples)  instead of MaxLumaPR (max luma pixel rate in samples/sec) from Table A-1 in [1].  This avoids the problem of compensating for frame rate and removes the 1/60 factor used previously.

Another suggestion was that some constraint also be applied to levels 1 and 2 which previously did not have any constraint.

There was additional comments made after the second presentation and this version of the document captures those suggested changes:

- Directly constrain the max number of slices per picture instead of using a formula.

- Round the values to appropriate numbers

- Specify higher values (16) for levels 1 and 2

- Add the additional level(s) adopted earlier in the day

  - Proposed text changes

Current text in Section A.4.2:

- In bitstreams conforming to the Main profile, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( Max( PicSizeLuma, fR * MaxLumaPR ) + MaxLumaPR * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture 0 and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture 0.

- In bitstreams conforming to the Main profile, the difference between consecutive removal time of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to MaxLumaPR * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where MaxLumaPR and SliceRate are the values specified in Table A-1 and Table A-3, respectively, that apply to picture n.

Proposed text in Section A.4.2:

- In bitstreams conforming to the Main profile, the number of slices in a picture is less than or equal to MaxSlicesPerPicture, where MaxSlicesPerPicture is specified in Table A-3.

Current text in Table A-3:

| Level number | SliceRate |
| --- | --- |
| 1 | - |
| 2 | - |
| 3 | 22 |
| 3.1 | 60 |
| 4 | 60 |
| 4.1 | 24 |
| 4.2 | 24 |
| 4.3 | 24 |
| 5 | 24 |
| 5.1 | 24 |
| 5.2 | 24 |
| 6 | 24 |
| 6.1 | 24 |
| 6.2 | 24 |

Proposed text in Table A-3:

| Level number | MaxSlicesPerPicture |
| --- | --- |
| 1 | 16 |
| 2 | 16 |
| 2.x | 20 |
| 3 | 30 |
| 3.1 | 40 |
| 4 | 75 |
| 4.1 | 75 |
| 4.2 | 75 |
| 4.3 | 75 |
| 5 | 200 |
| 5.1 | 200 |
| 5.2 | 200 |
| 6 | 600 |
| 6.1 | 600 |
| 6.2 | 600 |

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK, Feb. 2012.

- ISOIEC 14496-10 | ITU-T Rec. H.264, “Advanced video coding for generic audiovisual services”, Mar. 2010.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).