| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0240 |
| --- | --- |

| Title: | CE2.d.1: Cross-verification of Number of regions in RA mode (JCTVC-I0041) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peisong Chen Broadcom Corporation 16340 West Bernardo Dr. San Diego, CA 92127 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-858-521-4835 peisongc@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE2.d.1: Number of regions in RA mode (JCTVC-I0041). CE2.d.1 is setup to evaluate the performance of RA mode in picture based ALF of HM-6.0. There are two configurations: 1/16 size and 1/64 size. This contribution crosschecks the 1/64 size results. Compared to picture-based ALF of HM-6.0, there is 0.3%, 0.5%, 0.3% and 0.4% loss for AI-HE10, RA-HE10, LB-HE10 and LP-HE10 respectively. The BD rate results match the ones reported by proponents. One possible reason for the loss is that the regions are specified by run coding and copy row flag.

# Introduction

In the picture-based ALF of HM-6.0, 16 filters are used and BA mode is enabled for filter selection. In this test, a picture is split into 64 regions and each region has one filter.

# Experimental results

The CE2 common test condition is used [1]. The anchor is HM-6.0 (picture-based ALF). The experimental results are summarized in the following tables. The results match the ones reported by proponents. The encoder generated YUV and decoder generated YUV match also. Please be advised that the runtime may not be accurate.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.5% | 0.2% | 0.1% |
| Class B | 0.2% | 0.1% | 0.1% |
| Class C | 0.2% | 0.1% | 0.0% |
| Class D | 0.2% | -0.1% | -0.1% |
| Class E | 0.3% | 0.2% | 0.2% |
| Overall | 0.3% | 0.1% | 0.1% |
|  | 0.3% | 0.1% | 0.1% |
| Class F | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 178% |  |  |

|  | Random Access HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.9% | 0.0% | -0.2% |
| Class B | 0.5% | -0.2% | 0.0% |
| Class C | 0.3% | 0.0% | -0.1% |
| Class D | 0.3% | -0.2% | -0.1% |
| Class E |  |  |  |
| Overall | 0.5% | -0.1% | -0.1% |
|  | 0.5% | -0.1% | -0.1% |
| Class F | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 161% |  |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4% | -0.3% | -0.3% |
| Class C | 0.4% | -0.3% | -0.6% |
| Class D | 0.2% | -0.2% | -0.1% |
| Class E | 0.1% | 0.6% | -0.6% |
| Overall | 0.3% | -0.1% | -0.4% |
|  | 0.3% | 0.0% | -0.4% |
| Class F | 0.1% | -0.2% | -0.5% |
| Enc Time[%] | 118% |  |  |

|  | Low delay P HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.6% | 0.0% | 0.2% |
| Class C | 0.4% | -0.3% | -0.2% |
| Class D | 0.5% | 0.3% | 0.1% |
| Class E | -0.2% | 0.5% | 0.1% |
| Overall | 0.4% | 0.1% | 0.1% |
|  | 0.4% | 0.1% | 0.1% |
| Class F | 0.0% | -0.4% | -0.1% |
| Enc Time[%] | 121% |  |  |

# Conclusion

The cross-checker’s results completely match with those of proponents.

# References

[1] JCTVC-H1102, “Description of Core Experiment 2 (CE2): Adaptive loop filtering”, 8th Meeting: San Jose, USA, 1-10 February, 2012.