| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0243 |
| --- | --- |

| Title: | CE2.a.1: Cross-verification of Signalling mode change from slice header mode to interleaving mode (JCTVC-I0249) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peisong Chen Broadcom Corporation 16340 West Bernardo Dr. San Diego, CA 92127 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-858-521-4835 peisongc@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE2.a.1: Signalling mode change from slice header mode to interleaving mode. In HM-6.0, ALF parameters are coded either in the APS or the slice header by using variable length code. CE2.a.1 is to investigate interleaving ALF coefficients and control flags in each LCU and encoding them with CABAC. In the test, besides interleaving, the original merge up and merge left flags are removed and each LCU only uses one filter. Compared to LCU-based ALF of HM-6.0, there is 0.0%, 0.1%, 0.1% and 0.2% improvement for AI-HE10, RA-HE10, LB-HE10 and LP-HE10 respectively. The BD rate results match the ones reported by proponents.

# Introduction

In the LCU-based ALF of HM-6.0, ALF parameter is sent in the slice header. ALF coefficients and control flags are coded by variable length code.  CE2.a.1 is to investigate interleaving ALF coefficients and control flags in each LCU and encoding them with CABAC. Besides interleaving, the original merge up and merge left flags are removed and each LCU only uses one filter.

# Experimental results

The CE2 common test condition is used [1]. The anchor is HM-6.0 (LCU-based ALF). The experimental results are summarized in the following tables. The results match the ones reported by proponents. The encoder generated YUV and the decoder generated YUV match also. Please be advised that the runtime may not be accurate.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.3% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Class F | -0.1% | 0.3% | 0.3% |
| Enc Time[%] | 159% |  |  |

|  | Random Access HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.1% | 0.2% |
| Class B | -0.1% | 0.3% | 0.1% |
| Class C | -0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | -0.1% | 0.2% | 0.1% |
|  | -0.1% | 0.2% | 0.1% |
| Class F | -0.2% | 0.4% | 0.3% |
| Enc Time[%] | 160% |  |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.5% | 0.4% |
| Class C | -0.2% | 0.3% | 0.9% |
| Class D | -0.1% | 0.6% | 0.3% |
| Class E | -0.1% | 0.3% | 0.7% |
| Overall | -0.1% | 0.4% | 0.6% |
|  | -0.1% | 0.5% | 0.6% |
| Class F | -0.2% | -0.4% | 0.5% |
| Enc Time[%] | 146% |  |  |

|  | Low delay P HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 1.0% | 0.6% |
| Class C | -0.2% | 0.5% | 0.6% |
| Class D | -0.1% | 0.2% | 0.0% |
| Class E | -0.3% | 0.9% | 1.4% |
| Overall | -0.2% | 0.7% | 0.6% |
|  | -0.2% | 0.7% | 0.7% |
| Class F | -0.3% | 0.8% | 1.9% |
| Enc Time[%] | 124% |  |  |

# Conclusion

The cross-checker’s results completely match with those of proponents. It is recommended to interleave ALF control flags in each LCU and remove merge left and merge up flags.

# References

[1] JCTVC-H1102, “Description of Core Experiment 2 (CE2): Adaptive loop filtering”, 8th Meeting: San Jose, USA, 1-10 February, 2012.