| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0244 |
| --- | --- |

| Title: | Transform size dependent deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports that the current deblocking method in HM-6.0 sometimes fails to suppress blocking artifacts around 32x32 transform block boundaries since large transform size increases discontinuity with neighboring blocks. To address this issue, it proposes a transform size dependent deblocking method, which increases bS value for 32x32 transform boundary and applies strong filter always for intra 32x32 transform boundary. It also proposes to add a flag in APS and slice header that switches on/off the use of the proposed transform size dependent deblocking method. It is asserted that that the proposed method improves visual qualities at the cost of marginal BD-rate loss. When compared to HM-6.0 anchor, for main configuration, it is reported that BD-rate losses are (Y:0.6%, Cb:0.0%, Cr:0.0%), (Y:0.3%, Cb:0.3%, Cr:0.3%), (Y:0.1%, Cb:-0.1%, Cr:0.0%) and (Y:0.1%, Cb:-0.7%, Cr:-0.7%) for AI, RA, LDB and LDP, respectively. For HE10 configuration, it is reported that BD-rate losses are (Y:0.6%, Cb:0.0%, Cr:0.0%), (Y:0.3%, Cb:0.3%, Cr:0.3%), (Y:0.0%, Cb:0.0%, Cr:0.1%) and (Y:0.1%, Cb:-0.3%, Cr:-0.4%) for AI, RA, LDB and LDP, respectively.

# Introduction

The current deblocking method in HM-6.0 [1] is applied to 8x8 block boundaries when they are either prediction or transform boundaries. The filtering decision depends on bS value that is derived based on CU modes which two 8x8 blocks belong to, existence of non-zero transform coefficients in two blocks, MV, etc. If bS is equal to 0, no filter is applied to the boundary; otherwise, either weak filter or strong filter can be applied to the boundary if some conditions are satisfied.

Although the current deblocking method is quite effective in general, it was observed to sometimes fail to suppress blocking artifacts around 32x32 TU boundaries. Because large transform size increases discontinuity with neighboring blocks, applying deblocking filters regardless of transform size may not be the best approach always. It was especially noted that the blockiness around 32x32 TU boundaries remained even after deblocking filter in some sequences. Figure 1 shows some examples of blocking artifacts that remain even after deblocking filter is applied. The video sequences are from: VQEG (ftp://vqeg.its.bldrdoc.gov/HDTV/NTIA_source/) and TU Munich (riverbed.yuv).

Figure 1(a) Riverbed. HM-6.0 - Frame 31, QP32 (Y/Cb/Cr PSNR: 33.46/38.22/40.96 dB)

Figure 1(b) ControlledBurn. HM-6.0 - Frame 273, QP27 (Y/Cb/Cr PSNR: 37.72/45.64/44.85 dB)

Figure 1(c) WestWindEasy. HM-6.0 - Frame 26, QP32 (Y/Cb/Cr PSNR: 35.60/43.71/46.50 dB)

# Proposed Method

To improve visual qualities, we propose a transform size dependent deblocking method in this contribution. The proposed method increases bS values for 32x32 block boundaries so that there is more chance for these boundaries to be filtered by the strong filter. The high-level flag to switch on/off the use of the transform size dependent deblocking method is also introduced in APS and slice header. Experimental results show that the proposed method improve subjective visual qualities at the cost of marginal BD-rate loss under the common test conditions [2].

Figure 2 shows 32x32 TU boundaries. We define that a 32x32 boundary between two blocks, P and Q, is 32x32 TU boundary if at least one of blocks is located in a 32x32 CU of which square TU depth is equal to 0 or a 64x64 CU of which square TU depth is equal to 1.

Figure 2. 32x32 TU boundaries, red: vertical 32x32 TU boundaries, blue: horizontal 32x32 TU boundaries.

We proposed to filter such boundaries more aggressively. The proposed algorithm introduces changes in two parts of the current deblocking method, bS calculation and (strong/weak) filtering decision.

2.1 Modified bS Calculation

One of P and Q is intra

In the current deblocking method, if at least one of two blocks, P and Q, is intra coded, bS for the boundary between P and Q is set equal to 2. In the proposed algorithm, we additionally check if the boundary is 32x32 boundary first. If so, we further check if it is 32x32 TU boundary as follows:

Case 1: P is intra && Q is intra

We check if both CUs which P and Q belong to are either 32x32 CU of which square TU depth is equal to 0 or 64x64 CU of which square TU depth is equal to 1

Case 2: P is intra

We check if a CU which P belongs to is either 32x32 CU of which square TU depth is equal to 0 or 64x64 CU of which square TU depth is equal to 1

Case 3: Q is intra

We check if a CU which Q belongs to is either 32x32 CU of which square TU depth is equal to 0 or 64x64 CU of which square TU depth is equal to 1

If one of above condition is met, we set bS to 3.

Both P and Q are inter

When both P and Q are inter, the current deblocking filter first check if there is non-zero transform coefficient in one of TUs which P and Q belong to and set bS to 1. In this case, we additionally check if the boundary is 32x32 boundary. If so, we further check if the boundary is 32x32 TU boundary as follows:

We check if one of CUs which P and Q belong to are either 32x32 CU of which square TU depth is equal to 0 or 64x64 CU of which square TU depth is equal to 1

If this condition is met, we set bS to 2.

2.2 Modified Filtering decision

There is no change in Chroma filtering decision. There is no change in Luma filtering decision if bS is equal to or smaller than 2. The proposed method introduces the change only in Luma filtering decision when bS is equal to 3 as follows:

As the current method, the boundary is filtered only when β < d = dpq0 + dpq3. In this case, when bS is equal to 3, the proposed method applies strong filter to the boundary.

2.3 High-level Control Flag

We propose to add a high-level control flag, enable_tu_size_dependent_deblocking_filter_flag, in APS and slice header to turn on/off the proposed transform size dependent deblocking method.

# Experimental Results

The proposed method is implemented in HM-6.0 reference software and evaluated based on the common test condition [2]. The visual qualities by the proposed method are also compared with ones by the current deblocking method.

3.1 Objective Quality Evaluation

W evaluated coding efficiency of the proposed method in terms of BD-rate. Results are summarized in Table 1. When compared to HM-6.0 anchor, for main configuration, BD-rate losses are (Y:0.6%, Cb:0.0%, Cr:0.0%), (Y:0.3%, Cb:0.3%, Cr:0.3%), (Y:0.1%, Cb:-0.1%, Cr:0.0%) and (Y:0.1%, Cb:-0.7%, Cr:-0.7%) for AI, RA, LDB and LDP, respectively. For HE10 configuration, BD-rate losses are (Y:0.6%, Cb:0.0%, Cr:0.0%), (Y:0.3%, Cb:0.3%, Cr:0.3%), (Y:0.0%, Cb:0.0%, Cr:0.1%) and (Y:0.1%, Cb:-0.3%, Cr:-0.4%) for AI, RA, LDB and LDP, respectively.

Table 1. BD-rate results of the proposed method compared to HM-6.0.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.6% | 0.0% | 0.0% | 0.5% | 0.0% | 0.0% |
| Class B | 1.3% | 0.0% | 0.0% | 1.2% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class E | 0.7% | 0.0% | 0.0% | 0.7% | 0.0% | 0.0% |
| Overall | 0.6% | 0.0% | 0.0% | 0.6% | 0.0% | 0.0% |
|  | 0.6% | 0.0% | 0.0% | 0.6% | 0.0% | 0.0% |
| Class F | 1.3% | 0.0% | 0.0% | 0.4% | 0.0% | 0.1% |
| Enc Time[%] | 94% |  |  | 96% |  |  |
| Dec Time[%] | 93% |  |  | 96% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% |
| Class B | 0.5% | 0.3% | 0.3% | 0.5% | 0.3% | 0.3% |
| Class C | 0.1% | 0.6% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class D | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% | 0.3% |
| Class F | 0.4% | 0.7% | 2.2% | 0.3% | 0.9% | 2.5% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.4% | -0.2% | 0.1% | -0.5% | -0.1% |
| Class C | 0.1% | 1.0% | 0.8% | 0.0% | 1.2% | 1.3% |
| Class D | 0.1% | -0.1% | 0.2% | 0.0% | 0.0% | -0.4% |
| Class E | 0.1% | -1.3% | -1.0% | -0.1% | -0.7% | -0.6% |
| Overall | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.3% | 1.3% | 5.6% | 0.4% | 2.1% | 7.9% |
| Enc Time[%] | 95% |  |  | 92% |  |  |
| Dec Time[%] | 91% |  |  | 89% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.5% | -1.0% | 0.1% | -0.5% | -0.8% |
| Class C | 0.1% | -0.1% | 0.3% | 0.0% | 0.6% | 0.7% |
| Class D | 0.0% | -0.5% | -0.5% | 0.0% | -0.1% | -0.4% |
| Class E | 0.1% | -2.1% | -1.9% | 0.1% | -1.4% | -1.5% |
| Overall | 0.1% | -0.7% | -0.7% | 0.1% | -0.3% | -0.4% |
|  | 0.1% | -0.6% | -0.7% | 0.1% | -0.2% | -0.3% |
| Class F | 0.2% | 0.9% | 4.6% | 0.4% | 3.0% | 6.9% |
| Enc Time[%] | 91% |  |  | 95% |  |  |
| Dec Time[%] | 86% |  |  | 94% |  |  |

3.2 Visual Quality Evaluation

We compare visual qualities of the proposed deblocking method and the current HM-6.0 deblocking methods in Figures 3, 4 and 5. Even though the proposed method results in lower PSNR, its visual qualities are noticeably better.

(a) HM-6.0 - Frame 31, QP32 (Y/Cb/Cr PSNR: 33.46/38.22/40.96 dB)

(b) Proposed - Frame 31, QP32 (Y/Cb/Cr PSNR: 33.33/38.16/40.95 dB)

Figure 3. HM-6.0 Deblocking vs. Proposed Deblocking for Riverbed, LDB-Main

(a) HM-6.0 - Frame 273, QP27 (Y/Cb/Cr PSNR: 37.72/45.64/44.85 dB)

(b) Proposed - Frame 273, QP27 (Y/Cb/Cr PSNR: 37.70/45.58/44.70 dB)

Figure 4. HM-6.0 Deblocking vs. Proposed Deblocking for ControlledBurn, LDB-Main

(a) HM-6.0 - Frame 26, QP32 (Y/Cb/Cr PSNR: 35.60/43.71/46.50 dB)

(b) Proposed - Frame 26, QP32 (Y/Cb/Cr PSNR: 35.58/43.74/46.24 dB)

Figure 5. HM-6.0 Deblocking vs. Proposed Deblocking for WestWindEasy, LDB-Main

# Conclusion

The transform size dependent deblocking method is proposed in this contribution. By subjective and objective quality evaluation, it was shown that the proposed method improve visual quality noticeably for some sequences at the cost of marginal BD-rate loss.

# CD Text Changes

7.3.2.5 Adaptive parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| … |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| enable_tu_size_dependent_deblocking_filter_flag | u(1) |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| enable_tu_size_dependent_deblocking_filter_flag | u(1) |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

8.7.1.3 Derivation process of boundary filtering process

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If enable_tu_size_dependent_deblocking_filter_flag is equal to 1, and if the sample p0 or q0 is in a coding unit coded with intra prediction mode and any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- If the sample p0 and q0 are both in a coding unit coded with intra prediction mode and the sample p0 and q0 are both in a 32x32 transform unit.

- Otherwise, if the sample p0 is in a coding unit coded with intra prediction mode and the sample p0 is in a 32x32 transform unit.

- Otherwise, if the sample q0 is in a coding unit coded with intra prediction mode and the sample q0 is in a 32x32 transform unit.

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if enable_tu_size_dependent_deblocking_filter_flag is equal to 1, and if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level and the sample p0 or q0 is in a 32x32 transform unit.

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

8.7.1.4.1 Decision process for luma block edge

- if d is less than β, the following ordered steps apply:

- If bS is equal to 3, the following ordered steps apply:

- The variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

- Otherwise, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0, the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).