| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0245 M24491 |
| --- | --- |

| Title: | LCU based Input order scanning for HASH SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Ranga Ramanujam Srinivasan Mihir Mody Chaitanya Ghone | Tel: Email: | ramanujam@ti.com mihir@ti.com csghone@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The HEVC specification (HM 6.0) specifies Hash SEI to provide a checksum of the decoded picture in the current access unit.  As per specification, this is done as end of frame operation. This proposal is to enable calculation of checksum during LCU decoding process and immediately after in-loop filtering process. This will enable complexity reduction of frame-end computations and reduce memory bandwidth.

# Introduction

Decoded picture hash SEI message provides checksum of the decoded picture in the current access unit as below.

Decoded picture hash SEI message syntax

| Decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| Picture_md5[ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| picture_crc | u(16) |
| } |  |

This message computation requires scanning of lines in raster order and calculating checksum at the end of frame. There is a sequential dependency between lines in raster-order and also between different color components. This computation of checksum can’t be done at LCU level decoding process as input pixel order to check-sum calculation is line based raster.

This when done as frame-end computation requires fetching of output pixel one additional time and so increases memory bandwidth. In case of 1080p@30fps, this requires additional memory bandwidth of ~93 Mbytes/second.  This can be avoided if the operation can be done at LCU decoding processing, which is called ‘on the fly calculation’ in this proposal.

# Proposal

The proposed solution is based on two techniques

- Parallelizing check sum calculation across components:

This is by means of sending one checksum for each component (in total 3 checksum instead of 1 checksum).  The proposed SEI is as below.

Proposed Decoded picture hash SEI message syntax

| Decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| if( hash_type = = 0 ) |  |
| for(comp=0; comp<3; comp++) |  |
| for( i = 0; i < 16; i++) |  |
| Picture_md5[comp][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| for(comp=0; comp<3; comp++) |  |
| picture_crc[comp] | u(16) |
| } |  |

The proposed SEI sends 3 different checksums, one for each component namely Y, Cb and Cr respectively. This enables parallel computation of checksum across components.

- The second technique defines regions to enable LCU based checksum computation by demarking completely filtered pixels during LCU decoding process from partial filtered pixels.  Nine different regions are defined as below.

# As marked in figure, dotted line is LCU aligned boundary, while solid lines indicate completely filtered output pixel within LCU (filtering here indicates de-blocking, SAO and ALF operations). The check-sum computation shall happen only for completely filtered output pixel within an LCU which results in nine different dimensions of completely filter output as shown in the figure above. Within the LCU, the completely filter output pixel is taken in raster-scan order and given as input to checksum engine (which could be MD5 or CRC algorithm).  MD5 checksum or CRC checksum created from one LCU feeds in as input for the next LCU checksum computation and LCUs are processed in raster-scan order. The same process is repeated for all three colour components. Dimension of completed filter pixels in each region is given below.

| Region number | Height | Width |
| --- | --- | --- |
| 0 | Lcu_height - k | Lcu_width - k |
| 1 | Lcu_height - k | Lcu_width |
| 2 | Lcu_height - k | Lcu_width + k |
| 3 | Lcu_height | Lcu_width - k |
| 4 | Lcu_height | Lcu_width |
| 5 | Lcu_height | Lcu_width + k |
| 6 | Lcu_height + k | Lcu_width - k |
| 7 | Lcu_height + k | Lcu_width |
| 8 | Lcu_height + k | Lcu_width + k |

Where Lcu_height and Lcu_width are height and width of Largest Coding Unit respectively.

k is dependent on following in-loop filter tool-set. It is in the range of 0 to 9 based on color component and combination of tools used.

| Filtering Process | Luma | Chroma |
| --- | --- | --- |
| No Inloop de-blocking | 0 | 0 |
| Only De-blocking filter | 4 | 2 |
| Only SAO filter | 1 | 1 |
| Only ALF | 4 | 4 |

# Conclusion

This proposal enables LCU based decoding operation for checksum along with parallel computation for different components and this results in memory bandwidth saving and on-the-fly checksum computation possible.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).