| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0245 M24491 |
| --- | --- |

| Title: | LCU based Input order scanning for HASH SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Ranga Ramanujam Srinivasan Mihir Mody Chaitanya Ghone | Tel: Email: | ramanujam@ti.com mihir@ti.com csghone@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Hash SEI messages are defined to provide checksum of the decoded picture in the current access unit. With the current definition, computation of these checksum can be done only as an end of frame operation, as there is a sequential dependency between different color components and within in a color component, lines taken in raster scan order are input to checksum engine. This proposal suggests breaking dependency between different color components and also proposes a way to do this checksum calculation during LCU decoding process immediately after in-loop filtering process. There by making the checksum calculation as a LCU based operation, instead of a line-based operation. This reduces frame-end computation complexity and reduces memory bandwidth.

# Introduction

Decoded picture hash SEI message provides checksum of the decoded picture in the current access unit as below.

Decoded picture hash SEI message syntax

| Decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| Picture_md5[ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| picture_crc | u(16) |
| } |  |

This message computation requires scanning of lines in raster order and calculating checksum at the end of frame. There is a sequential dependency between lines in raster-order and also between different color components. This computation of checksum can’t be done at LCU level decoding process as input pixel order to check-sum calculation is line based raster.

This when done as frame-end computation requires fetching of decoded picture pixels one additional time and so increases memory bandwidth. In case of 1080p@30fps, this requires additional memory bandwidth of ~93 Mbytes/second.  This can be avoided if the operation can be done at LCU decoding processing, which is called ‘on the fly calculation’ in this proposal.

# Proposal

The proposed solution is based on two techniques,

## Parallelizing check sum calculation across components

This is by means of sending one checksum for each component (in total 3 checksum instead of 1 checksum).  The proposed SEI is as below.

Proposed Decoded picture hash SEI message syntax

| Decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| if( hash_type = = 0 ) |  |
| for(comp=0; comp<3; comp++) |  |
| for( i = 0; i < 16; i++) |  |
| Picture_md5[comp][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| for(comp=0; comp<3; comp++) |  |
| picture_crc[comp] | u(16) |
| } |  |

The proposed SEI sends 3 different checksums, one for each component namely Y, Cb and Cr respectively. This enables parallel computation of checksum across components.

## LCU based Input Order scanning

## The second technique defines regions to enable LCU based checksum computation by demarking completely filtered pixels during LCU decoding process from partial filtered pixels.  Nine different regions are defined as below.

# As marked in figure, dotted line is LCU aligned boundary, while solid lines indicate completely filtered output pixel within LCU (filtering here indicates de-blocking, SAO and ALF operations). The check-sum computation shall happen only for completely filtered output pixel within an LCU which results in nine different dimensions of completely filter output as shown in the figure above. Within the LCU, the completely filter output pixel is taken in raster-scan order and given as input to checksum engine (which could be MD5 or CRC algorithm).  MD5 checksum or CRC checksum created from one LCU feeds in as input for the next LCU checksum computation and LCUs are processed in raster-scan order. The same process is repeated for all three colour components.

| Region number | Height | Width |
| --- | --- | --- |
| 0 | Lcu_height - k | Lcu_width – k |
| 1 | Lcu_height - k | Lcu_width |
| 2 | Lcu_height - k | LastColWidth |
| 3 | Lcu_height | Lcu_width – k |
| 4 | Lcu_height | Lcu_width |
| 5 | Lcu_height | LastColWidth |
| 6 | LastRowHeight | Lcu_width – k |
| 7 | LastRowHeight | Lcu_width |
| 8 | LastRowHeight | LastColWidth |

Where,

WidthModLcuSz = ((floor)(Width + Lcu_width  - 1/Lcu_width) );

LastColWidth = Width – (WidthModLcuSz-1)* Lcu_width + k;

HeightModLcuSz = ((floor)(Height + Lcu_height  - 1/Lcu_height) );

LastRowHeight = Height – (HeightModLcuSz-1)* Lcu_height + k;

Lcu_height and Lcu_width are height and width of Largest Coding Unit respectively. Note that these values will be different for Luma and Chroma.
Width and Height are the display width and display height. Note that these values will be different for Luma and Chroma.

E.g.: For a 176x144 video, with k=1, the table for Luma component shall looks as follows:

| Region number | Height | Width |
| --- | --- | --- |
| 0 | 63 | 63 |
| 1 | 63 | 64 |
| 2 | 63 | 49 |
| 3 | 64 | 63 |
| 4 | 64 | 64 |
| 5 | 64 | 49 |
| 6 | 17 | 63 |
| 7 | 17 | 64 |
| 8 | 17 | 49 |

k is dependent on following in-loop filter tool-set. It is in the range of 0 to 9 based on color component and combination of tools used. It may be different for each Y, Cb and Cr components.

E.g.: If Only deblocking and ALF is enabled. The k value for Y will be 8 and for Cb/Cr will be 6.

| Filtering Process | Luma | Chroma |
| --- | --- | --- |
| No Inloop de-blocking | 0 | 0 |
| Only De-blocking filter | 4 | 2 |
| Only SAO filter | 1 | 1 |
| Only ALF | 4 | 4 |

# Conclusion

This proposal enables LCU based decoding operation for checksum along with parallel computation for different color components which results in memory bandwidth saving and on-the-fly checksum computation.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# CD Text Changes

### D.1.20 Decoded picture hash SEI message syntax

| Decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| if( hash_type = = 0 ) |  |
| for(comp=0; comp<3; comp++) |  |
| for( i = 0; i < 16; i++) |  |
| Picture_md5[comp][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| for(comp=0; comp<3; comp++) |  |
| picture_crc[comp] | u(16) |
| } |  |

D.2.20  Decoded picture hash SEI message semantics

This message provides three checksums of the decoded picture in the current access unit.

Prior to computing the checksum, the decoded picture data is arranged into a string of bytes using following pseudo code process for each component in Y, Cb, and Cr..

// Loop over each LCU

for (y=0; y<height; y += sizeY)

{

if(y == 0)

sizeY = lcuSz-k;

else if(((height+lcuSz-1)/lcuSz)*lcuSz - (y + lcuSz + k) >= lcuSz)

sizeY = lcuSz;

else

sizeY = height-y; //To handle non-multiple of lcuSz

for (x=0; x<width; x += sizeX)

{

if(x == 0)

sizeX = lcuSz-k;

else if(((width+lcuSz-1)/lcuSz)*lcuSz - (x + lcuSz + k) >= lcuSz)

sizeX = lcuSz;

else

sizeX = width-x;//To handle non-multiple of lcuSz

//Linearize the block in raster-scan order

Read_data_lcu_block_1D(linear_lcu_block_data);

// Call hash function e.g. MD5

md5_block<OUTPUT_BITDEPTH_DIV8>(md5, linear_lcu_block_data, data_size);

}

}

The same process is repeated for Y, Cb and Cr separately and three check-sums are created.

hash_type indicates the method used to calculate the checksum according to Table D-1.

Table D-1 – Interpretation of hash_type

| - hash_type | - Method |
| --- | --- |
| - 0 | - MD5 (RFC 1321) |
| - 1 | - CRC |
| - 2..255 | - Reserved |

picture_md5[comp][ i ] is the MD5 hash of the decoded picture. comp can take values 0 to 2, Y, Cb and Cr respectively. It is a requirement of bitstream conformance that the value of picture_md5[comp][ i ], when present, shall be equal to the value of digestVal obtained by performing the following pseudocode process using the MD5 functions defined in RFC 1321 for each of the color components Y, Cb and Cr.

MD5Init( context )
MD5Update( context, pictureData, pictureDataLen )
MD5Final( digestVal, context )

picture_crc[comp] is the cyclic redundancy check (CRC) of the decoded picture. Comp can take values 0 to 2, Y, Cb and Cr respectively. It is a requirement of bitstream conformance that the value of picture_crc[comp ], when present, shall be equal to the value of crcVal obtained by performing the following pseudo code process for each of the color components Y, Cb and Cr.

crcVal = 0xFFFF
pictureData[ pictureDataLen ] = 0
pictureData[ pictureDataLen + 1 ] = 0
for( bitIdx = 0; bitIdx < ( pictureDataLen + 2 ) * 8; bitIdx++) {
	crcMsb = ( crcVal >> 15 ) & 1
	bitVal = ( data[ bitIdx >> 3 ] >> (7 – ( bitIdx & 7 ) ) ) & 1
	crcVal = ( ( ( crcVal << 1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
}

NOTE – The same CRC specification is found in Rec. ITU-T H.271.