| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0246 |
| --- | --- |

| Title: | SAO Type Coding Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Maani 1730 N 1st ST San Jose, CA 95134 | Tel: Email: | (408) 352-4710 ehsan.maani@am.sony.com |
|  | K. Minoo, D. Baylon 6450 Sequence Drive San Diego, CA 92121 E. Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742 | Email: Email: | kminoo@motorola.com david.baylon@motorola.com elena_a.alshina@samsung.com |
| Source: | Sony Corp., Motorola Mobility, Samsung |  |  |

_____________________________

# Abstract

This document proposes a modification to SAO type coding. Only two types are considered: Edge Offset (EO), one Band Offset (BO). Therefore, a 1-bit flag is used to signal the SAO type. The SAO on/off switch is also coded using a 1-bit flag with a separate context. The rest of the side information is coded using Fixed Length (FL) codes and bypass mode in CABAC.

# Proposed Method

Current SAO types (in WD 6) consist of SAO on/off switch, 4 Edge Offset (EO) types, and one Band Offset (BO). These options (SAO types) are signalled to the decoder using a Unary code with two context (for CABAC coding), one on the first bin and one for the rest of the bins. In addition, for the BO type, extra information (the starting band) is transmitted using FL codes (while for the EO no extra information is needed). This design can be improved  from three different aspects:

1) While there are only two actual types in SAO (namely EO and BO) the SAO types in current design consists of 4 EO types but only 1 BO type. Therefore, the extra context in CABAC mode cannot track the statistics of EO versus BO.

2) The four EO types correspond to 4 directions but have very different code lengths. Logically, due to the symmetry in a large-enough set of images, there should not be an advantage to a direction compared to the rest. This has been confirmed even on the HEVC test set which is a much smaller test set. The number of occurrences of the four directions are almost the same if FL codes are used.

3) Side information is transmitted only in one of the types (i.e., BO). Side information for the EO type (directions), however, are coded with SAO type and on/off switch.

Based on the above argument, we propose the following coding structure for the SAO information signalling (if the partition is not merged with any of the neighbours). This concept is also visualized in Figure 1.

- SAO on/off: a 1-bit flag with a separate context to track number of LCU's for which SAO is used.

- SAO Type: a 1-bit flag to indicate EO or BO when SAO is on. A context is used to track the statistics of each type. Since BO statistics change drastically from sequence to sequence this context is very effective.

- SAO subtype index: An index when SAO is on. This subtype index is coded using FLC and bypass mode. For BO this index is 5 bits and corresponds to the starting band while for EO this index is 2 bits and corresponds to one of the four directions.

- Offsets: Offset coding is not modified compared to WD 6. Four offsets are coded regardless of the SAO type.

Figure 1. Proposed flowchart for coding SAO information of a partition (LCU).

# Results

Results are obtained according to CE1 test configuration and compared against CE1 anchors.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | 0.0% | 0.0% | -0.4% | -0.3% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.3% |
| Class D | 0.0% | -0.5% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class F | -0.1% | -0.2% | -0.3% | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main CU64x64 |  |  | Low delay B HE10 CU64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.8% | -0.9% | 0.0% | -0.6% | -0.7% |
| Class C | 0.0% | -0.7% | -0.7% | 0.0% | -0.3% | -0.7% |
| Class D | 0.0% | -0.6% | -0.4% | 0.0% | -0.8% | -0.3% |
| Class E | 0.1% | -0.8% | -1.5% | 0.1% | -0.4% | -1.0% |
| Overall | 0.0% | -0.7% | -0.7% | 0.0% | -0.6% | -0.6% |
|  | 0.0% | -0.7% | -0.7% | 0.0% | -0.6% | -0.6% |
| Class F | -0.2% | -0.9% | -1.0% | 0.0% | -0.3% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main CU32x32 |  |  | Low delay B HE10 CU32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.7% | 0.0% | -0.2% | -0.2% |
| Class C | -0.1% | -0.5% | -0.9% | -0.1% | -0.3% | -0.4% |
| Class D | 0.0% | -0.7% | -0.3% | 0.0% | -0.1% | -0.5% |
| Class E | 0.0% | -0.2% | -0.5% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | -0.4% | -0.6% | 0.0% | -0.1% | -0.3% |
|  | 0.0% | -0.4% | -0.6% | 0.0% | -0.1% | -0.2% |
| Class F | -0.2% | 0.0% | -1.4% | -0.1% | -0.4% | -0.7% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main CU16x16 |  |  | Low delay B HE10 CU16x16 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | 0.2% |
| Class D | 0.0% | 0.2% | -0.3% | 0.0% | 0.3% | 0.3% |
| Class E | -0.1% | -0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.2% | -0.6% | -0.9% | -0.1% | 1.0% | 0.6% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Patent rights declaration(s)

Sony Corp., Motorola Mobility, or Samsung,  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).