| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0248 |
| --- | --- |

| Title: | CE2.b.2: Removing explicit K table signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Tomoo Yamakage, Takayuki Itoh, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp |
| Source: | Qualcomm Inc., MediaTek Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this test, we studied the effectiveness of removal of explicit K table signaling. This test use picture/LCU-based encoding. Results show coding loss of 0.0/0.1/0.0/0.1% (AI/RA/LB/LP) for picture based encoding, and coding loss of 0.0/0.0/0.0/0.0% (AI/RA/LB/LP) for LCU based encoding.

# Introduction

Currently ALF supports explicit signaling of K value of Golomb code for each coefficients. In this CE2 subset, we study the effectiveness of removal of explicit K table signaling (i.e., using implcit K table instead).

# Proposed Method

This test will not modify the signal processing part in WD6/HM6, and will remove only signalling of K table. Instead of explicit signaling of K table, following implicit K table is used for luma coefficients.

| Syntax element | Filter coefficient | Golomb coding parameter k |
| --- | --- | --- |
| alf_filt_coeff[0] | C0 | 2 |
| alf_filt_coeff[1] | C1 | 3 |
| alf_filt_coeff[2] | C2 | 3 |
| alf_filt_coeff[3] | C3 | 4 |
| alf_filt_coeff[4] | C4 | 3 |
| alf_filt_coeff[5] | C5 | 1 |
| alf_filt_coeff[6] | C6 | 2 |
| alf_filt_coeff[7] | C7 | 3 |
| alf_filt_coeff[8] | C8 | 4 |
| alf_filt_coeff[9] | C9 | 1 |

# Related WD Changes

[JCTVC-H1003_dK : 7.3.3.6	Adaptive loop filter info syntax]

| alf_info( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_new_filter_set_flag | u(1) |
| if( alf_new_filter_set_flag = = 0 && NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_stored_filter_set_idx[ cIdx ] | u(v) |
| else { |  |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 15; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |
| if( alf_no_filters_minus1 > 0 ) |  |
| alf_pred_flag[ cIdx ][ ry ][ rx ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| alf_nb_pred_luma_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |
| if( AlfNumFilters > 1 ) { |  |
| alf_min_kstart_minus1 | ue(v) |
| for( i = 1; i < 4; i++ ) |  |
| alf_golomb_index_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ i ][ j ] | ge(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ 0 ][ j ] | se(v) |
| } |  |
| } |  |

7.4.3.6	Adaptive loop filter info semantics

alf_min_kstart_minus1 plus 1 specifies the minimum order k of k-th order exponential golomb code for the luma filter coefficients for the adaptive loop filter.

alf_golomb_index_flag[ i ] specifies the difference in the orders of k-th order exponential golomb codes between i-th group and (i+1)-th group of the luma filter coefficients. There are several groups of the luma filter coefficients where each group may have a different order k. The group value of the j-th luma filter coefficient alfCoeffGroup[ j ] is derived as follows.

alfCoeffGroup[ j ] = { 2, 3, 3, 4, 3, 1, 2, 3, 4, 1 } with j = 0..9	(7-1)

The table of different orders k of the j-th coefficient alfkVal[ alfCoeffGroup[ j ] ] with j=0..9 is derived as follows.

alfkVal[ i ] = (alf_min_kstart_minus1 +1)+ Σr alf_golomb_index_flag[r] with i= 1..3 and r= 1.. i	(7-2)

The k-th order of j-th alf_filter_coeff alfkMap[j] is derived as follows.

alfkMap[j] = alfkVal[ alfCoeffGroup[ j ] ] with j = 0..9

When alf_golomb_index_flag [i ] is not present, the alfkMap[ j ] = alfCoeffGroup[ j ]

alfkMap [ j ] specifies the order of k-th order exponential golomb codes of j-th luma ALF coefficient, and is defineds as follows.

alfkMap [ j ] = { 2, 3, 3, 4, 3, 1, 2, 3, 4, 1 } with j = 0..9

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE2 common test condition is used. Note that only the interleaving mode is implemented and tested.

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.01% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 100% |  |  |
| #NUM! |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| 0.1% | 0.1% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.1% | 0.1% | -0.1% |
| 0.05% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 100% |  |  |
| #NUM! |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.0% | 0.0% | -0.1% |
| 0.0% | 0.2% | -0.2% |
| 0.1% | -0.2% | 0.0% |
| 0.0% | 0.0% | -0.5% |
| 0.01% | 0.0% | -0.2% |
| 0.0% | 0.0% | -0.2% |
| 0.0% | 0.0% | 0.0% |
| 100% |  |  |
| #NUM! |  |  |
| Low delay P HE10 |  |  |
| Y | U | V |
| 0.0% | -0.3% | 0.0% |
| 0.0% | -0.1% | -0.1% |
| 0.1% | 0.4% | 0.0% |
| 0.1% | 0.0% | -0.1% |
| 0.06% | 0.0% | 0.0% |
| 0.1% | 0.0% | 0.0% |
| 0.1% | -0.3% | 0.4% |
| 100% |  |  |
| #NUM! |  |  |

Table 1. BD-rate results of the proposed method compared to CE2 anchor (Picture based encoding).

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.00% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 99% |  |  |
| #NUM! |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | -0.1% | 0.0% |
| 0.00% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 100% |  |  |
| #NUM! |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.0% | 0.0% | 0.1% |
| 0.0% | 0.0% | 0.4% |
| 0.0% | -0.2% | 0.2% |
| 0.0% | 0.0% | 0.3% |
| 0.00% | -0.1% | 0.2% |
| 0.0% | 0.0% | 0.2% |
| 0.0% | -0.8% | -0.7% |
| 99% |  |  |
| #NUM! |  |  |
| Low delay P HE10 |  |  |
| Y | U | V |
| 0.0% | -0.1% | -0.1% |
| 0.0% | -0.1% | 0.0% |
| 0.0% | 0.1% | -0.1% |
| 0.0% | -0.2% | 0.5% |
| 0.00% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.1% |
| 0.0% | 0.3% | 0.2% |
| 99% |  |  |
| #NUM! |  |  |

Table 2. BD-rate results of the proposed method compared to CE2 anchor (LCU based encoding).

# Conclusion

In this test, we studied the effectiveness of removal of explicit K table signaling. This test use picture/LCU-based encoding. Results show coding loss of 0.0/0.1/0.0/0.1% (AI/RA/LB/LP) for picture based encoding, and coding loss of 0.0/0.0/0.0/0.0% (AI/RA/LB/LP) for LCU based encoding.

# Patent rights declaration(s)

Qualcomm Inc., MediaTek Inc., TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).