| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0249 |
| --- | --- |

| Title: | CE2.a.1: Signalling mode change from slice header mode to interleaving mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Tomoo Yamakage, Takayuki Itoh, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp |
| Source: | Qualcomm Inc., MediaTek Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this test, we modify signaling of LCU based ALF. We moved ALF parameters (alf_info( )) from slice header to slice data (LCU interleaved way). And we also remove merge_up and merge_left, limit the size of filter storage (to 16), apply “Chroma filter is off when luma filter is off for each LCU”, and use only 1 filter per LCU. Results show coding gain of 0.0/-0.1/-0.1/-0.2% (AI/RA/LB/LP) compared to CE2 anchor using LCU based encoding.

# Introduction

Currently ALF supports two signalling modes: picture based and LCU based modes. Picture based mode sends ALF coefficients in APS and CU level on/off flag in slice header. LCU based mode sends ALF coefficients and LCU level on/off flag in slice header. In this CE2 subset, we study the “real” LCU-based signalling mode based on the syntax in JCTVC-H0066.

# Proposed Method

This test will not modify the signal processing part in WD6/HM6, and will modify only signalling position of ALF coefficients and control flag to be interleaved in each LCU. The interleaved information will be coded by CABAC. In summary new signaling scheme is as follows.

SPS: Transmit ALF Mode flag

If(ALF Mode == FRAME based ALF)

Same as HM6.0

else (ALF Mode == LCU based ALF)

APS: Transmit nothing.

Slice header: Transmit nothing.

LCU: Use CABAC. Interleave following with LCU data

alf_lcu_enable_flag

alf_new_filter_set_flag

alf_stored_filter_set_idx or one ALF filter set (up to 16 luma filters + 2 chroma filters)

This test will use LCU-based encoding, however, coding results will also be compared with picture-based encoding in WD6/HM6.

We tested two options for LCU-based encoding: allowing multiple filters per LCU (up to 16 as in HM6.0) vs. forcing 1 filter per LCU. Even though current HM6.0’s RDO mostly chooses to use 1 filter per LCU, in order to see whether there is any difference in performance between current RDO based filter selection vs. forcing 1 filter per LCU. There was no difference in performance. Therefore, we forced one filter per LCU in normative way. Current ALF uses codebook approach to reuse previously sent filters which reduce the usefulness of merge_up and merge_left. Also when there is not enough bit budget to enable luma filter of current LCU, it is highly possible that Chroma can not use filter. Therefore we did the following things to improve coding efficiencies: i) Remove merge_up and merge_left, ii) Chroma filter is off when luma filter is off for each LCU. In summary, we did the following things.

- Moving ALF parameters (alf_info( )) from slice header to slice data (LCU interleaved way)

  - Remove merge_up and merge_left

  - Limit the size of filter storage (to 16)

  - Chroma filter is off when luma filter is off for each LCU

- Use only 1 filter per LCU

# Related WD Changes

[7.3.4	Slice data syntax]

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| if(alf_coef_in_slice_flag) |  |
| alf_unit_cabac( rx, ry, cIdx ) |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

[JCTVC-H1003_dK : 7.3.3.x	Adaptive loop filter unit cabac syntax for LCU interleaved mode]

| alf_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if(cIdx ==0 ¦¦ (cIdx !=0 && alf_lcu_enabled_flag[0][ry][rx] ==1)) |  |
| { |  |
| alf_lcu_enable_flag[ cIdx ][ ry ][ rx ] | u(1) |
| if (alf_lcu_enable_flag[ cIdx ][ ry ][ rx ]) |  |
| alf_info_cabac( rx, ry, cIdx ) |  |
| } |  |
| } |  |

[JCTVC-H1003_dK : 7.3.3.6	Adaptive loop filter info cabac syntax for LCU interleaved mode]

| alf_info_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_new_filter_set_flag | u(1) |
| if( alf_new_filter_set_flag = = 0 && NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| { |  |
| if(NumALFFiltersInStoredBuffer[cIdx] > 1) |  |
| { |  |
| alf_use_prev_stored_filter_flag | u(1) |
| if(alf_use_prev_stored_filter_flag ==0 && NumALFFiltersInStoredBuffer[cIdx] > 2) |  |
| alf_remaining_stored_filter_idx[ cIdx ] | u(v) |
| } |  |
| } |  |
| else { |  |
| if( cIdx = = 0 ) { |  |
| alf_nb_pred_luma_flag[ cIdx ][ ry ][ rx ] | u(1) |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ j ] | ge(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ 0 ][ j ] | se(v) |
| } |  |
| } |  |

7.4.3.6	Adaptive loop filter info semantics

alf_use_prev_stored_filter_flag specifies whether the most recently stored filter of stored filter set is used.

alf_new_filter_set_flag equal to 1 specifies that the current coding treeblock uses a new filter set; equal to 0 specifies that the current coding treeblock uses the stored filter set with the buffer index equal to 0 if alf_use_prev_stored_filter_flag =1, otherwise (alf_use_prev_stored_filter_flag =0) buffer index is set equal to alf_remaining_stored_filter_idx[cIdx ]+1.

When alf_new_filter_set_flag is equal to 1, NumALFFiltersInStoredBuffer[cIdx] is calculated as  NumALFFiltersInStoredBuffer[cIdx]  = max(16, NumALFFiltersInStoredBuffer[cIdx]+1).

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE2 common test condition is used. Note that only the interleaving mode is implemented and tested.

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.0% | 0.1% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| -0.1% | 0.1% | 0.1% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.3% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| -0.1% | 0.3% | 0.3% |
| 98% |  |  |
| 100% |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| -0.1% | 0.1% | 0.2% |
| -0.1% | 0.3% | 0.1% |
| -0.1% | 0.2% | 0.1% |
| 0.0% | 0.0% | 0.0% |
| -0.1% | 0.2% | 0.1% |
| -0.1% | 0.2% | 0.1% |
| -0.2% | 0.4% | 0.3% |
| 98% |  |  |
| 100% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| -0.2% | 0.5% | 0.4% |
| -0.2% | 0.3% | 0.9% |
| -0.1% | 0.6% | 0.3% |
| -0.1% | 0.3% | 0.7% |
| -0.1% | 0.4% | 0.6% |
| -0.1% | 0.5% | 0.6% |
| -0.2% | -0.4% | 0.5% |
| 97% |  |  |
| 100% |  |  |
| Low delay P HE10 |  |  |
| Y | U | V |
| -0.2% | 1.0% | 0.6% |
| -0.2% | 0.5% | 0.6% |
| -0.1% | 0.2% | 0.0% |
| -0.3% | 0.9% | 1.4% |
| -0.2% | 0.7% | 0.6% |
| -0.2% | 0.7% | 0.7% |
| -0.3% | 0.8% | 1.9% |
| 97% |  |  |
| 100% |  |  |

Table 2. BD-rate results of the proposed method compared to CE2 anchor.

# Conclusion

In this test, we modify signaling of LCU based ALF. We moved ALF parameters (alf_info( )) from slice header to slice data (LCU interleaved way). And we also remove merge_up and merge_left, limit the size of filter storage (to 16), apply “Chroma filter is off when luma filter is off for each LCU”, and use only 1 filter per LCU. Results show coding gain of 0.0/-0.1/-0.1/-0.2% (AI/RA/LB/LP) compared to CE2 anchor using LCU based encoding.

# Patent rights declaration(s)

Qualcomm Inc., MediaTek Inc., TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).