| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0250 |
| --- | --- |

| Title: | CE2.c.1: Reducing number of filters of BA (Encoder only) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this test, we studied the effect of reducing the number of filters of BA mode using encodoer changes without changing BA computation. This test use picture-based encoding. Results show coding loss of 0.0/0.1/0.1/0.3% (AI/RA/LB/LP) for 10 filters, 0.0/0.1/0.2/0.3% (AI/RA/LB/LP) for 7 filters, 0.1/0.3/0.2/0.5% (AI/RA/LB/LP) for 4 filters, and 0.5/0.9/0.7/1.4% (AI/RA/LB/LP) for 1 filter for picture based encoding.

# Introduction

Currently picture based ALF supports up to 16 filters. In this CE2 subset, we the effect of reducing the number of filters of BA mode using encodoer changes without changing BA computation.

# Proposed Method

This test will not modify the signal processing part in WD6/HM6, and will change number of filters in BA.

For BA with up to 10 filter per frame, the encoder can choose among:

  Whole frame as 1 region, with up to only 10 BA classes (merged from 16 BA classes).

  A frame is divided into 4 regions, each has up to 2 BA classes (essentially no BA).

For BA with up to 7 filter per frame, the encoder can choose among:

  Whole frame as 1 region, with up to only 7 BA classes (merged from 16 BA classes).

  A frame is divided into 4 regions, each has up to 1 BA classes (essentially no BA).

For BA with up to 4 filter per frame, the encoder can choose among:

  Whole frame as 1 region, with up to only 4 BA classes (merged from 16 BA classes).

  A frame is divided into 4 regions, each has up to 1 BA classes (essentially no BA).

For BA with up to 1 filter per frame, the encoder can choose among:

  Whole frame as 1 region, with up to only 1 BA classes (merged from 16 BA classes).

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE2 common test condition is used. Note that only the interleaving mode is implemented and tested.

|  | IO_HE10 | RA_HE10 | LDB_HE10 | LDP_HE10 |
| --- | --- | --- | --- | --- |
| BA with 10 filters (pic) | 0.00% | 0.06% | 0.09% | 0.27% |
| BA with 7 filters (pic) | 0.03% | 0.12% | 0.15% | 0.34% |
| BA with 4 filters (pic) | 0.14% | 0.25% | 0.20% | 0.46% |
| BA with 1 filters (pic) | 0.47% | 0.91% | 0.74% | 1.40% |

Table 1. BD-rate results of the proposed method compared to CE2 anchor (pciture based encoding).

# Conclusion

In this test, we studied the effect of reducing the number of filters of BA mode using encodoer changes without changing BA computation. This test use picture-based encoding. Results show coding loss of 0.0/0.1/0.1/0.3% (AI/RA/LB/LP) for 10 filters, 0.0/0.1/0.2/0.3% (AI/RA/LB/LP) for 7 filters, 0.1/0.3/0.2/0.5% (AI/RA/LB/LP) for 4 filters, and 0.5/0.9/0.7/1.4% (AI/RA/LB/LP) for 1 filter for picture based encoding.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).