| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0251 |
| --- | --- |

| Title: | On NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 Jung Won Kang, Hahyun Lee, Jin Soo Choi, 218, Gajeong-no, Yuseong, Daejeon, Korea | Tel: Email: | thang@u-aizu.ac.jp +82-42-860-5137 {jungwon, hanilee, jschoi} @ etri.re.kr |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

In this contribution, some revisions to the existing syntax elements of NAL unit header are proposed to support more efficient extraction of bitstreams. First nal_ref_flag is suggested to be removed, saving one bit for other purpose. Second, the 5 bits of reserved_one_5bits are used as a priority value for extracting bitstreams conforming to the base specification.

# Introduction

In the last meeting in San Jose, 5 reserved bits are added to the NAL unit header as a hook for future extensions of HEVC. The purpose was to have a fixed size (2bytes) NAL unit header while still supporting the future extensions. In this contribution, we propose some revisions to the existing syntax elements of NAL unit header to support more efficient extraction of bitstreams. First nal_ref_flag is suggested to be removed, saving one bit for other purpose. Second, the 5 bits of reserved_one_5bits are used as a priority value for extracting bitstreams conforming to the base specification.

# On nal_ref_flag

Currently, nal_ref_flag is used to indicate the non-reference pictures and reference pictures. This information gives some rough prioriry between these two types of pictures. However, the use of this flag for extraction is rather limited. Specifically, if all non-reference pictures (mostly those in the highest temporal layer) are discarded, all remaining pictures have nal_ref_flag equal to 1. Yet, it is obvious that some pictures (again mostly at the highest temporal layer) of the adapted/extracted bitstream in fact become non-reference pictures though their nal_ref_flag’s are equal to 1. On the other hand, some other syntax elements (e.g. temporal_id) in the NAL unit header are already effective in support adaptive extraction.

The one bit of this flag can be used for either nal_unit_type, temporal_id, or the future layer_id. Also, it can be used as a reserved bit for the future.

For the above reason, we propose to remove nal_ref_flag from NAL unit header. Some related options are described in the following.

- Option 1: nal_ref_flag is renamed into slice_ref_flag and relocated into slice header. The slice header syntax is modified as follows.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_ref_flag | u(1) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| ... | ue(v) |
| } |  |

  - slice_ref_flag equal to 1 indicates that the given slice is a part of a reference picture. slice_ref_flag equal to 0 indicates that the given slice is a part of a non-reference picture.

- Option 2: nal_ref_flag is renamed into au_ref_flag and relocated to access unit delimiter. The access unit delimiter syntax is modified as follows.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| au_ref_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

  - au_ref_flag equal to 1 indicates that the given access unit contains a reference picture. au_ref_flag equal to 0 indicates that the given access unit contains a non-reference picture.

- Option 3: simply removing the nal_ref_flag from NAL unit header without relocation.

# On reserved_one_5bits

These 5 bits are reserved for future extension. However, we believe that these bits can be used for the extraction of bitstreams conforming to the base spec of HEVC. Specifically, these 5 bits (or some of these) are used for a new syntax element called “priority_id” by default as shown in the following syntax.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| priority_id | u(5) |
| reserved_one_5bits | u(5) |
| ... |  |
| } |  |

If this feature should be “switchable”, the signaling can be done in different ways. The first possible way is to check whether profile_idc is equal to any value reserved for the base spec of HEVC as in the syntax below. If it is the case, the 5 bits are used for priority_id. Another way is to use a flag in either NAL unit header (e.g. the bit of nal_ref_flag) or in a parameter set for signaling.

As for the semantics of priority_id element, we propose two options

- Option 1: priority_id describes the priority values of NAL units of a temporal layer. Within a temporal layer, NAL units having higher priority_id values will be discarded before NAL units having lower priority_id values.

- Option 2: priority_id describes the priority values of NAL units regardless of temporal_id values. Within a bitstream, NAL units having higher priority_id values will be discarded before NAL units having lower priority_id values.

# Patent rights declaration(s)

ETRI and University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).