| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0252 |
| --- | --- |

| Title: | High-level syntax modifications to support extractor operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 Jung Won Kang, Hahyun Lee, Jin Soo Choi, 218, Gajeong-no, Yuseong, Daejeon, Korea | Tel: Email: | thang@u-aizu.ac.jp +82-42-860-5137 {jungwon, hanilee, jschoi} @ etri.re.kr |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

Currently, to discard NAL units of an HEVC bitstream, an extractor needs to parse NAL unit header, some parameter sets, and even slice headers. This means that, an extractor must understand all the syntax elements in parameter sets and slice headers. Whereas, only a small part of those syntax elements actually contains the necessary information for extraction. To better support the operation of extractors, two new tools are proposed in this contribution. The first is a basic parameter set, a place holder containing the essential information for an extractor. The second is activation signaling of basic parameter set.

# Introduction

Currently, to discard NAL units of an HEVC bitstream, an extractor needs to parse NAL unit header and various parameter sets. Especially, to know which SPS is active, an extractor must parse slice headers and related PPS as well. This means that, an extractor must understand all the syntax elements in parameter sets and slice headers. Whereas, only a small part of those syntax elements actually contains the necessary information for extraction. This is obviously a big burden for any extractor. To solve this problem, we propose the following tools:

- Basic parameter set: a place holder containing the essential information for an extractor. This parameter set may also contain some syntax elements of extension specs of HEVC.

- Activation signaling of basic parameter set: some simple methods to let an extractor know which basic parameter set is activated without parsing slice headers and related PPS.

# Basic Parameter Set

In our solution, the basic parameter set contains the essential information for an extractor to understand a bitstream. The syntax of the basic parameter set is provided as below.

| basic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| basic_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| priority_id_flag | u(1) |
| extension_info_flag | u(1) |
| if( extension_info_flag = = 0) { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offsetv | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| chroma_format_idc | u(2) |
| temporal_id_nesting_flag | u(1) |
| for ( i = 0; i <= num_temporal_layers_minus1; i++ ) { |  |
| profile_idc[ i ] | u(8) |
| reserved_zero_8bits[ i ] /* equal to 0 */ | u(8) |
| level_idc[ i ] | u(8) |
| } |  |
| } |  |
| if( extension_info_flag = = 1) { |  |
| extension_info() |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

All the syntax elements have the same semantics as those in SPS of current HEVC WD, except the following new elements:

- basic_parameter_set_id: indicates the identifier of the basic parameter set. This identifier may be referenced from SPS, an SEI message, or an access unit delimiter.

- priority_id_flag: if equal to 1, indicates that the bits of the syntax element reserved_one_5bits (or layer_id) can be used to represent priority value of the NAL unit.

- extension_info_flag: if the bitstream conforms to the base spec of HEVC, this flag shall be equal to 0. If the bitstream conforms to an extension spec of HEVC, this flag shall be equal to 1 and extension information will be provided.

# Modifications to SPS

With the introduction of basic parameter set, the following syntax elements (in red color) are removed from SPS. Also, the following syntax element is added to SPS.

- bps_id identifies the basic parameter set referred by the sequence parameter set. The value of bps_id shall be in the range of 0 to X, inclusive.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| bps_id | ue(v) |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| max_temporal_layers_minus1 | u(3) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| chroma_format_idc | u(2) |
| temporal_id_nesting_flag | u(1) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Activation Signaling for Basic Parameter Set (BPS)

The basic parameter set could be sometimes updated. To let an extractor know which BPS is activated without having to parse all slice headers, one of two following methods can be used in the bitstream.

In the first method, a new element bps_id is added to the access unit delimiter, indicating the identifier of the basic parameter set to be used for the NAL units inside the related access unit. The syntax of the modified access unit delimiter is shown below.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| bps_id | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

In the second method, a new SEI message called parameter_set_reference() is used to convey the element bps_id, indicating the identifier of the basic parameter set to be used for the NAL units inside the related access unit. The syntax of the SEI message is shown below.

| parameter_set_reference( payloadSize ) { | Descriptor |
| --- | --- |
| bps_id | ue(v) |
| } |  |

To extract a bitstream, an extractor just has to maintain one or more BPS and detects the value of bps_id element if the access unit delimiter or parameter_set_reference SEI message is present in an access unit.

# Patent rights declaration(s)

ETRI and University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).