| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0253 |
| --- | --- |

| Title: | High-level syntax for future scalable extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 Jung Won Kang, Hahyun Lee, Jin Soo Choi, 218, Gajeong-no, Yuseong, Daejeon, Korea | Tel: Email: | thang@u-aizu.ac.jp +82-42-860-5137 {jungwon, hanilee, jschoi} @ etri.re.kr |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

To better support the operation of extractors, the basic parameter set, a place holder containing the essential information for an extractor, is proposed in JCTVC-I0252. For future scalable extensions of HEVC, a rich syntax is described in this contribution. This syntax can be considered as the extension part of the basic parameter set.

# Introduction

As discussed in I0252, an extractor may need to understand all the syntax elements in parameter sets and slice headers to carry out extraction operation. Whereas, only a small part of those syntax elements actually contains the necessary information for extraction. To solve this problem, I0252 proposes a “basic parameter set”, which is a place holder containing the essential information for an extractor. In this contribution, we propose some syntax elements which can support an extractor to extract bitstreams conforming to future scalable extensions of HEVC.

# Description of Bitstream in Scalable Extension

Syntax for HEVC scalable extensions can be an extension part of the basic parameter set as below.

| basic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| basic_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| priority_id_flag | u(1) |
| extension_info_flag | u(1) |
| if( extension_info_flag = = 0) { |  |
| ........... |  |
| } |  |
| if( extension_info_flag = = 1) { |  |
| extension_info() |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The syntax within extension_info() to convey a detailed description of a bitstream is provided below.

| extension_info( ){ |  |
| --- | --- |
| num_frame_sizes_minus1 | ue(v) |
| num_rep_formats_minus1 | ue(v) |
| for( i = 0; i <= num_frame_sizes_minus1; i++ ) { |  |
| pic_width_in_luma_samples[ i ] | ue(v) |
| pic_height_in_luma_samples[ i ] | ue(v) |
| pic_cropping_flag[ i ] | u(1) |
| if( pic_cropping_flag[ i ] ) { |  |
| pic_crop_left_offset[ i ] | ue(v) |
| pic_crop_right_offsetv[ i ] | ue(v) |
| pic_crop_top_offset[ i ] | ue(v) |
| pic_crop_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= num_rep_formats_minus1; i++ ) { |  |
| bit_depth_luma_minus8[ i ] | ue(v) |
| bit_depth_chroma_minus8[ i ] | ue(v) |
| chroma_format_idc[ i ] | u(2) |
| } |  |
| num_layers_minus1 | ue(v) |
| dependency_id_flag | u(1) |
| quality_id_flag | u(1) |
| view_id _flag | u(1) |
| region_id_flag | u(1) |
| layer_dependency_info_flag | u(1) |
| for( i = 1; i <= num_layers_minus1; i++ ) { |  |
| frame_size_idx[ i ] | ue(v) |
| rep_format_idx[ i ] | ue(v) |
| if( dependency_id_flag) { |  |
| one_dependency_id_flag[i] | u(1) |
| if ( one_dependency_id_flag[i] = = 1) |  |
| dependency_id[i] | ue(v) |
| else if( one_dependency_id_flag[i] = = 0) { |  |
| dependency_id_min[i] | ue(v) |
| dependency_id_max[i] | ue(v) |
| } |  |
| } |  |
| if( quality_id_flag ) { |  |
| one_quality_id_flag[i] | u(1) |
| if ( one_quality_id_flag[i] = = 1) |  |
| quality_id[i] | ue(v) |
| else if( one_quality_id_flag[i] = = 0) { |  |
| quality_id_min[i] | ue(v) |
| quality_id_max[i] | ue(v) |
| } |  |
| } |  |
| if( view_id_flag ) { |  |
| one_view_id_flag[i] | u(1) |
| if ( one_view_id_flag[i] = = 1) { |  |
| view_id[i] | ue(v) |
| depth _flag | u(1) |
| } |  |
| else if( one_view_id_flag[i] = = 0) { |  |
| view_id_min[i] | ue(v) |
| view_id_max[i] | ue(v) |
| } |  |
| } |  |
| if( region_id_flag ) { |  |
| num_regions_minus1[i] | u(8) |
| for( j = 1; j <= num_regions_minus1[i]; j++ ) |  |
| region_id[i][j] | u(8) |
| } |  |
| if( layer_dependency_info_flag ) { |  |
| num_directly_dependent_layers[ i ] | u(8) |
| for( j = 0; j < num_directly_dependent_layers[ i ]; j++ ) |  |
| directly_dependent_layer_id_delta_minus1[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |

The semantics of the above syntax elements is defined as follows.

- num_frame_sizes_minus1 plus 1 specifies the maximum number of different sets of frame size information (i.e., the sets of pic_width_in_luma_samples[i], pic_height_in_luma_samples[i], pic_cropping_flag[i], pic_cropping_flag[i], pic_crop_left_offset[i], pic_crop_right_offsetv[i], pic_crop_top_offset[i], pic_crop_bottom_offset[i]) in the coded video sequence. The value of num_rep_formats_minus1 shall be in the range of 0 to X, inclusive.

- num_rep_formats_minus1 plus 1 specifies the maximum number of different sets of bit depth and chroma format (i.e., the sets of bit_depth_luma_minus8[i], bit_depth_chroma_minus8[i], and chroma_format_idc values[i]) in the coded video sequence. The value of num_rep_formats_minus1 shall be in the range of 0 to X, inclusive.

- pic_width_in_luma_samples[i], pic_height_in_luma_samples[i], pic_cropping_flag[i], pic_cropping_flag[i], pic_crop_left_offset[i], pic_crop_right_offsetv[i], pic_crop_top_offset[i], pic_crop_bottom_offset[i] specify the i-th set of pic_width_in_luma_samples, pic_height_in_luma_samples, pic_cropping_flag, pic_cropping_flag, pic_crop_left_offset, pic_crop_right_offsetv, pic_crop_top_offset, pic_crop_bottom_offset values in the coded video sequence.

- bit_depth_luma_minus8[ i ], bit_depth_chroma_minus8[ i ], and chroma_format_idc[ i ] specify the i-th set of bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc values in the coded video sequence.

- num_layers_minus1: indicates the number of scalable layers available in the bitstream

- dependency_id_flag: if equal to 1, indicates that one or more dependency_id values are associated with a layer_id value.

- quality_id_flag: if equal to 1, indicates that one or more quality_id values are associated with a layer_id value.

- view_id_flag: if equal to 1, indicates that one or more view_id values are associated with a layer_id value.

- region_id_flag: if equal to 1, indicates that one or more region_id values are associated with a layer_id value.

- layer_dependency_info_flag: if equal to 1, indicates that the dependency information of scalable layers is provided.

- frame_size_idx[ i ] specifies the index to the set of frame size information that applies to the layer with layer_id equal to i. The value of rep_format_idx[ i ] shall be in the range of 0 to X, inclusive.

- rep_format_idx[ i ] specifies the index to the set of bit depth and chorma format that applies to the layer with layer_id equal to i. The value of rep_format_idx[ i ] shall be in the range of 0 to X, inclusive.

- one_dependency_id_flag[i] if equal to 1, indicates that only one dependency_id value is associated with a layer_id value equal to i. If equal to 0, this flag indicates that more than one dependency_id values are associated with a layer_id value equal to i.

- dependency_id[i] indicates the dependency_id value which is associated with the layer_id equal to i.

- dependency_id_min[i] and dependency_id_max[i]: indicate the range of dependency_id values, which is from dependency_id_min[i] to dependency_id_max[i], associated with the layer_id equal to i.

- one_quality_id_flag[i] if equal to 1, indicates that only one quality_id value is associated with a layer_id value equal to i. If equal to 0, this flag indicates that more than one quality_id values are associated with a layer_id value equal to i.

- quality_id[i] indicates the quality_id value which is associated with the layer_id equal to i.

- quality_id_min[i] and quality_id_max[i]: indicate the range of quality_id values, which is from quality_id_min[i] to quality_id_max[i], associated with the layer_id equal to i.

- one_view_id_flag[i] if equal to 1, indicates that only one view_id value is associated with a layer_id value equal to i. If equal to 0, this flag indicates that more than one view_id values are associated with a layer_id value equal to i.

- view_id[i] indicates the view_id value which is associated with the layer_id equal to i.

- depth_flag: if equal to 1, indicates that current scalable layer (with layer_id equal to i) contains the depth information of a 3D video bitstream.

- view_id_min[i] and view_id_max[i]: indicate the range of view_id values, which is from view_id_min[i] to view_id_max[i], associated with the layer_id equal to i.

- num_regions_minus1: plus 1 indicates the number of regions associated with layer_id equal to i.

- region_id[j]: indicates the identifier of the region j associated with layer_id equal to i.

- num_directly_dependent_layers[ i ]: specifies the number of scalable layers that the current scalable layer i is directly dependent on.

- directly_dependent_layer_id_delta_minus1[ i ][ j ] plus 1 specifies the difference between layer_id[ i ] and the layer identifier of the j-th scalable layer that the current scalable layer is directly dependent on. The layer identifier of the j-th directly dependent scalable layer is equal to (layer_id[i] – directly_dependent_layer_id_delta_minus1[ i ][ j ] – 1 ).

# Patent rights declaration(s)

ETRI and University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).