| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0254 |
| --- | --- |

| Title: | Clarifying the Range of the coeff_abs_level_remaining Syntax Element |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Yong Yu Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-1345 yongyu@broadcom.com +1-949-926-6387 wwan@broadcom.com |  |
| Source: | Broadcom Corporation |  |  |  |

_____________________________

# Abstract

This contribution seeks to clarify the permissible values for coeff_abs_level_remaining.  It states that the clipping operations in the inverse transform process seem to imply that any value for coeff_abs_level_remaining is permissible.  Such an interpretation, it is claimed, would present a decoder burden when dealing with pathological streams.  The proposed clarification is a normative limitation on the coefficient range on the encoder.  It is further claimed that no burden is added to the encoder, since the coefficient range is already restricted, and that several clip operations can be removed from the draft text.

# Problem Statement & Solution

The last paragraph of Section 7.4.10, “Residual coding semantics” of [1] states that the absolute values of the transform coefficients levels is limited in Section 8.6.3:

coeff_abs_level_remaining[ n ] is the remaining absolute value of a transform coefficient level that is coded with Golomb-Rice code at the scanning position n. The value of coeff_abs_level_remaining is constrained by the limits in subclause Error: Reference source not found.When coeff_abs_level_remaining [ n ] is not present, it is inferred as 0.

However, no such limit is explicitly stated in that section.  Instead, that section, “Scaling process for transform coefficients before the inverse transform”, presents a set of clipping operations on the unscaled transform coefficient:

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

yij = Clip3( −32768, 32767, cij )		(8-1)

dij = Clip3( −32768, 32767,( ( yij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-2)

–	Otherwise, if shiftScale is greater than 0,

yij = Clip3( −32768, 32767, cij )		(8-3)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] +( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-4)

–	Otherwise,

yij = Clip3( -LevelLimit, LevelLimit − 1, cij )		(8-5)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( -shiftScale ) )	(8-6)

This appears to allow an unbounded value for the transform coefficient cij, which would allow a compliant encoder to send any size coefficient, producing a particularly thorny worst-case.  This issue was also raised in the US National Body ballot comments, comment #49 [4].

An explicit encoder limit of -32768 to 32767 for the unscaled transform coefficient would solve the problem, and allow clips 266 and 268 to be removed, as enumerated in section 5.

This change leads to efficient designs of decoders that are able to process all possible legitimate streams, and carries no burden for the encoder, which must always clip the transmitted coefficients in any case.

# Conclusions

The proposed change simplifies coefficient parsing and storage in decoder designs.  There is no extra cost on the encoder.  This contribution recommends adoption of this change.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003_dK, Feb. 2012.

- X. Li et al, “Non-CE4: Clipping of Transformed Coefficients before De-Quantization”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0312, Feb. 2012.

- L. Kerofsky et al, “Non CE4: Limiting dynamic range when using a quantization weighting matrix”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0541, Feb. 2012.

- US Ballot Comments on ISO/IEC CD 23008-2, Mar 2012

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes in Section 8.6.3

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,  It is a requirement of bitstream conformance that the values all elements of array c fall within the range -32768 to 32767.

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

…

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

yij = Clip3( −32768, 32767, cij )		(8-7)

dij = Clip3( −32768, 32767,( ( ycij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-8)

–	Otherwise, if shiftScale is greater than 0,

yij = Clip3( −32768, 32767, cij )		(8-9)

dij = Clip3( −32768, 32767, ( ycij * M[ i ][ j ]*levelScale[ qP%6 ] + 					( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-10)

–	Otherwise,

yij = Clip3( -LevelLimit, LevelLimit − 1, cij )		(8-11)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( -shiftScale ) )	(8-12)