| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0257_r1 |
| --- | --- |

| Title: | On Intensity Dependent Quantisation in the HEVC codec (WD changes) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak David Flynn Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk davidf@rd.bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document illustrates the changes required by the intensity dependent quantisation in the HEVC working draft specification.

# Changes in picture parameter set RBSP syntax

The following changes are required to the syntax which defines the Picture Parameter Set (PPS) RBSP. The changes are highlighted in red.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if(slice_type = =P ¦ ¦ slice_type = = B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| idq_enabled_flag | u(1) |
| if( idq_enabled_flag ){ |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ){ |  |
| idq_num_intervals_minus1[ cIdx ] | ue(v) |
| idq_init_qp_offset[ cIdx ] | ue(v) |
| for( i = 0; i < idq_num_intervals_minus1[ cIdx ]; i++ ){ |  |
| idq_interval_width_minus1[ cIdx ][ i ] | ue(v) |
| idq_qp_offset_delta_sign[ cIdx ][ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

# Picture parameter set RBSP semantics

idq_enabled_flag equal to 0 specifies that intensity dependent quantisation is disabled. idq_enabled_flag equal to 1 specifies that intensity dependent quantisation is enabled.

idq_num_intervals_minus1[ cIdx ] plus 1, specifies the number of intervals in the IDQ histogram for component cIdx.

idq_init_qp_offset[ cIdx ] specifies the QP offset of the first IDQ histogram bin for component cIdx.

idq_interval_width_minus1[ cIdx ][ j ] plus 1, specifies the width of the j’th interval of the IDQ histogram for component cIdx.

idq_qp_offset_delta_sign[ cIdx ][ j ] specifies the sign of the ±1 difference between the j’th and (j+1)’th intervals of the IDQ histogram for component cIdx.

The variable idqProfile[ cIdx ][ j ] contains the reconstructed QP offset that is applied to TUs that have a predictor with an average sample intensity j for component cIdx. idqProfile[ cIdx ][ j ] is derived as follows:

for( cIdx = 0; cIdx < 3; cIdx++ ){
	intervalEnd = 0
	i = j = 0
	qpOffset = idq_init_qp_offset[ cIdx ]
	for(; i < idq_num_intervals_minus1[ cIdx ]; i++ ){
		intervalEnd += idq_interval_width_minus1[ cIdx ][ i ] + 1
		while( j < intervalEnd && j < 256 )
			idqProfile[ cIdx ][ j++ ] = qpOffset
		qpOffset += 1 – ( idq_qp_offset_delta_sign[ cIdx ][ i ] << 1 )
	}
	while( j < 256 )
		idqProfile[ cIdx ][ j++ ] = qpOffset
}

# Changes in derivation process for quantization parameters

### Derivation process for intensity dependent quantization parameters

Inputs of this process are:

–	a variable nW specifying the width of the current block,

–	a variable nH specifying the height of the current block,

–	an (nW)x(nH) array predSample containing the predicted samples for the current block.

Outputs of this process are:

–	the values IdqOffsetQpY, IdqOffsetQpCb and IdqOffsetQpCr for the current block.

If idq_enabled_flag is equal to zero, the following applies, the value of IdqOffsetQpY, IdqOffsetQpCb and IdqOffsetQpCr shall be set to zero.

Otherwise, idq_enabled_flag is equal to one, the values IdqOffsetQpY, IdqOffsetQpCb and IdqOffsetQpCr are derived as follows:

IdqOffsetQpY = idqProfile[ 0 ][ computeIdqAverage( bit_depth_luma_minus8 ) ]
IdqOffsetQpCb = idqProfile[ 1 ][ computeIdqAverage( bit_depth_chroma_minus8 ) ]
IdqOffsetQpCr = idqProfile[ 2 ][ computeIdqAverage( bit_depth_chroma_minus8 ) ]

And computeIdqAverage( bitDepthMinus8 ):

for( y = nT; y < nT + nH; y++ )
	for( x = nT; x < nT + nW; x++ ){
		sum += predSample[ x, y ]
	}
offset = bitDepthY = = 8 ? 0 : 1 << ( bitDepthY – 9 )
sum = ( sum + offset ) >> (bitDepthY – 8)
return sum / ( nW * nH )

### Derivation process for quantization parameters

Inputs of this process are:

–	luma location ( xB, yB ) specifying the top-left luma sample of the current coding unit quantization group relative to the top-left luma sample of the current picture,

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

–	the variables IdqOffsetQpY, IdqOffsetQpCb and IdqOffsetQpCr, derived according to subclause XXX.

Outputs of this process are:

–	luma quantization parameter QP’Y,

–	chroma quantization parameters QP’Cb and QP’Cr

Let a variable QPY_A specifying the luma quantization parameter of the coding unit quantization group covering ( xB−1,  yB ), a variable tbAddrA specifying the treeblock address of treeblock containing the coding unit quantization group covering ( xB−1,  yB ), a variable QPY_B specifying the luma quantization parameter of the coding unit quantization group covering ( xB,  yB−1 ), a variable , tbAddrB specifying the treeblock address of treeblock containing the coding unit quantization covering ( xB,  yB−1 ), and a variable QPY_PREV specifying the luma quantization parameter of the previous coding unit quantization group in decoding order, respectively.

When at least one of the following conditions is true, QPY_PREV is initially set equal to SliceQPY at the start of each slice for the first coding unit quantization group in the slice.

–	A quantization group of coding units is the first quantization group of coding units in a slice.

–	A quantization group of coding units is the first quantization group of coding units in a tile.

–	A quantization group of coding units is the first quantization group of coding units in a treeblock row and tiles_or_entry_coding_sync_idc is equal to 2.

The prediction of luma quantization parameter QPY_PRED is derived as the following ordered steps:

- If the coding unit quantization group covering ( xB−1,  yB ) in the current slice is not available or tbAddrA is not equal to CurrTbAddr, then QPY_A is set equal to QPY_PREV.

- If the coding unit quantization group covering ( xB,  yB−1 ) in the current slice is not available or tbAddrB is not equal to CurrTbAddr, then QPY_B is set equal to QPY_PREV.

- The prediction of luma quantization paramaeter QPY_PRED is derived as:

QPY_PRED =  (QPY_A + QPY_B + 1) >> 1

The value of QPY is derived as

QPY = ( ( ( QPY_PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY

The value of luma quantization parameter QP’Y is derived as

QP’Y = QPY + QpBdOffsetY + IdqOffsetQpY

The values of chroma quantization parameters QP’Cb and QP’Cr are derived as

QP’Cb = Clip3( −QpBdOffsetC, 51, QPCb + cb_qp_offset) + QpBdOffsetC + IdqOffsetQpCb

QP’Cr = Clip3( −QpBdOffsetC, 51, QPCr + cr_qp_offset) + QpBdOffsetC + IdqOffsetQpCr

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the transform unit containing the sample p0,0.	(8-1)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-2)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-3)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-4)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-5)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-6)

dpq0 = dp0 + dq0		(8-7)

dpq3 = dp3 + dq3		(8-8)

dp = dp0 + dp3		(8-9)

dq = dq0 + dq3		(8-10)

- The variable d is derived as follows:

d = dpq0 + dpq3		(8-11)

- The variables dE, dEp and dEq are set equal to 0.

- if d  is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0, the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-12)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-13)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 | 		(8-14)

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-15)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-16)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-17)

dpq0 = dp0 + dq0		(8-18)

dpq3 = dp3 + dq3		(8-19)

dp = dp0 + dp3		(8-20)

dq = dq0 + dq3		(8-21)

- The variables d is derived as follows:

d = dpq0 + dpq3		(8-22)

- The variables dE, dEp and dEq are set equal to 0.

- if d is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0 the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + Xb + 3, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

##### Filtering process for chroma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is greater than 1.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the transform unit containing the sample p0,0.	(8-23)

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 53, qPL + 2*(bS − 1) + (tc_offset_div2 << 1 ) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-24)

pi = s’[ xC + xB – i − 1, yC + yB + k ]		(8-25)

- The following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-26)

s’[ xC + xB − 1, yC + yB + k ] = p0’		(8-27)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to −1 and 2 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 2 respectively. Otherwise ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-28)

pi = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-29)

- If xPOS is less than 0, and if bSL, is greater than 1, for each sample location ( xC + xB - 1, yC + yB ), the following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSL and the variable tCL as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-30)

s’[ xC + xB +k, yC + yB – i − 1 ] = pi’		(8-31)

- If bS is greater than 1, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke, the following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-32)

s’[ xC + xB +k, yC + yB − 1 ] = p0’		(8-33)