| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0257_r1 |
| --- | --- |

| Title: | On Intensity Dependent Quantisation in the HEVC codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak David Flynn Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk davidf@rd.bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This contribution presents the application of a quantisation tool that exploits masking phenomena related to the Human Visual System (HVS) in the High Efficiency Video Coding (HEVC) codec. The pixel intensity masking is considered in this contribution, which leads the HVS to be less sensitive to the coding distortion introduced in image areas where the average pixel intensity is either low or high. To exploit this space varying distortion, a mapping between the average block pixel intensity and the quantisation step is considered and used in the HEVC quantiser. With this mapping, the quantisation step is perceptually tuned on a Transform Unit (TU) basis and does not need to be transmitted at the decoder as a decoder side average pixel intensity estimation is adopted. Finally, this contribution presents a method to signal the mapping between the average pixel intensity and the quantisation step. The performance for the proposed perceptual quantisation are assessed by measuring the bitrate reductions at the same perceptual quality level of the HM-6.0 and by measuring the BD-rates assuming as reference a perceptual quantisation method which transmits the QP variations. For the same perceptual quality level the bitrate is reduced on average by 3%. Moreover, an average BD-rate saving of 3% is reported against the HM-6.0 codec which transmits the QP variations.

# Introduction

The Human Visual System (HVS) shows nonlinear sensitivity to the distortion introduced by lossy image and video coding [1], [2]. This nonlinear sensitivity is due to spatial and temporal masking phenomena. The former masking relates to the spatial features in the image being coded while the latter is related to the motion component in videos. The overall effect brought by these masking phenomena is the so-called Just Noticeable Distortion (JND) [1]. The JND refers to the maximum level of distortion that can be introduced in an image or video content without being noticeable by human observers. Literature reports several proposals to model the aforementioned masking phenomena (see [1] and references therein) and the associated JND. In this contribution, only the masking due to the average pixel intensity is considered. In particular, this masking leads the HVS to be less sensitive to the distortion introduced in image areas (e.g. blocks) with lower or higher average pixel intensities. Given this different distortion sensitivity, the quantisation step may be coarser in image areas with lower or higher pixel intensities and may be finer otherwise. For a given range of intensity values associated to a given image block, the mapping between each intensity value and the quantisation step value is hereafter denoted as Intensity Dependent Quantisation (IDQ) profile. An example of IDQ profile is depicted in Figure 1 where µ denotes the average pixel intensity for a given image block in the original video being compressed while Δ denotes the quantisation step associated via the profile. The IDQ profile can be used to increase the quantisation step in image areas where the HVS is less sensitive to the distortion. In this way a perceptual quantisation step Qperc is derived which depends on the average intensity value µ for the original image block being coded. As stated above, µ is computed over the original video data which is now needed also at the decoder side for inverse quantisation. It should be noted that sending the µ value for each block may significantly increase the coding rate. Another issue related to the usage of the IDQ profile is how to transmit the profile values. Considering that the video content producers should be left free to choose the IDQ profile with a fine level of granularity (e.g. frame level), the bits associated to the IDQ profile transmission may be considerably high. This contribution addresses the following issues related to the usage of IDQ:

- How to transmit/recover the average pixel intensity µ for each coded block.

- How to translate the IDQ profile values into the QP to be used in the TU being considered.

- How to transmit the IDQ profile values to the decoder.

- How to use the perceptual quantisation step in the deblocking filter

Figure 1: IDQ profile curve example whereby µ denotes the average pixel intensity and Δ the associated quantisation step.

# Intensity dependent quantisation in the HM codec

## Average pixel intensity estimation

Most of the IDQ profiles are proposed to operate in the frequency domain as this is the domain where quantisation takes place in video codecs. In this proposal, the TU is the level where µ is computed to obtain the Δ(µ). Instead of using the average pixel intensity value for each TU, this proposal uses the method derived in [3] whereby µ is estimated by computing the average over the TU predictor pixels. In particular, with this method an estimate μ for µ can be derived as follows:

μ=1M×Nx=0M−1y=0N−1P(x,y),	(1)

where M, N denote the TU height and width respectively, and P(·) is the (temporal or spatial) predictor for the TU being coded. In this proposal, the image component used to drive the proposed IDQ is the luminance one. Therefore, also for chroma components, their quantisation step selection is driven by the average luminance intensity.

All the IDQ profiles that have been proposed in the literature ([4], [5] and [6]) assume the luminance values in the range [0, 255] (i.e. 8 bit pixel depth). Therefore, in this proposal, the average intensity value μ is scaled to normalise its value in the range [0, 255]. In particular, the shift quantity shift1 is defined as:

shift1 = processingBitDepth – 8,	(2)

where the variable processingBitDepth represent the internal bit depth used by the HEVC codec to process data. The shift1 quantity is used to perform an intermediate scaling of the summation in (1) as follows:

scaledSum=x=0M−1y=0N−1P(x,y)+(1<<(shift1−1))>>shift1,	()

where >> and << denote the shift operation toward right and left, respectively. Finally, given that the TU sizes (height or width) in HEVC are always power of 2, the final μ value may be computed using only additions and shifts as follows:

μ=scaledSum+(1<<(log2(M×N)−1))>>(log2(M×N)).	(4)

## Mapping between Δ(µ) and the quantisation parameter

To translate the IDQ profile values Δ(µ) into the QP to be used for a given TU, the relation between the quantisation step and the QP for the HEVC quantiser is considered:

Δ=2QP−46.	(5)

By inverting equation (5) the QP associated to Δ can be computed. Also, the QP variation given by the IDQ profile can be expressed with respect to the base (non-perceptual) QP used for the TU being coded. This quantity is hereafter denoted as intensity differential QP (idQP):

idQP(μ)=6⋅log2(Δ(μ))+0.5,	(6)

where the rounding operation is necessary to keep the idQP values integers. The final perceptual QP (QPperc) can be then obtained as QPperc = QP + idQP(µ).

## IDQ profile signalling

Once each IDQ profile value Δ(µ) is converted into its correspondent idQP(µ) the initial IDQ profile depicted in Figure 1 assumes the staircase shape shown in Figure 2. This staircase shape can be exploited to signal the IDQ profile to the decoder. Therefore, in this proposal the idQP values are encoded using Differential Pulse Code Modulation (DPCM). More precisely, the whole µ range ([0, 255]) is divided into bins whereby each bin is characterized by its width (w) and the quantity δ = idQP(bink) – idQP(bink-1). As an example, the idQP in Figure 2 has 15 bins. In this proposals is assumed that δ takes values +1 or -1 since the smooth variations for all the IDQ profiles proposed in the literature ([1], [4]) prevent δ to take different values. Once all the bin values (i.e. w and δ) are computed for the whole IDQ profile they are transmitted in the Picture Parameter Set (PPS).

Figure 2: IDQ profile conversion when the IDQ profile value Δ(µ) is translated into the idQP(µ).

## Perceptual quantisation step usage in the deblocking filter

The deblocking stage of HEVC depends on the level of quantisation; in particular, deblocking uses the value of the quantisation step to determine the level of filtering to apply. This proposal has the potential to derive a different quantisation step for each TU in a coding unit. Moreover, using two different IDQ profiles for the two chroma components may lead to different quantisation steps. Therefore it is necessary to select the appropriate quantisation step for the calculation of the deblocking filter for each component.

For the first issue, quantisation step selected at the TU level, it is proposed to derive the deblocking filter parameters (variable qPL in [7]) using the TU level quantisation step. With the reference to Figure 3, the current HEVC standard draft sets qPL to:

qPL = (QPY + QPP + 1) >> 1, 	(7)

where QPP refers to the QP of CU(i). This proposal assumes that QPP refers to the QP of the TU which contains the pixel P in Figure 3.

Figure 3: Example of vertical edge filtered by deblocking (in red).

# Experimental results

The perceptual quantization presented in this proposal has been integrated in the HEVC codec version 6.0 (HM-6.0) and tested over the JCT-VC test contents. The chosen encoder configurations are the ones relative to the common test conditions [7]. The chosen IDQ profile is the one proposed in [4] whereas the values have been modified by the authors of this proposal after a preliminary subjective viewing on the test contents carried out at the BBC R&D premises. The IDQ profile values for the different tested QPs are reported in the appendix of this document. To assess the performance brought by the proposed IDQ, two different experiments have been performed. In the first experiment, the bitrate reductions with respect to the HM-6.0 codec have been measured assuming the same level of perceptual quality. To verify the same level of perceptual quality, an informal viewing has been done by the authors over several tested contents, namely on those where differences in the PSNR values between the sequences coded with HM-6.0 and with IDQ were higher than 0.5 dBs. In the second experiment, the performance of the proposed IDQ are compared with a non-normative perceptual quantisation technique. In particular, the HM-6.0 codec has been modified to compute for each CU the average luminance value from the original data. The QP for this CU is then set in the same way as described in Section 2.2. The QP variations among the different CUs are then signalled as specified in the HEVC standard draft [7]. It should be noted that this non-normative perceptual quantisation technique can adjust the quantisation step at the minimum level of 8×8 CU rather than the TU level as in IDQ. Since the two perceptual quantisation algorithms work in the same fashion, the BD-rates will be reported for this second experiment.

## Bitrate reductions with respect to the HM-6.0 codec

Figure 4 reports for each tested configuration the Cumulative Distribution Function (CDF) of the bitrate reductions with respect to the HM-6.0 codec. As may be noted, there is an average bitrate reduction (across all tested QPs) of up to 3% for all intra configuration. At the 50% of tested sequences (i.e. the median point) the average bitrate reduction is about 4.5% for ra, ld and ldP configuration whilst is about 6% for intra. Moreover, by limiting the PSNR variations between HM-6.0 and the HM-6.0 equipped with the proposed IDQ to 0.5 dB, the bitrate reductions for some video contents are listed in Table 3-1. The whole detailed results for the whole test are provided in the spreadsheet attached with this contribution.

Figure 4: Bitrate reductions with respect to HM-6.0 assuming the same level of perceptual quality.

Table 3-1: Bitrate reductions for some sequences from the test set.

| Configuration | Sequence | QP | ΔPSNR [dB] | ΔRate [%] |
| --- | --- | --- | --- | --- |
| ra_he10 | SteamLocomotive | 22 | 0.31 | -18.06 |
| ld_he10 | BQTerrace | 27 | 0.25 | -14.32 |
| ldP_he10 | Cactus | 22 | 0.26 | -13.38 |
| ra_he10 | NebutaFestival | 37 | 0.11 | -11.54 |
| ldP_he10 | Kimono | 22 | 0.25 | -8.82 |
| ldP_he10 | BasketballDrive | 22 | 0.19 | -7.91 |
| ldP_main | Kimono | 22 | 0.23 | -8.14 |
| i_he10 | Kimono | 32 | 0.47 | -7.51 |
| ldP_he10 | ChinaSpeed | 37 | 0.38 | -7.01 |
| ld_he10 | BQSquare | 27 | 0.45 | -6.53 |
| i_main | SteamLocomotive | 37 | 0.36 | -6.31 |
| ra_he10 | PeopleOnStreet | 22 | 0.48 | -5.98 |
| ldP_he10 | BQMall | 22 | 0.33 | -5.96 |
| ra_he10 | SteamLocomotive | 32 | 0.17 | -5.35 |
| ld_he10 | KristenAndSara | 22 | 0.23 | -5.64 |
| ld_he10 | Johnny | 22 | 0.18 | -5.26 |

## BD-rates with respect to the non-normative perceptual quantisation

Table 3-1 reports the BD-rates obtained between the proposed IDQ and the non-normative perceptual quantisation described above and assumed as reference. As may be noted an average bitrate reduction of 3% is observed.

Table 3-2: BD-rates assuming as reference the non-normative perceptual quantization.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -4.3% | -1.8% | -1.3% | -4.3% | -1.6% | -1.3% |
| Class B | -2.8% | -0.8% | -0.4% | -2.8% | -0.9% | -0.6% |
| Class C | -2.8% | -1.3% | -1.3% | -2.8% | -1.5% | -1.4% |
| Class D | -2.4% | -0.9% | -0.8% | -2.3% | -0.9% | -0.9% |
| Class E | -4.6% | -1.4% | -1.1% | -4.5% | -1.5% | -0.8% |
| Overall | -3.2% | -1.1% | -0.9% | -3.2% | -1.2% | -1.0% |
|  | -3.2% | -1.2% | -0.9% | -3.1% | -1.2% | -1.0% |
| Class F | -1.7% | 0.7% | 0.7% | -1.8% | 0.5% | 0.9% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -3.5% | -3.5% | -3.2% | -3.6% | -3.4% | -3.0% |
| Class B | -2.8% | -3.4% | -2.8% | -2.8% | -3.5% | -2.9% |
| Class C | -2.7% | -2.6% | -2.6% | -2.7% | -2.8% | -2.3% |
| Class D | -2.6% | -3.0% | -3.2% | -2.4% | -3.0% | -3.1% |
| Class E |  |  |  |  |  |  |
| Overall | -2.8% | -3.1% | -2.9% | -2.8% | -3.2% | -2.8% |
|  | -2.8% | -3.1% | -3.0% | -2.8% | -3.3% | -2.9% |
| Class F | -2.0% | -1.4% | -0.9% | -1.8% | -1.3% | -1.2% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.4% | -5.1% | -5.1% | -3.3% | -5.2% | -4.1% |
| Class C | -3.3% | -4.1% | -4.1% | -3.1% | -4.0% | -4.1% |
| Class D | -3.3% | -4.6% | -5.2% | -2.9% | -4.6% | -4.5% |
| Class E | -3.3% | -5.7% | -4.2% | -3.4% | -6.6% | -6.3% |
| Overall | -3.3% | -4.8% | -4.7% | -3.2% | -5.0% | -4.6% |
|  | -3.3% | -4.9% | -4.7% | -3.2% | -5.2% | -4.8% |
| Class F | -3.3% | -5.3% | -5.6% | -3.2% | -4.3% | -3.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

# Conclusions

This proposal has presented the integration of some HVS characteristics in the HM, notably the JND due to the average pixel intensity. The integrated IDQ profile allows to perform perceptual quantization, thus to allocate the available rate in a perceptual fashion. Over the whole test set and configurations it can guarantee a bitrate reduction of up 3% for the same subjective quality. Moreover, compared to a non-normative perceptual quantisation, the bitrate saving by not sending the varying quantisation step is on average 3% as well. It should be noted that the presented perceptual quantization can be coupled with the default frequency weighting matrices [9] or with others customized matrices to further optimize the perceptual quality for a given target rate.

# References

- H. R. Wu and K. R. Rao, “Digital video image quality and perceptual coding”, CRC press, pages 640, November 2005.

- N. Jayant, J. Johnston and R. Safranek, “Signal compression based on models of human perception”, Proceedings of the IEEE, vol. 81, no. 10, pp. 1385-1422, October 1993.

- M. Naccari and F. Pereira, “Advanced H.264/AVC-based perceptual video coding: architecture, principles and assessment”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 21, no. 6, pp. 766-782, June 2011.

- Y. Jia, W. Lin and A. A. Kassim, “Estimating the just-noticeable distortion for video”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 16, no. 7, pp. 820-829, July 2006.

- C.-H. Chou and Y.-C. Li, “A perceptually tuned subband image coder based on the measure of just-noticeable distortion profile”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 5, no. 6, pp. 467-476, June 1995.

- Z. Wei and K. N. Ngan, “Spatio-temporal just noticeable distortion profile from grey scale image/video in DCT domain”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 19, no. 3, pp. 337-346, March 2009.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, 8th meeting, San José, CA, USA, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-H1100, 8th meeting, San José, CA, U.S.A, February 2012.

- M. Haque, A. Tabatabai and Y. Morigami, “HVS model based default quantization matrices”, JCTVC-G880, 7th meeting, Geneva, Switzerland, November 2011.

# Appendix: used IDQ for each image component

As stated in Section 3 the used IDQ profile is the one proposed in [4]; this profile consists in a parabolic piecewise profile whose values are given as follows:

Δ(μ)=2⋅1−2μ2563+1μ≤1280.8⋅2μ256−12+1otherwise.	(8)

The IDQ profile used in this contribution uses the same shape as in (8) with the values reported from Table 6-3-Table 6-5 expressed as pairs (µ, idQP) where µ is the average luminance intensity where the idQP value is observed. This idQP value is set equal for all the subsequent µ values until a new pair is encountered.

Table 6-3: IDQ luminance profile pairs for QP 22 and 27.

|  | Pair number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0..13] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| µ | 0 | 3 | 7 | 12 | 17 | 21 | 26 | 30 | 35 | 40 | 45 | 51 | 57 | 64 |
| idQP | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  | Pair number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [14..27] | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| µ | 76 | 120 | 136 | 147 | 158 | 168 | 178 | 187 | 197 | 207 | 217 | 227 | 238 | 249 |
| idQP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |

Table 6-4: IDQ luminance profile pairs for QP = 32.

|  | Pair number |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0..11] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| µ | 0 | 3 | 8 | 13 | 18 | 23 | 29 | 34 | 39 | 46 | 52 | 61 |
| idQP | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  | Pair number |  |  |  |  |  |  |  |  |  |  |  |
| [12..23] | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| µ | 73 | 123 | 142 | 156 | 169 | 180 | 192 | 203 | 215 | 226 | 238 | 250 |
| idQP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

Table 6-5: IDQ luminance profile pairs for QP = 37.

|  | Pair number |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0..10] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| µ | 0 | 2 | 8 | 13 | 18 | 24 | 30 | 36 | 42 | 49 | 58 |
| idQP | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  | Pair number |  |  |  |  |  |  |  |  |  |  |
| [11..21] | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| µ | 72 | 126 | 147 | 163 | 177 | 190 | 203 | 215 | 228 | 241 | 254 |
| idQP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).