| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0258_r1 WG11 Number: m24502 |
| --- | --- |

| Title: | On deblocking clipping table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document proposes to modify the deblocking clipping tables by using higher clipping values in the part related to higher QPs. This change would enable stronger filtering at higher QP as well as the stronger deblocking filtering by sending deblocking filter offset. The proposed changes do not have effect on common test conditions.

# Proposed modifications to deblocking clipping table

Currently, deblocking filtering in HM uses a clipping table shown below.

const UChar tctable_8x8[56] =

{

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,1,2,2,2,2,3,3,3,3,4,4,4,5,5,6,6,7,8,9,9,10,10,11,11,12,12,13,13,14,14

};

It is proposed to replace the deblocking clipping table with the following table.

const UChar tctable_8x8[54] =

{

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,1,2,2,2,2,3,3,3,3,4,4,4,5,5,6,6,7,8,9,10,11,13,14,16,18,20,22,24

};

The difference between the current and the proposed clipping tables is shown in Figure 1.

Figure 1 Current and proposed clipping (tc) tables plotted against the QP value.

The motivation to change deblocking table is that the current deblocking tables allows less filtering for the higher QP range compared to H.264/AVC deblocking tables (see Figure 2).

Figure 2 Comparison of proposed deblocking table (black and crosses) to H.264/AVC tables (color lines)

Comparison of proposed deblocking table against the H.264/AVC deblocking filters is shown in Figure 3

Figure 3 Comparison of proposed deblocking clipping tables (black, dots) against current HEVC clipping  table (black, crosses) and H.264/AVC (color lines).

One should also notice that the proposed modifications do not change the results on common test conditions since the modified filter values lie outside the common conditions QP range.

# Conclusions

It is proposed to adopt the changes to the deblocking clipping table described in this proposal and include them in the next version of HM and the standard draft.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).