| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0261 M24508 |
| --- | --- |

| Title: | Non-CE1: Edge Offset Index Coding for LCU-based SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, SAO parameter is signaled for each LCU. Signaling offset directly for each color component of every LCU is expensive. To reduce overhead for edge offset (EO) coding, this contribution proposes to use a fixed-size table and to signal two indices, one for Category 1 & 2 and the other for Category 3 & 4, respectively. The size of table is fixed to 8, and default pairs of offsets in all 8 entries were chosen and sorted based on the statistics gathered offline so that the most probable pair of offsets has the index value of 0. Each color component has its own EO table, and they are initialized to the default EO table before decoding the first LCU in a slice. After decoding a LCU, the EO table entries of each color component are reordered based on the decoded indices, so that the EO tables can be adapted on the fly.

Compared to CE1 anchor, for low-delay B Main configuration, it is reported that the proposed method provides BD-rate gain of (Y: 0.0%, Cb: 1.3%, Cr: 1.6%), (Y: 0.1%, Cb: 0.6%, Cr: 0.9%) and (Y: 0.1%, Cb: 0.0%, Cr: -0.1%) for LCU size of 64, 32 and 16. For low-delay B HE10 configuration, it is reported that the proposed method provides BD-rate gain of (Y: 0.2%, Cb: 1.0%, Cr: 1.1%), (Y: 0.2%, Cb: 0.4%, Cr: 0.7%) and (Y: 0.1%, Cb: -0.2%, Cr: -0.3%) for LCU size of 64, 32 and 16.

# Introduction

In HM-6.0 [1], SAO parameters can be signaled for each LCU, where in case of EO four offsets are signaled for each color component. Signaling offsets directly for each color component of every LCU can be expensive in LCU-based SAO. Therefore, it is desirable to develop an efficient method to signal those offsets.

We observed that the range of edge offset value shows different statistics depending on edge category. Fig. 1 shows edge shapes according to edge category. It can be noticed that edge strength of edges in categories 1 and 4 tends to be stronger than those in categories 2 and 3. We have verified this through our experiments using HM-6.0. It can be also noticed that edge shapes in categories 1 and 2 may not have strong correlation to those in categories 3 and 4.

Fig. 1. Edge shapes depending on edge offset categories.

Based on these statistics, to reduce bits for edge offset coding, we propose an EO coding method using indices with an adaptive fixed-size EO table for each color component.

The proposed method consists of two parts: initialization of EO tables and EO index coding/decoding, and adaptive EO table reordering based on coded/decoded indices.

# Proposed Method

2.1 Edge offset Index Coding

The proposed method signals two indices, one for Category 1 & 2 and the other for Category 3 & 4, respectively, instead of coding four edge offsets directly. For this purpose, the default EO tables were generated for Main configuration (8-bit) and HE10 configuration (10-bit) based on the statistics we gathered offline according to the common test condition [2]. First, an offset pair is formed by combining offsets of categories 1 & 2, or offsets of categories 4 & 3, in order. Note that absolute value was taken in case of offsets of categories 3 and 4. Then, we selected 8 most frequently used pairs to form the table, and according to its frequency an index is assigned as in Table 1. Note that the same table is used for these two indices.

Table 1. Default EO tables

|  | Main (8-Bit) |  | HE10 (10-Bit) |  |
| --- | --- | --- | --- | --- |
| Index | Offset_1 | Offset_2 | Offset_1 | Offset_2 |
| 0 | 1 | 0 | 3 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 | 1 | 1 | 7 | 0 |
| 3 | 2 | 0 | 1 | 0 |
| 4 | 2 | 1 | 2 | 0 |
| 5 | 3 | 0 | 6 | 0 |
| 6 | 3 | 1 | 11 | 0 |
| 7 | 4 | 1 | 12 | 0 |

Encoder signals two indices to code four edge offsets for each color component, one for Category 1 & 2 and the other for Category 3 & 4. Encoder is allowed to use offset pairs defined in Table 1 only. So, encoder performs RDO among the offsets in Table 1 to find the best edge offsets. At the decoder side, for the first index decoded, EOCategory1 and EOCategory2 are set to Offset_1 and Offset_2, respectively. For the second index decoded, EOCategory3 is set to –Offset_2 and EOCategory4 is set to –Offset_1.

Each color component has its own EO table, but all EO tables of Y, Cb, and Cr are initialized to the default EO table before decoding the first LCU in a slice.

2.2 Edge Offset Index Reordering

The occurrence of each pair of offsets in Table 1 can vary depending on the characteristics of sequence, frame, and coding conditions like QP, GOP structure, etc. To make the order of offset pairs in the EO table adaptive to their frequencies, the index reordering method is proposed, so that more probable pair of offsets can have a smaller index value. We introduce the index counter to count how many times each index is used for each color component. All the counters are initialized to 0 before encoding/decoding the first LCU in a slice.

For each color component, after decoding both indices for Category 1 & 2 and Category 3 & 4, the index reordering is performed twice (once per one index) as follows:

1. Let k (0 <= k <= 7) be the index. Whenever index k is used, increase the counter of index k by 1, i.e.    numIndexCount[k] = numIndexCount[k] + 1, where numIndexCount is the array of the counters of all indices.

2. If k is larger than 0, compare numIndexCount[k] and numIndexCount[k- 1]. If numIndexCount[k] is larger than numIndexCount[k-1], swap the table entries of two indices.

That is, rather than sorting all entries again, we simply compare the current index count with the previous index count, and swap them when the current index count is larger. For the first LCU, the default EO table is used for all color components to get edge offsets from decoded indices. From the second LCU, the reordered EO tables are used, which can be different for different color component after reordering. In case of interleaving mode, the index is coded using CABAC with unary binarization. Note that what is coded is not the index value in Table 1, but the table entry number (from 0 to 7) after reordering.

# Experimental Results

The proposed method is evaluated based on the CE1 common test condition [3]. We used CABAC for RDO to estimate SAO parameter coding bitrate at the encoder side. Note that only the interleaving mode is implemented and tested.

Experimental results are summarized in Table 2. Compared to CE1 anchor, for low-delay B Main configuration, the proposed method provides BD-rate gain of (Y: 0.0%, Cb: 1.3%, Cr: 1.6%), (Y: 0.1%, Cb: 0.6%, Cr: 0.9%) and (Y: 0.1%, Cb: 0.0%, Cr: -0.1%) for LCU size of 64, 32 and 16. For low-delay B HE10 configuration, the proposed method provides BD-rate gain of (Y: 0.2%, Cb: 1.0%, Cr: 1.1%), (Y: 0.2%, Cb: 0.4%, Cr: 0.7%) and (Y: 0.1%, Cb: -0.2%, Cr: -0.3%) for LCU size of 64, 32 and 16.

Table 2. BD-rate results of the proposed method compared to CE1 anchor.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | 0.1% |
| Class C | -0.1% | 0.0% | 0.2% | -0.1% | 0.1% | 0.5% |
| Class D | -0.1% | -0.4% | 0.2% | 0.0% | 0.9% | 0.6% |
| Class E | -0.2% | 0.2% | 0.1% | -0.1% | 0.0% | -0.2% |
| Overall | -0.1% | 0.0% | 0.1% | -0.1% | 0.2% | 0.3% |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.3% | 0.3% |
| Class F | -0.1% | -0.6% | 0.6% | -0.3% | -1.5% | 1.1% |
| Enc Time[%] | 105% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 95% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.6% | -1.1% | -0.2% | -0.6% | -0.3% |
| Class C | -0.1% | -0.4% | -0.5% | -0.3% | -0.4% | -0.4% |
| Class D | -0.1% | -0.6% | -0.8% | -0.1% | -0.3% | -1.5% |
| Class E | 0.0% | -0.7% | -1.2% | -0.3% | -0.3% | -0.6% |
| Overall | -0.1% | -0.6% | -0.9% | -0.2% | -0.4% | -0.7% |
|  | -0.1% | -0.7% | -1.0% | -0.2% | -0.4% | -0.7% |
| Class F | -0.2% | 0.9% | 0.0% | -0.3% | -1.3% | -1.9% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.3% | -2.1% | -0.1% | -0.9% | -0.9% |
| Class C | 0.0% | -1.3% | -1.3% | -0.2% | -1.1% | -1.5% |
| Class D | 0.0% | -1.3% | -1.9% | -0.2% | -1.3% | -1.5% |
| Class E | 0.0% | -1.5% | -1.0% | -0.2% | -0.6% | -0.2% |
| Overall | 0.0% | -1.3% | -1.6% | -0.2% | -1.0% | -1.1% |
|  | 0.0% | -1.3% | -1.6% | -0.2% | -1.0% | -1.1% |
| Class F | 0.0% | -0.6% | -0.3% | -0.3% | 0.3% | -0.1% |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 91% |  |  | 98% |  |  |

# Conclusion

The EO coding method using indices in an adaptive fixed-size EO table is proposed and tested. It was shown that the proposed method provides coding efficiency improvements in Luma and Chroma for Main (8-bit) and HE10 (10-bit) configurations when compared to CE1 anchor.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb. 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).