| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0262 WG11 Number: m24509 |
| --- | --- |

| Title: | Extension of HEVC NAL Unit Syntax Structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document presents the high level syntax hooks in the “base” HEVC NAL Unit Syntax structure for its potential extensions. An efficient and generic NAL unit header can be designed in a unified fashion with an extension segment that enables it to address various possible extensions of HEVC extensions in the future.

- HEVC NAL Unit Syntax Structure Extension

The NAL Unit syntax structure in the current Committee Draft does not address next HEVC extensions data structures.  In our contribution, the existing NAL Unit syntax structure is extended to include future HEVC extensions by using one of the 5 reserved bits as a flag to indicate such extensions. Besides this, there is no other impact in the "base" NAL Unit structure. In the extension part of this NAL Unit structure, syntax parameters are introduced with register type fixed byte-sizes. This extension process also provides a generic framework for various future combinations of scalability and multi-view coding extensions. This may result some bits in some syntaxes may not be fully utilized in some cases. However this is expected, as more generalizations are attempted in such syntax element structures to meet the varied requirements for different HEVC extensions.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| nal_byte_1 | u(8) |
| if ((nal_byte_1 & 8) > 0) { |  |
| nal_flag_set | u(8) |
| if ((nal_byte_1 & 16) ==0) { |  |
| nal_bytes_2 | u(16) |
| NumBytesInRBSP +=3 |  |
| } |  |
| else { |  |
| nal_bytes_3 | u(24) |
| NumBytesInRBSP +=4 |  |
| } |  |
| } |  |
| NumBytesInRBSP += 1 |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

In the above syntax structure, 4 new syntax elements are introduced which are described below:

- nal_byte_1  Syntax Element:

The syntax element “nal_byte1” has 2 HEVC configurations: “base” or “Extension”

- The first configuration is for “base” HEVC. It uses 1 bit out of the original “reserved_one_5bits” as an extension flag (ext_flag1) and keeps the remaining 4 bits as reserved for possibly “debugging” related hardware/software issues. Here “ext_flag1” is set to 0.

| reserved (4-bits) | ext_flag1 (1-bit) | temporal_id (3-bits) |
| --- | --- | --- |

- The second configuration is for HEVC Extensions. For the syntax element “nal_byte1”, “ext_flag1” is set to 1. In addition, this syntax element contains 3 more fields of “ext_flag2”, “nal_id_mask1” and “nal_id_mask2” as shown below:

| ext_flag2: For the HEVC Extensions, this 1-bit flag of “ext_flag2” when set to 0, it indicates nal_id_mask2 (3-bits) | ext_flag2 (1-bit) | ext_flag1 (1-bit) | nal_id_mask1 (3-bits) |
| --- | --- | --- | --- |

the allocation of a 2-byte long syntax element “nal_bytes_2”, and if set to 1, indicates the allocation of a 3-byte long syntax element “nal_bytes_3” in the next syntax-flow.

nal_id_mask1, nal_id_mask2: These two fields, each being 3-bits long, are used to indicate which types of nal_IDs are present in the next syntax elements for an HEVC extension. The individual bit in these two fields, when set to 1, represents the presence of a specific nal_ID in next syntax elements in the order such bit is present from right to left locations. This is explained in the tables shown below with some examples. In these two fields, although the sizes of such nal_IDs are listed according to the previous extensions specifications of AVC - SVC and MVC, but these can be adjusted or redistributed among these IDs themselves.

The “reserved” location in “nal_id_mask2” can be used for inserting any other nal_ID for other HEVC extensions. The example definitions of the two fields of (nal_id_mask2, nal_id_mask1) can be modified according to any future HEVC extension. As mentioned earlier, the sizes of such nal_IDs can be re-adjusted according to a specific HEVC extension type, as needed.

Some default examples for the two fields (nal_id_mask2, nal_id_mask1) are provided below for HEVC extensions according to the previous AVC extensions of SVC and MVC:

Scalable => (001, 111), Multi-view => (010, 011), Multi-view Scalable =>  (010, 111) and

so on. Note that these two fields are used to differentiate various HEVC extensions according to the nal_ID choices.

For some HEVC extensions, such as the example “Multi-view” or “Multi-view Scalable” cases, the next syntax elements (nal_bytes_2, or nal_bytes_3) containing the corresponding nal_IDs may remain “incomplete” structure-wise, i.e., some “msb” bit-locations in such syntax elements may remain empty to be filled up with “reserved” bits. Such filling up will be automatic if no more nal_IDs are present in “nal_iD_mask2”.

- nal_flags_set  Syntax Element (1 byte)

This 1-byte long syntax element consolidates all possible 1-bit flags that may be needed for HEVC extensions. The table below shows an example for the HEVC extensions about how this syntax element structure can be created by using the flag bits as present in AVC extensions of SVC and MVC.

| nal_flags_set (example) | value |
| --- | --- |
| idr_flag | 0000 0001 |
| no_inter_layer_pred_flag | 0000 0010 |
| use_ref_base_pic_flag | 0000 0100 |
| discardable_flag | 0000 1000 |
| output_flag | 0001 0000 |
| anchor_pic_flag | 0010 0000 |
| inter_view_flag | 0100 0000 |
| reserved | 1000 0000 |

- nal_bytes_2  Syntax Element (2 bytes)

This 2-bytes long syntax element contains a group of nal_IDs as indicated by the (nal_id_mask2, nal_id_mask1) fields-pair, as explained in “nal_byte_1” syntax element definition earlier.

Below is shown one example of bit allocations in “nal_bytes_2” for an HEVC Extension of “Scalable Video”.  Here the (nal_id_mask2, nal_id_mask1) fields-pair can be pre-defined or fixed as (001, 111) for this HEVC extension.

| quality_id (4-bits) | dependency_id (3-bits) | priority_id (6-bits) | temporal_id (3-bits) |
| --- | --- | --- | --- |

- nal_bytes_3  Syntax Element (3 bytes)

This 3-bytes long syntax element contains a group of nal_IDs as indicated by the (nal_id_mask2, nal_id_mask1) fields-pair, as explained in “nal_byte_1” syntax element definition before.

Below are shown three examples of bit allocations in “nal_bytes_3” for three HEVC Extension cases of “Multi-view video”, “Multi-view & Scalable Video” and “3DV extension”.  Here the (nal_id_mask2, nal_id_mask1) fields-pair can be pre-defined or fixed as shown below.

- Example-1 (Multi-view Video) : (nal_id_mask2, nal_id_mask1) = (010, 011)

| reserved (5-bits) | view_id (10-bits) | priority_id (6-bits) | temporal_id (3-bits) |
| --- | --- | --- | --- |

- Example-2 (Multi-view & Scalable Video) : (nal_id_mask2, nal_id_mask1) =  (010, 111)

| reserved (2-bits) | view_id (10-bits) | dependency_id (3-bits) | priority_id (6-bits) | temporal_id (3-bits) |
| --- | --- | --- | --- | --- |

- Example-3 (3DV Extension) with different bit-assignment for nal_IDs

- Keep nal_id_mask1 the same as before, but change nal_id_mask2 as follows:

| nal_id_mask2 | nal_IDs | Example Sizes | HEVC Base & Extensions |
| --- | --- | --- | --- |
| 001 | view_id | u(7) | Multi-view |
| 010 | texture_id | u(4) | 3DV Extension |
| 100 | depth_id | u(4) | 3DV Extension |

- (nal_id_mask2, nal_id_mask1) =  (111, 011)

| depth_id (4-bits) | texture_id (4-bits) | view_id (7-bits) | priority_id (6-bits) | temporal_id (3-bits) |
| --- | --- | --- | --- | --- |

# Conclusion

In this contribution, the HEVC NAL Unit Structure is extended to deal with future coded video contents using HEVC Extension algorithms of Scalability and Multi-view coding schemes, etc.

In this proposed syntax modification, only one bit from the unused "reserved_one_5bits" is utilized without affecting the "Base" HEVC NAL unit structure. The extension parts of this new NAL Unit structure are also capable of handling Scalable or Multi-view coding and also various combinations of them, as it has a sufficiently general-purpose structure. Examples are provided to elaborate such concepts. We believe this new structure can solve the future HEVC extensions related issues rather efficiently.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).