| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0263 WG11 Number: m24510 |
| --- | --- |

| Title: | Extension of HEVC VUI Syntax Structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document presents the extension of the “base” HEVC VUI Parameters Syntax structure to consider the future HEVC Extensions of Scalability and Multi-view coding. It also has a provision to include VUI parameters combinations for other HEVC extensions like 3DV. Essentially only one extra 1-bit flag is added to the existing HEVC VUI parameter syntax structure for HEVC VUI parameters extension.

- HEVC VUI Syntax Structure Extension

The VUI syntax structure in the current Committee Draft (CD) does not address next HEVC extensions data structures.  In our contribution, the existing VUI syntax structure is extended to include future HEVC extensions by using one additional 1-bit flag to the "base" HEVC VUI Syntax structure.

It may be noted that most of the AVC VUI syntax parameters are borrowed (JCTVC-F289) to create the HEVC VUI syntax structure in the current CD. A similar method of using SVC and MVC VUI extension parameters is used to extend the HEVC VUI syntax structure for future HEVC extensions of Scalable and Multi-view Coding.

The VUI parameter syntax elements from SVC and MVC specifications are first compared against those in HEVC VUI syntaxes, and then a combined syntax structure for HEVC VUI extension is created below. The "yellow" highlighted part in HEVC VUI Syntaxes (shown below) have commonality with those of SVC and MVC syntaxes.

## HEVC VUI Parameters Syntax Structure

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_indication_presence_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| low_delay_hrd_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

## HEVC VUI Parameters Syntax Structure Extension

| vui_parameters( ) { |  |
| --- | --- |
| .............. < omitted non-yellow top-part of HEVC VUI, as shown above>..................... |  |
| vui_flags_byte[0] | u(8) |
| if((vui_flags_byte[0] & 0x1) == 0) // no vui_extension |  |
| vui_num_entries = 1 |  |
| else // vui_extensions |  |
| vui_num_entries | ue(v) |
| for (i=0; i< vui_num_entries; i++) { // iterative loop starts |  |
| if((vui_flags_byte[i] & 0x01) > 0) { // vui_extensions |  |
| vui_ temporal_id[ i ] | u(3) |
| If (i>0) vui_flags_byte[i] | u(8) |
| if((vui_flags_byte[i] & 0x02) == 0x00) { // Scalable video only |  |
| vui_dependency_id[ i ] | u(3) |
| vui_quality_id[ i ] | u(4) |
| } |  |
| else { // Multi-view video only |  |
| vui_ num_target_output_views_minus1[ i ] | ue(v) |
| for( j = 0; j <= vui_num_target_output_views_minus1[ i ]; j++ ) |  |
| vui_view_id[ i ][ j ] | ue(v) |
| } |  |
| } // end of vui-extension |  |
| if((vui_flags_byte[i] & 0x04) > 0) { // timing_info_present_flag | u(3) |
| vui_num_units_in_tick[ i ] | u(32) |
| vui_time_scale[ i ] | u(32) |
| } |  |
| if((vui_flags_byte[i] & 0x10) > 0) // nal_hrd_parameters_present_flag |  |
| hrd_parameters( ) |  |
| if((vui_flags_byte[i] & 0x20) > 0) // vcl_hrd_parameters_present_flag |  |
| hrd_parameters( ) |  |
| } // iterative loop ends |  |
| ................... |  |
| } |  |

Semantics for VUI  parameters

Most of the VUI extension parameters above are already available from AVC extension of SVC and MVC VUI parameters, and those can be copied or slightly modified perhaps.

vui_flags_byte

This is a 1-byte long syntax element that can combine various common flags present in VUI parameter structures of HEVC and its possible extension. This is to develop a unified syntax for the parameter flags instead of multiple scattered ones for more compact representation.

Table-1 shows a comparative look for various VUI syntax parameters in HEVC VUI and SVC, MVC VUI extensions. Here different colored texts are used for ease in comparison (red texts show common 1-bit flags, green ones show other common parameters).

| Table 1: Comparison between HEVC VUI & AVC-SVC and AVC-MVC VUI-extension parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| On Syntaxes | HEVC | SVC | MVC | Comment |
| Additional syntax | x | vui_ext_num_entries_minus1 : ue(v) | vui_mvc_num_ops_minus1 : ue(v) | # of iterations |
| Iterative Loop, i | i = 0 | i = 0 to vui_ext_num_entries_minus1 | i = 0 to vui_mvc_num_ops_minus1 |  |
| New syntaxes | x | vui_ext_temporal_id[ i ] : u(3) | vui_mvc_temporal_id[ i ] : u(3) |  |
|  | x | vui_ext_dependency_id[ i ] : u(3) | vui_mvc_num_target_output_views_minus1[ i ] : ue(v) | # of next syntax view_id in MVC case |
|  | x | vui_ext_quality_id[ i ] : u(4) | vui_mvc_view_id[ i ][ j ] : ue(v) | j=0 to vui_mvc_num_target_output_views_minus1[ i ] |
| Common syntaxes | timing_info_present_flag | vui_ext_timing_info_present_flag[ i ] | vui_mvc_timing_info_present_flag[ i ] |  |
|  | num_units_in_tick | vui_ext_num_units_in_tick[ i ] | vui_mvc_num_units_in_tick[ i ] |  |
|  | time_scale | vui_ext_time_scale[ i ] | vui_mvc_time_scale[ i ] |  |
|  | fixed_frame_rate_flag | vui_ext_fixed_frame_rate_flag[ i ] | vui_mvc_fixed_frame_rate_flag[ i ] | For HEVC, frame->pic |
|  | nal_hrd_parameters_present_flag | vui_ext_nal_hrd_parameters_present_flag[ i ] | vui_mvc_nal_hrd_parameters_present_flag[ i ] |  |
|  | vcl_hrd_parameters_present_flag | vui_ext_vcl_hrd_parameters_present_flag[ i ] | vui_mvc_vcl_hrd_parameters_present_flag[ i ] |  |
|  | low_delay_hrd_flag | vui_ext_low_delay_hrd_flag[ i ] | vui_mvc_low_delay_hrd_flag[ i ] |  |
|  | field_indication_presence_flag (or, pic_struct_present_flag) | vui_ext_pic_struct_present_flag[ i ] | vui_mvc_pic_struct_present_flag[ i ] | For HEVC, different name |

As an example, Table-2 below shows how 8 bits of "vui_flags_byte" syntax element are used to indicate various common flags (six 1-bit flags for "base" and five 1-bit flags for "extension").

For "base" HEVC, only 1-bit out of these 8-bits is used to show whether VUI syntax structure contains "base" only (=0) or "Extension" (=1) VUI parameters. Another 1 bit data is made available as "reserved" in the "base" HEVC VUI parameter syntax structure.

Table-2: vui_flags_byte Syntax Element Example: bit distributions and logics for various flag-bits derivation

| vui_flags_byte[i], p=assigned bit | HEVC (Base), i=0 | HEVC (Scalability Extension), i=0 to vui_num_entries-1 | HEVC (Multi-view Extension), i=0 to vui_num_entries-1 | Conditional logics |
| --- | --- | --- | --- | --- |
| 0000 000p | vui_ext_flag (p=0) | vui_ext_flag (p=1) | vui_ext_flag (p=1) | (vui_flags_byte & 0x01) > 0 |
| 0000 00p0 | reserved | vui_ext_flag2 (p=0) | vui_ext_flag2 (p=1) | (vui_flags_byte & 0x02) > 0 |
| 0000 0p00 | timing_info_present_flag | timing_info_present_flag | timing_info_present_flag | (vui_flags_byte & 0x04) > 0 |
| 0000 p000 | fixed_pic_rate_flag | fixed_pic_rate_flag | fixed_pic_rate_flag | (vui_flags_byte & 0x0C) ==0x0C |
| 000p 0000 | nal_hrd_parameters_present_flag | nal_hrd_parameters_present_flag | nal_hrd_parameters_present_flag | (vui_flags_byte & 0x10) > 0 |
| 00p0 0000 | vcl_hrd_parameters_present _flag | vcl_hrd_parameters_present _flag | vcl_hrd_parameters_present _flag | (vui_flags_byte & 0x20) > 0 |
| 0p00 0000 | low_delay_hrd_flag | low_delay_hrd_flag | low_delay_hrd_flag | (vui_flags_byte & 0x40) > 0 |
| p000 0000 | field_indication_presence_flag (or, pic_struct_present_flag) | pic_struct_present_flag | pic_struct_present_flag | (vui_flags_byte & 0x80) > 0 |

In the Table above, the "Conditional logics" column (last) is used to derive the common flags and also HEVC VUI extension flags.

As can be seem from this last column above, the following flag is further modified for the final derivation:

fixed_pic_rate_flag = ((vui_flags_byte & 0x08) > 0) & ((vui_flags_byte & 0x04) > 0) => (vui_flags_byte & 0x0C) == 0x0C

In HEVC extensions, “vui_ext_flag2” is set to 0 for SVC and 1 for MVC.

# Conclusion

In this contribution, the HEVC VUI Syntax Structure is extended to deal with future coded video contents using HEVC Extension algorithms of Scalability and Multi-view coding schemes, etc.

In this proposed syntax modification, a single bit extra flag is used in the "Base" HEVC VUI syntax structure to differentiate it from the VUI extension; while in extension, another 1-bit to differentiate the VUI parameters of  either Scalable or  Multi-view coding.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).