| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0264r1 |
| --- | --- |

| Title: | AHG5: Harmonized diagonal scans for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal unifies the diagonal scan tables for 32x32 and 8x8 TUs in HM6.0, which results in neat and simple scanning for coefficient coding with negligible impact in coding performance.

# Introduction

In HM6.0, coefficient coding with diagonal scans is based on coefficient group (CG). The CG is scanned diagonally first and inside the coefficient group each coefficient is scanned diagonally. The CG scanning pattern reuses coefficient scanning pattern with smaller sizes as shown in Table 1. For example, 16x16 TU has 16 CGs (16x16/4x4 = 4x4). The 4x4 CG is scanned using the same pattern as 4x4 TU, e.g. (Row-3, Col-3) is the same as (Row-6, Col-5) of Table 1. However, for 32x32 TU, CG is scanned using a pattern as shown in Figure 1, which is different from the scanning pattern for each coefficient of 8x8 TU shown in Figure 2. Thus, HM6.0 can generate all coefficient scan pattern up to 32x32 by using generic blocks with size of 2x2, 4x4 and 8x8, i.e. (Row-3, Col-4), (Row-3, Col-3) and (Row-3, Col-2) of Table 1. But HM6.0 maintains these two tables with size of 8x8 for the coefficient scanning as shown in Figure 1 and Figure 2.

Table 1 Coefficient scanning for up-right diagonal scans in HM6.0

| TU Size | 32x32 | 16x16 | 8x8 | 4x4 |
| --- | --- | --- | --- | --- |
| CG scanning pattern | 8x8 | 4x4 | 2x2 | 1x1 |
| CG scanning pattern |  |  |  |  |
| Inside CG scanning | 4x4 |  |  |  |
| Inside CG scanning pattern |  |  |  |  |
| Coefficient scanning pattern |  |  |  |  |

Figure 1 CG scanning pattern of 32x32 TU. Each box is a CG with size of 4x4 pixels.

Figure 2  Coefficient scanning pattern of 8x8 TU. Each box is a pixel.

# Proposed solution

To unify the coefficient coding procedure, we propose to remove the table in Figure 1. The table in Figure 2 will be shared for 32x32 TU and 8x8 TU. The coefficient and CG scans are summarized in Table 2. This solution reduces the size of memory and numbers of branches in the software. The coefficient scanning can be generated generically using two blocks of size 2x2, and 4x4, i.e. (Row-3, Col-4) and (Row-3, Col-3) of Table 2, instead of 3 tables.

Table 2 Coefficient scanning for up-right diagonal scans of proposed solution

| TU Size | 32x32 | 16x16 | 8x8 | 4x4 |
| --- | --- | --- | --- | --- |
| CG scanning pattern | 8x8 | 4x4 | 2x2 | 1x1 |
| CG scanning pattern |  |  |  |  |
| Inside CG scanning | 4x4 |  |  |  |
| Inside CG scanning pattern |  |  |  |  |
| Coefficient scanning pattern |  |  |  |  |

# Simulation results

The proposed solution is implemented on top of HM6.0 and tested with common test conditions [1]. Summarized results are shown in Table 3 and detailed results are available in excel file “JCTVC-I0264.xls”. As shown in Table 3, the impact of proposed solution on BD-rate is negligible.

Table 3 Results of proposed solution vs HM6.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.1% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | -0.2% | 0.1% | 0.0% | -0.5% | 0.1% |
| Class E | 0.2% | -0.9% | 0.8% | 0.1% | -0.2% | 0.0% |
| Overall | 0.1% | -0.2% | 0.2% | 0.0% | -0.2% | 0.0% |
|  | 0.1% | -0.2% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class F | -0.2% | -0.1% | 0.3% | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This proposal provided unified solution for diagonal scans in coefficient coding of HM6.0. Unified solution will reduce the memory size and numbers of branches in software, while negligible coding difference is observed. It is proposed to adopt this solution into HEVC.

# Reference

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-H1100, San Jose, February 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text

There will be no change to WD text Version dK for coefficient scans related parts (Section 6.5.3, 7.3.10).

There is a ticket #372 on version dK. This proposal will remove the changes related to #372.