| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0265r1 |
| --- | --- |

| Title: | Chroma QP extension and signalling enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Alexis Tourapis 591 Yosemite Drive Milpitas, CA 95035 | Tel: Email: | +1-408-934-3766 alexis.tourapis@magnumsemi.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc., Magnum Semiconductor Inc., Sony Corp. |  |  |

_____________________________

# Abstract

In HM6.0, Chroma QP can only take values in the range of [0, 39]. This proposal extends chroma QP to vary in the range of [0, 51] using as a basis the proposal made in JCTVC-H0400. Similar to the luma QP offset signaled at the slice level, a chroma QP offset that is also signaled at the slice level is proposed. This proposal can provide better flexibility at the encoder, potentially improve rate control, and objective/subjective video quality, as well as provide for better luma/chroma operating point tradeoffs. Finally, in order to satisfy the HM6.0 common test conditions a few non-normative encoder methods are also introduced. It is shown that the application of these methods can result in almost identical RD performance as the current HM6.0. More specifically, the impact of the proposed methods is within ~0.01% in terms of BD-rate while still providing the desired flexibility.

# Problem Statement

The current Chroma QP derivation process in HEVC replicates that of the H.264/MPEG-4 AVC specification as shown in Table 1. For QP values in the range of 0 to 29 a linear relationship (QPC = QPY) is followed, whereas a non-linear relationship is followed for higher QP values. Chroma QP also saturates at a maximum value of 39 regardless of the color format that may be used. This table (g_aucChromaScale[52] in HM6.0) actually defines the relationship between Luma and Chroma at different quality levels. Assuming an 8-bit video input and based on the Cb_QP_offset and Cr_QP_offset values that are signaled in the Picture Parameter Sets (PPS) as described in [1], the chroma QP is obtained through Eqs. (1) and (2).

Table 1 H.264/AVC specification: Table 8-15 – Specification of QPC as a function of qPI

| qPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | =qPI | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

QPCb = g_aucChromaScale (Clip(0, 51, QPY+Cb_QP_offset)),                                 (1)

QPCr = g_aucChromaScale(Clip(0, 51, QPY+Cr_QP_offset)).                                 (2)

# Proposed solution

In the 8th HEVC meeting, the extension of the chroma QP range from [0, 39] to [0, 51] was proposed in JCTVC-H0400 [2]. JCTVC-H0400  removed the mapping table g_aucChromaScale[52] in HM5.0 and set the chroma QP equal to the Luma QP such that the Chroma QP can take values in the range of [0, 51] as follows:

QPCb = Clip(0, 51, QPY+Cb_QP_offset),                                         	    (3)

QPCr = Clip(0, 51, QPY+Cr_QP_offset).                                        		    (4)

There are several advantages of extending chroma QP to [0, 51]. Firstly, with the extension of chroma QP, rate control algorithms can provide better flexibility in allocating or finding the desired bit budget balancing point between luma and chroma. In the scenario of low bit-rate video applications, rate control algorithms will have the ability to quantize chroma to the desirable extent by increasing chroma QP up to 51.

Secondly, based on recent online discussions about QP prediction and rate control, the QP distribution has an impact on the interaction of QP prediction algorithms and rate control algorithms. The non-linear relationship between luma QP and chroma QP in high QP regions will have an impact on the model for bit to QP mapping employed in rate control algorithms, especially in very low bitrate scenarios. A linear relationship between luma QP and chroma QP as proposed will enhance the accuracy of these models. This may mean that such rate control algorithms would yield more stable QP values even at very low bitrate scenarios.

Thirdly, video coding standards without such limit on chroma QP can provide the functionality and flexibility for video sources with different color components or even other color formats (e.g. RGB and YCoCg).

This approach is henceforth referred as Method A.

An alternative method (Method B) is to keep the chroma mapping table but extend chroma QP to the range of [0, 51] using a modified version of Eqs. (1) and (2) as follows:

QPCb = Clip(0, 51, g_aucChromaScale (QPY)+Cb_QP_offset),                                 (5)

QPCr = Clip(0, 51, g_aucChromaScale(QPY)+Cr_QP_offset).                                 (6)

In Eqs. (5) and (6), chroma QP offsets are added to the output of the mapping table (g_aucChromaScale), while only the QP of luma is now the input of the mapping table.

The semantic changes for both methods can be found in Appendix. The comparison of HM6.0, Method A and Method B is shown in Figure 1.

- HM6.0

- Method A

- Method B

Figure 1 Comparison of HM6.0, Method A and Method B.

## Slice level chroma QP offset

To explore the control of chroma with finer granularity, similar to the luma QP offset, we propose to introduce two chroma QP offsets in the slice header. These offsets are encoded differentially with respect to chroma_qp_offset in their corresponding slice’s PPS. As mentioned earlier this approach is expected to provide more flexibility in various applications.

This approach provides a more unified rate control scheme including chroma quality control down to the slice level. For videos with different characteristics (e.g. sky vs ground/water), this scheme enables the encoder to select different chroma QP values or a different chroma/luma QP relationship for different areas and guarantee more consistent visual quality as opposed to using a single fixed chroma QP offset for the whole picture in current HM.

The semantics for slice level chroma QP offsets are presented in the following scenarios.

### Method A + slice level chroma QP offsets

When g_aucChromaScale[52] is removed from HEVC as proposed in Method A, chroma QP is derived by

QPCb = Clip(0, 51, QPY+Cb_QP_offset+slice_qp_delta_cb),    		                       (7)

QPCr = Clip(0, 51, QPY+Cr_QP_offset+slice_qp_delta_cr),                                   (8)

where Cb_QP_offset and Cr_QP_offset are the chroma QP offset parameters in the PPS and slice_qp_delta_cb and slice_qp_delta_cr are the proposed chroma QP offset parameters in the slice header.

### Method B + slice level chroma QP offsets

If only semantics of HM6.0 is modified as Method B, chroma QP is determined by

QPCb = Clip(0, 51, g_aucChromaScale (QPY)+Cb_QP_offset+slice_qp_delta_cb),                (9)

QPCr = Clip(0, 51, g_aucChromaScale(QPY)+Cr_QP_offset+slice_qp_delta_cr),                (10)

where Cb_QP_offset and Cr_QP_offset are chroma QP offset parameters in PPS and slice_qp_delta_cb and slice_qp_delta_cr are proposed chroma QP offset parameters in slice header.

The syntax and semantic changes can be found in Appendix.

## HM6.0 Common Test Conditions

In the 8th HEVC meeting, some concerns were raised about Method A in relationship to its coding performance impact to the common test conditions. We have explored some non-normative methods for achieving almost identical RD performance as that of HM6.0. In this section, a brief description is provided to explain these ideas. Detailed description for each test will be found under the experiment section.

Basically, the idea is to guarantee exactly the same QP as HM6.0 for chroma. The non-normative methods utilize the chroma_QP_offset in the PPS or/and the Slice Header. By sending multiple PPS with different chroma_QP_ offsets and PPS id, the encoder can select the chroma_QP_offset to achieve the same or approximately the same QP for chroma as HM6.0 using Eq. (3) and (4). Then the encoder will set the corresponding PPS ID in the slice header for each picture.

On the decoder side, each slice will decode the PPS ID in a slice header and select the chroma_QP_offset from the PPS with decoded PPS ID. Finally, the chroma QP will be determined by Eq. (3) and (4).

For Method B, the coding performance is exactly the same as that of HM6.0 since chroma_qp_offset are set to zeros under common test conditions.

# Experimental results

To test proposed solutions, several experiments are set up. All proposed solutions are implemented on top of HM6.0 and tested using the common test conditions [3].

## Direct performance

In this section, coding performance of Method A without non-normative methods in Section 2.3 is tested. In details, the implementation is as follows.

- Remove g_aucChromaScale[52] table.

- Replace all g_aucChromaScale[x] with x + chroma_qp_offset.

The coding performance is summarized in Table 2 and detailed results are available in “JCTVC-I0265_test1.xls”. The coding loss (more than 6%) for chroma is expected since more chroma coefficients are quantized to zeros. As a result, bitrate is decreased which provides almost 1% bit-saving for luma as shown in Table 2.

Table 2 Results summary of removing chroma QP mapping table

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | 6.1% | 6.1% | -0.8% | 5.8% | 5.9% |
| Class B | -1.2% | 6.3% | 6.1% | -1.0% | 6.3% | 6.2% |
| Class C | -1.3% | 5.9% | 6.0% | -1.2% | 5.9% | 6.2% |
| Class D | -1.2% | 5.6% | 6.0% | -1.1% | 5.6% | 6.1% |
| Class E | -1.0% | 6.1% | 6.7% | -1.0% | 6.1% | 6.4% |
| Overall | -1.1% | 6.0% | 6.2% | -1.0% | 6.0% | 6.1% |
|  | -1.1% | 6.6% | 6.8% | -1.0% | 6.5% | 6.9% |
| Class F | -1.1% | 3.9% | 4.2% | -0.9% | 4.0% | 4.2% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | 12.1% | 11.8% | -0.5% | 11.8% | 12.0% |
| Class B | -1.2% | 10.2% | 9.6% | -1.1% | 10.3% | 9.2% |
| Class C | -1.3% | 8.9% | 8.8% | -1.2% | 8.8% | 8.9% |
| Class D | -1.0% | 8.5% | 8.9% | -1.0% | 8.1% | 8.6% |
| Class E |  |  |  |  |  |  |
| Overall | -1.0% | 9.9% | 9.8% | -0.9% | 9.8% | 9.7% |
|  | -1.0% | 10.2% | 10.1% | -1.0% | 10.0% | 10.0% |
| Class F | -1.1% | 6.1% | 6.5% | -1.1% | 6.2% | 6.5% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | 9.6% | 9.1% | -0.9% | 9.7% | 9.1% |
| Class C | -1.0% | 8.9% | 9.3% | -1.0% | 8.5% | 9.2% |
| Class D | -0.8% | 9.0% | 9.7% | -0.8% | 8.9% | 9.1% |
| Class E | -0.5% | 7.6% | 9.0% | -0.4% | 8.5% | 9.5% |
| Overall | -0.8% | 8.9% | 9.3% | -0.8% | 9.0% | 9.2% |
|  | -0.8% | 9.4% | 9.9% | -0.8% | 9.4% | 9.8% |
| Class F | -1.2% | 5.7% | 5.9% | -0.9% | 5.0% | 8.1% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

## HM emulation

In this section, experimental results are presented to demonstrate how the coding performance of HM6.0 can be emulated by the non-normative methods in Section 2.2. Only experimental results for Method A are provided. As stated in Section 2.2, Method B will have exactly the same coding performance as HM6.0 does under common test conditions.

### Chroma QP extension

The non-normative method is applied combined with Method A on top of HM6.0. The implementation is elaborated as follows:

- Remove g_aucChromaScale[52] table.

- Replace all g_aucChromaScale[x] with x+chroma_qp_offset.

- Send multiple PPS to cover all possible QPs which have g_aucChromaScale[x] != x by using nonzero chroma_qp_offset = g_aucChromaScale[x]- x.

- Set PPSID in a slice header to ensure the slice will have correct chroma_qp_offset.

The algorithm described above is implemented based on HM6.0. Simulation is done with the common test conditions. The results are summarised in Table 3. Detailed results are available in the excel file named as “JCTVC-I0265_test2.xls”. Results show that the simulation achieves exactly the same PSNR as HM6.0 does for each picture in all test video sequences. Although more than one PPS are sent in the bitstream compared to only one PPS in HM6.0 bitstream, the overhead is negligible (less than 0.01%) as shown in Table 3.

Table 3 Results for HM6.0 emulation of Method A

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 0.01% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

### Slice level chroma QP offsets

The slice level chroma QP offsets syntax is implemented in HM6.0 and tested under the common test conditions while trying to emulate the coding performance of HM6.0. Here are the details about implementation:

- Remove g_aucChromaScale[52] table.

- Replace all g_aucChromaScale[x] with x+chroma_qp_offset + slice_qp_delta_cb/cr.

- Send multiple PPS to cover all possible QPs which have g_aucChromaScale[x] != x by using nonzero chroma_qp_offset = g_aucChromaScale[x]- x.

- Set PPSID in slice header to ensure the slice will have correct chroma_qp_offset.

- Set slice_qp_delta_cb and slice_qp_delta_cr to zeros.

Summarized results are shown in Table 4 and detailed results are available in “JCTVC-I0265_test3.xls”. The overhead for slice level chroma QP offset is negligible for videos with large resolution (e.g. Class A, B, C and F).

Table 4 Results of slice level chroma QP offsets

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusion

This proposal extended chroma QP to the range of [0, 51] and introduced slice level chroma QP offsets, which simplified the design and enhanced the functionality of the standard. Non-normative methods are proposed to achieve almost exactly coding performance of HM6.0, which guarantee that the proposed solutions will not impact other tools in the current HM. It is proposed to adopt this proposal into HEVC.

# Reference

- S. Liu and K. Sato, “Support of ChromaQPOffset in HEVC”, JCTVC-G509, Geneva, November 2011.

- J. Xu, A. Tourapis, E. Manni and K. Sato, “Chroma QP extension”, JCTVC-H0400, San Jose, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-H1100, San Jose, February 2011.

# Patent rights declaration(s)

Sony Electronics Inc. and Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Magnum Semiconductor Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# Appendix

All the changes can be found as highlighted text in yellow as follows.

## Syntax changes

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ..... |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if (chroma_format_idc !=0) { |  |
| slice_qp_delta_cb | se(v) |
| slice_qp_delta_cr | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| ...... |  |

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, idr_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding treeblock in the picture.

......

slice_qp_delta specifies the initial value of QPY to be used for all the macroblocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantization parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-1)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

slice_qp_delta_cb, slice_qp_delta_cr are specifying the offset to cb_qp_offset, cr_qp_offset used for deriving QP’Cb and QP’Cr , respectively.

inherit_dbl_params_from_aps_flag equal to 1 specifies that deblocking parameters from the APS shall be used for deblocking the current slice. inherit_dbl_params_from_aps_flag equal to 0 specifies that deblocking parameters in the slice header shall be used. When inherit_dbl_params_from_aps_flag is not present, it is inferred to be equal to 0.

disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall be disabled and disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be enabled.

……

### Derivation process for quantization parameters

Inputs of this process are:

–	luma location ( xB, yB ) specifying the top-left luma sample of the current coding unit quantization group relative to the top-left luma sample of the current picture,

Outputs of this process are:

–	luma quantization parameter QP’Y,

–	chroma quantization parameters QP’Cb and QP’Cr

Let a variable QPY_A specifying the luma quantization parameter of the coding unit quantization group covering ( xB−1,  yB ), a variable tbAddrA specifying the treeblock address of treeblock containing the coding unit quantization group covering ( xB−1,  yB ), a variable QPY_B specifying the luma quantization parameter of the coding unit quantization group covering ( xB,  yB−1 ), a variable , tbAddrB specifying the treeblock address of treeblock containing the coding unit quantization covering ( xB,  yB−1 ), and a variable QPY_PREV specifying the luma quantization parameter of the previous coding unit quantization group in decoding order, respectively.

When at least one of the following conditions is true, QPY_PREV is initially set equal to SliceQPY at the start of each slice for the first coding unit quantization group in the slice.

–	A quantization group of coding units is the first quantization group of coding units in a slice.

–	A quantization group of coding units is the first quantization group of coding units in a tile.

–	A quantization group of coding units is the first quantization group of coding units in a treeblock row and tiles_or_entry_coding_sync_idc is equal to 2.

The prediction of luma quantization parameter QPY_PRED is derived as the following ordered steps:

- If the coding unit quantization group covering ( xB−1,  yB ) in the current slice is not available or tbAddrA is not equal to CurrTbAddr, then QPY_A is set equal to QPY_PREV.

- If the coding unit quantization group covering ( xB,  yB−1 ) in the current slice is not available or tbAddrB is not equal to CurrTbAddr, then QPY_B is set equal to QPY_PREV.

- The prediction of luma quantization paramaeter QPY_PRED is derived as:

QPY_PRED =  (QPY_A + QPY_B + 1) >> 1		(8-2)

The value of QPY is derived as

QPY = ( ( ( QPY_PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY	(8-3)

The value of luma quantization parameter QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

The values of chroma quantization parameters QP’Cb and QP’Cr are derived as

Method A:

QP’Cb = Clip3( −QpBdOffsetC, 51, QPY + cb_qp_offset + slice_qp_delta_cb) + QpBdOffsetC	(8-5)

QP’Cr = Clip3( −QpBdOffsetC, 51, QPY + cr_qp_offset + slice_qp_delta_cr) + QpBdOffsetC	(8-6)

Method B:

QP’Cb = Clip3( −QpBdOffsetC, 51, Y2C(QPY) + cb_qp_offset + slice_qp_delta_cb) + QpBdOffsetC	(8-7)

QP’Cr = Clip3( −QpBdOffsetC, 51, Y2C(QPY)+ cr_qp_offset + slice_qp_delta_cr) + QpBdOffsetC	(8-8)