| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0266 |
| --- | --- |

| Title: | Modifications on signaling collocated picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC committee draft, collocated_from_l0_flag and collocated_ref_idx at slice header are used to specify the collocated picture. Another flag enable_temporal_mvp_flag at PPS is used to control whether temporal motion vector is used or not. This document proposes some modifications on signaling collocated picture. When enable_temporal_mvp_flag is 0, both collocated_from_l0_flag and collocated_ref_idx are no longer necessary.

# Introduction

In the current HEVC committee draft (CD) [1], at slice header, collocated_from_l0_flag is used to specify if the collocated picture is in either reference picture list 0 or reference picture list 1, and collocated_ref_idx is further used to explicitly index the collocated picture in the reference picture list specified by collocated_from_l0_flag.

In CD, there is another flag, enable_temporal_mvp_flag, at PPS to control whether temporal motion vector is used or not. More specifically, if enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. If the temporal motion vector predictors are not used, collocated_from_l0_flag and collocated_ref_idx are no longer necessary.

# Modifications on signaling collocated picture

The slice header can be modified as shown in Table 1.

Table 1 Modified slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B && enable_temporal_mvp_flag) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && enable_temporal_mvp_flag && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

# References

[1] HEVC committee draft, JCTVC-H1003, Feb., 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).