| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0272 |
| --- | --- |

| Title: | 4:4:4 Screen Content Coding using Dual-coder Mixed Chroma-sampling-rate (DMC) Techniques |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Lin Peijun Zhang Shuhui Wang Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution presents a Dual-coder Mixed Chroma-sampling-rate (DMC) technique for full-chroma (YUV444) screen content coding. The proposed DMC coding adds a full-chroma dictionary-entropy coder to the existing chroma-subsampled (YUV420) HEVC coder. The existing YUV420 HEVC syntax and semantics can be used AS IS. Three new syntax elements (distance, length, literal) are added to enable YUV444 dictionary-entropy coding.

The main point of DMC coding is to enable full-chroma YUV444 content coding in HEVC, which could expand their market adoption especially in cloud-mobile computing and wireless display areas, with minimum addition, modification, and complexity increment to the current YUV420 HEVC coder design.

Coding experimental results are that DMC coding can achieve much higher PSNR than the current HEVC. There are also obvious subjective visual quality difference between DMC coding and the current HEVC.

# Introduction

In the new generation cloud-mobile computing architecture, Screen Content Coding (SCC) is a key technology to solve the cloud computing bottleneck problem to transfer huge amount of data between the computing units (cloud) and the users (including mobile ones). Examples of SCC based new generation cloud-mobile computing include Virtualized Screen [1], Cloudlet-screen Computing [2], Remote Desktop [3], Virtual Desktop Infrastructure [4], Zero Client [5], PCoIP [6], and quite a few commercialized products [3]-[9]. Besides playing a critical role in cloud-mobile computing, SCC is also an indispensable technology for a number of other rapidly growing application areas such as wireless or wired external, second, or mirror display for iPad/iPhone like mobile devices at office or home [10]-[12].

Driven by these applications, a few SCC related ad hoc groups have been established to progress SCC technology and standardization. In the most recent three JCTVC meetings [13]-[17] alone, there are more than three dozen SCC related contributions proposed.

Screen content can be classified into two essentially different types: mostly computer-generated discontinuous-tone content such as text, chart, graphics, and icon, and mostly camera-captured continuous-tone content such as photograph, movie/TV clips, and natural picture video sequences. The discontinuous-tone content features very sharp edges, uncomplicated shapes, and thin lines with few colors, even one-pixel-wide single-color lines while continuous-tone content features relatively smooth edges, complicated textures, and thick lines with virtually unlimited colors. Therefore, the most effective way for SCC is to use two types of coders together: one, e.g. dictionary-entropy coder for discontinuous-tone content and the other, e.g. traditional hybrid coder for continuous-tone content.

It is well known that for continuous-tone content, chroma-subsampling is almost visually lossless, hence all existing video coding standards such as MPEG, JPEG, JPEG-2000, H.264/AVC, VC-1, and AVS are designed primarily to code chroma-subsampled contents. Actually, a majority of CODEC products on the market today can handle only YUV420 or YUV422 format video. On the other hand, chroma-subsampling significantly degrades the visual quality of discontinuous-tone content as shown in Figure 1. We can see that converting a full-chroma YUV444 picture into a chroma-subsampled YUV420 picture generates severe color artifact such as the black pixels in the YUV420 picture. The artifact is more obvious after scaling-up, which is applied to screen content from time to time in cloud computing display and wireless display applications.

Full-chroma YUV444 picture

Chroma-subsampled YUV420 picture

(a) Chroma-subsampling causes severe color artifact such as black pixels in the YUV420 picture.

(b) Artifact is more obvious after scaling-up (200% in the example) of the screen content.

- YUV444 vs. YUV420: chroma-subsampling causes severe color artifact.

For SCC, although the visual quality of chroma-subsampled continuous-tone content oriented coders is not good, almost all applications today still uses them primarily due to two reasons: 1) the majority of video CODEC software, hardware, and human assets developed in the past 25 years are chroma-subsampled ones and completely switching to full-chroma coders (at least two coders, one for continuous-tone content and the other for discontinuous-tone content) not only significantly increase the unit cost but also requires tremendous new investment; 2) since quite some portions of SCC are continuous-tone content, coding the entire image using two full-chroma coders is an overkill and way too expensive.

Hence, the optimal solution to achieve best 3-way balance between bitrate, coding quality and coding complexity for SCC is to add a new full-chroma coder to a traditional chroma-subsampled coder, and to mix full-chroma YUV444 coding with chroma-subsampled YUV420 coding at some properly selected boundaries in an adaptive way or in a pre-defined way.

This contribution proposes a Dual-coder Mixed Chroma-sampling-rate (DMC) coding technique for SCC that combines one chroma sampling-rate coding with another chroma sampling-rate coding at certain boundaries during the coding process. One aspect of the technique is to mix two chroma sampling-rate coders at CU boundary in an adaptive way. Another aspect of the technique is to mix two chroma sampling-rates at the coding chain component tool (such as prediction, transform, quantization, entropy coding etc.) boundary. The two aspects are complementary and are used together.

# A Partial-lossless DMC Coding System

The proposed DMC coding system uses a lossless full-chroma YUV444 dictionary-entropy coder such as gzip or lzma as the base full-chroma coder (Coder 1 in Figure 2) and the current YUV420 HEVC as the base chroma-subsampled coder (Coder 2 in Figure 2). YUV444 Constructed Pixel Buffer (CPB) in Figure 2 is the dictionary for string searching and matching of Coder 1, and is also used as the neighboring pixels for intra prediction and the reference pictures for inter prediction of Coder 2. Since Coder 1 is lossless and Coder 2 is lossy, the resulting DMC coding system is a partial-lossless (defined as partially lossless and partially lossy) coding system.

Coder 2: chroma-subsampled hybrid encoding tool chain

Decoder 1: full-chroma dictionary-entropy decoding tool chain

Coder 1: full-chroma dictionary-entropy encoding tool chain

Decoder 2: chroma-subsampled hybrid decoding tool chain

Pred

Entropy Encode

Tran

Qu

Input CU

R-D Based Selector

DMC Bitstream

Constru-cted CU

CU Coding Type Parser

DMC Bitstream

UV

Pred

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

UV

(a) Encoder

(b) Decoder

The existing HEVC YUV420 coding tools are used AS IS

= the only new additions

The existing HEVC YUV420 coding tools are used AS IS

= the only new additions

Entropy encoding

String search & matching

Entropy decoding

String construction

Pixel reorder

Pixel reorder

= Upgrade but no change to existing syntax and semantics

YUV444 Constructed Pixel Buffer for Reference, Prediction, Dictionary

shared

Scan

Construct to YUV444

shared

YUV444 Constructed Pixel Buffer for Reference, Prediction, Dictionary

- CU-adaptive DMC coding architecture.

This DMC coding system is actually upgraded from the current YUV420 HEVC hybrid coder with the only new additions in the encoder being a chroma-subsampler, a pixel reorder, and a string search & matching tool similar to that used in gzip or lzma, which is very effective for discontinuous-tone content coding as reported in [18]-[22]. Correspondingly, the only new additions in the decoder are a chroma-upsampler, a pixel reorder, and a string construction tool. In the hybrid coder, the YUV420 prediction (both intra and inter) is upgraded to YUV444 prediction. Only the calculation formula of prediction is extended from handling YUV420 data to handling YUV444 data, i.e. the amount of chroma data quadruples in the prediction calculation. No change to the existing YUV420 hybrid coding syntax and semantics including prediction modes and motion vector formats is needed. All other YUV420 hybrid coding tools are not changed and can be used AS IS

The DMC coding system is not simply putting two base coders together. The two base coders need to be properly enhanced and seamlessly unified to make the overall coding system really effective. The following is a few key points to enhance and unify the two coders.

  - Dictionary in DMC Coding

In a dictionary-entropy coder, the maximum dictionary size ranges from 32KB (in gzip) to 4GB (in LZMA), which is far more than the data size of one CU. Obviously, if only the current LCU data of 4096x3=12K are used in the dictionary, the compression performance will suffer seriously. Therefore, in the proposed DMC coding system, pixels data in previously constructed CUs are also used in the dictionary. The constructed pixels from the CUs previously coded by the lossless dictionary-entropy coder are all used in the dictionary. The constructed pixels from the CUs previously coded by the lossy hybrid coder can also optionally be used in the dictionary.

  - Pixel Format and Scan Order in CU and Dictionary

In hybrid coding, pixels are organized in planar format as luma blocks and chroma blocks, and the scan order is zig-zag. It turns out that this pixel format and scan order are not suitable for lossless dictionary-entropy coder in DMC coding. In fact, if the input CU to lossless coders use packed pixel format (R0, G0, B0, R1, G1, B1, …… or Y0, U0, V0, Y1, U1, V1, ……, where Rn, Gn, Bn or Yn, Un, Vn are the three components of a pixel) and column-wise scanning as shown in Figure 3, the compression performance can be significantly improved. Therefore, the pixel format and scan order of an input CU to the lossless coder is different from that to the lossy hybrid coder. For the lossless coder, both the input CU and the dictionary use packed pixel format and column-wise scan order. The pixel format reorder is done by the pixel reorder unit in Figure 2. In most cases, the pixels are stored in a memory space and the pixel reorder unit can be implemented as a memory address translator.

left

neighboring

CU

current

CU

- Column-wise scanning of packed pixel in CUs and dictionary.

  - Seamless Integration and Enhancement to Hybrid Coding

In HEVC hybrid coder, when coding one CU, some coding information such as the pixel sample values and the intra or inter prediction mode of its neighboring CU is needed. In DMC coding, if a neighboring CU is coded using the full-chroma non-hybrid coder, then the neighboring information may be unavailable or incompatible to the hybrid coder. In these cases, special treatments and various conversions may become necessary for seamlessly unifying two different types of coders. Furthermore, as discussed before, enhancement to traditional hybrid coding such as extending prediction operation from YUV420 data to YUV444 data is necessary to take full advantage of hybrid coding. From HEVC hybrid coding point of view, if a neighboring CU is coded using the lossless coder, then the neighboring CU can be treated as an I_PCM CU and the coding process proceeds following the corresponding specification given in the current HEVC hybrid coding.

  - CU Layer Syntax of DMC Coding

DMC bitstream is a mix of full-chroma dictionary-entropy coder bitstream segment and chroma-subsampled hybrid coder bitstream segment. For the DMC decoder to parse the coder type (dictionary-entropy coder or hybrid coder) properly, in the coding unit layer, a flag is added to indicate the particular coder of each CU. The CU layer syntax of DMC described by conditioning statement pseudo code is shown in Figure 4, where chroma_format_idc or profile_idc is defined in SPS.

| coding_unit() { | Descriptor |
| --- | --- |
| if(chroma_format_idc!=DMC) //or if(profile_idc!=DMC) |  |
| {the current YUV420 HEVC hybrid coder syntax is used AS IS} |  |
| else { |  |
| coder_type_flag | ae(v) |
| if(coder_type_flag = = 0) //HYBRID_CODER |  |
| {the current YUV420 HEVC hybrid coder syntax is used AS IS} |  |
| else |  |
| dictionary_entropy_coder() |  |
| } |  |
| } |  |

- CU layer syntax in DMC coding system.

The dictionary-entropy coding is not trivial, but it is simpler than hybrid coding. The coded bitstream basically consists of only three types of syntax elements: literals (unmatched luma or chroma samples), distances (between the first sample of current sample-string to be decoded/constructed and the first sample of matching sample-string previously decoded/constructed in the dictionary, which is also called search buffer and is actually CPB in Figure 2), and length (of the matching sample-string). The syntax elements are entropy-coded by cabac. The dictionary_entropy_coder() syntax described by conditioning statement pseudo code is shown in Figure 5. NumSamplesInCU is the total number of Y samples, U samples, and V samples in a coding unit (e.g. 16x16x3=768 for a coding unit of size 16x16 pixels).

| dictionary_entropy_coder { | Descriptor |
| --- | --- |
| decoded_sample_count = 0 |  |
| while(decoded_sample_count < NumSamplesInCU) { |  |
| matching_string_flag | ae(v) |
| if(matching_string_flag = = 1) { |  |
| distance | ae(v) |
| length | ae(v) |
| decoded_sample_count += length |  |
| } |  |
| else { |  |
| literal | ae(v) |
| decoded_sample_count += 1 |  |
| } |  |
| } |  |
| } |  |

- Syntax of dictionary_entropy_coder.

  - Fast String Search and Matching in Dictionary Encoder

The basic operation of a dictionary encoder is to search in previously coded sample buffer (search buffer, also called dictionary) to find a sample-string that matches the current sample-string to be coded. If a good matching string is found, then the encoder outputs the position offset (distance) of the two strings and the length of the matching string. If no match is found or the length is too short (typically less than 3), then the encoder outputs the single current sample (literal) to be coded and move to the next sample. Obviously, an efficient dictionary encoder needs to have a large enough dictionary and find a matching string as long as possible in a time as short as possible. The most popular dictionary encoder architecture is shown in Figure 6. The dictionary typically has a size from 32Kbytes to a few hundred Mbytes. The hash table typically has a size of 16K to 64M entries.

Input Sample

Input Sample Buffer

Hash Function

2-4 Samples

(16-32 bits)

Hash Index

(14-26 bits)

Hash Table: Linked Lists of Pointers

Dictionary (=Search Buffer = Constructed Pixel Buffer)

Search Engine

to Find Best Matching String

Pointers to Position of First 2-4 Matching Samples

Output: Literal if no good match or Distance/Length of the best matching string

- Dictionary encoder architecture.

# Experimental Results

- DMC Intra Coding

In the experiments, the six screen capture pictures of [22] are used and are shown in Figure 7 (a) – (f). They are all captured in raw full-chroma 24-bit RGB format and then converted to 24-bit YUV444 format before coding. For YUV420 coding, they are further chroma-subsampled. PSNR is always calculated in YUV444 format. YUV420 format constructed pictures are first chroma-upsampled for PSNR calculation. For chroma-subsampling, 2x2 average filtering is used. Pixel duplication is used for chroma-upsampling.

The base hybrid coder used in the HEVC DMC experiments is HM4.0. The string search/matching function extracted from lzma source code is modified and unified into HM4.0 with the various enhancement described in section 2. The basic cabac function used in HM4.0 is also used to code the literal/distance/length syntax elements in the base dictionary-entropy coder. The string search/matching function uses a dictionary of 24Mbytes and a hash table of 40M entries. In the base HEVC hybrid coder, cabac option is used. The following three coding methods are used in the experiments:

- The current state of HEVC. This method is designated as HEVC 420;

- Upgrading 4:2:0 prediction of 1) to 4:4:4 prediction without any change to 4:2:0 syntax and semantics. This method is designated as HEVC 420 Pred444;

- The lzma-like string search and matching function is unified into 2) and the cabac function of 2) is also used in the dictionary-entropy coder, as shown in Figure 2. This method is designated as HEVC MCS Pred444.

R-D curve charts are shown in Figure 8. Each chart has five R-D curves: HEVC 420, HEVC 420 Pred444, HEVC MCS Pred444, H264 420, and H264 MCS Pred444. Two curves from H.264 based experiments [22] are included in the charts to compare the R-D performance between H.264 based DMC coding and HEVC DMC coding. It should be noted that in the experiments, HEVC DMC coding uses much larger dictionary and hash table than H.264 based DMC coding uses (a dictionary of 32KB and a hash table of 64K entries).

(b) IC design debug waveform plus nature picture.

(a) Integrated-circuit design debug waveform.

(c) Web page.

(d) PowerPoint, Word, and Excel file windows.

(e) Document editing.

(f) Various windows mix.

- Six screen capture pictures, (a), (b), (c): 704x448, (d): 1920x1200, (e): 1280x800, (f): 1280x1024.

- HEVC R-D curves of the six pictures in [22].

From these R-D curve charts and data, we have the following findings:

- DMC coding can achieve significantly higher PSNR than YUV420 hybrid coding;

- HEVC 420 Pred444 curves are almost the same as HEVC 420 curves, so using full chroma prediction (Pred444) alone in HEVC YUV420 coding does not help. However, using full chroma prediction in combination with dictionary-entropy coding can get better R-D performance than using full chroma prediction only.

- For screen picture without a lot of discontinuous-tone content such as picture (f) of Figure 7, the improvement of DMC coding over traditional hybrid coding is not very significant. As shown in Figure 9, picture (f) of Figure 7 contains a number of regions that have complicated background textures and/or background noise probably due to that the picture had been compressed at least once before. How to code screen content having complicated background textures and/or previous encoding noise is a topic requiring further work. In general, complicated background textures and repetitive (multi-generation) compression-decompression should be avoided in screen content and SCC.

Magnifying by 1000

Magnifying by 1000

Complicated background textures and/or previous encoding noise

- Screen captur pictures having complicated background textures and/or previous encoding noise.

Moreover, these R-D curve charts show that HEVC DMC coding can achieve the same PSNR as H.264 based DMC coding at a much lower bitrate than H.264 based DMC coding. For examples, at the same PSNR around 45dB, we see from Figure 8(a)-(f) that the bitrate reduction between HEVC DMC coding and H.264 based DMC coding is about 29%, 18%, 26%, 39%, 48%, 37%, respectively. Although not shown here, we also see the same subjective visual quality improvement as in the H.264 based experiments.

- DMC Inter Coding

The DMC inter coding experimental results are presented in JCTVC-I0272-v1.ppt

- Subjective Quality Comparison

The subjective visual quality improvement of HEVC DMC coding over the existing HEVC YUV420 hybrid coding is reported in JCTVC-I0336 and may be available in viewing room.

# Conclusions and Future Work

In this document, we have proposed DMC coding for full-chroma screen content. With minimum addition to the current state of YUV420 HEVC and three additional syntax elements, the coding performance for full-chroma (YUV444) screen contents is significantly improved. The current DMC implementation is based on HM4.0 and intra picture coding only. Future work includes integrating dictionary-entropy coding into future version of HM such as HM7.0 in a HM-friendly way and at the same time keeping highest possible efficiency, advantages, and advanced features of both base coders.

# References

- Yan Lu, Shipeng Li and Huifeng Shen, “Virtualized Screen: A Third Element for Cloud_Mobile Convergence,” IEEE Multimedia, Vol. 18, no. 2, pp. 4-11, Apr. 2011.

- Tao Lin and Shuhui Wang. “Cloudlet-Screen Computing: A Multi-core-based, Cloud-computing-oriented, Traditional-computing-compatible Parallel Computing Paradigm for the Masses,” IEEE Workshop on Multimedia Signal Processing and Novel Parallel Computing, pp. 1805-1808, Jul. 2009.

- Microsoft, Remote Desktop Protocol [Online]. Available: http://msdn.microsoft.com/en-us/library/aa383015.aspx (Accessed 15 March 2012).

- VMWARE. Virtual Desktop Infrastructure [Online]. Available: http://www.vmware.com/pdf/ virtual_desktop_infrastructure_wp.pdf (Accessed 15 March 2012).

- DISPLAYLINK. Zero Client – The Affordable Windows Experience [Online]. Available: http://www.displaylink.com/zero_client/wms.php (Accessed 15 March 2012).

- TERADICI. PCoIP technology explained [Online]. Available: http://www.teradici.com/pcoip/pcoip-technology.php (Accessed 15 March 2012).

- Walter S. Mossberg, “The iPad Now Can Take Command of Computers,” The Wall Street Journal, January 20, 2011

- Damon Poeter, “Intel Adds Splashtop Device-Bridging Tech to Sandy Bridge Boards,” PC Magazine, Dec. 9, 2011,  http://www.pcmag.com/article2/0,2817,2397481,00.asp.

- George Wong, “Splashtop Remote Desktop HD optimized for Tegra 3,” ubergizmo, Dec. 19, 2011, http://www.ubergizmo.com/2011/12/splashtop-remote-desktop-hd-optimized-tegra-3.

- R. Krishnamoorthy, “High definition, anywhere: How ultra wideband makes wireless HDMI possible”, 2007 4th Annual IEEE Consumer Communications and Networking Conf., pp. 395-399

- W&W Comm., “Wireless HDMI-The Next Frontier in HD Home Entertainment,” ww.wwcoms.com

- WHDI™ Technology-Overview, www.amimon.com/technology.shtml

- Wen Gao, Keiichi Chono, Jizheng Xu, Minhua Zhou, and Pankaj Topiwala,“ JCT-VC AHG report: Lossless Coding,” JCT-VC Doc JCTVC-H0019, Feb. 2012

- David. Flynn, Pankaj Topiwala, Dzung Hoang, and Ken McCann, “JCT-VC AHG report: Chroma format support,” JCT-VC Doc JCTVC-H0020, Feb. 2012

- Oscar C. Au, Jizheng Xu, and Haoping Yu, “JCT-VC AHG report: Screen Content Coding (SCC),” JCT-VC Doc JCTVC-G013, Nov. 2011.

- Wen Gao, Keiichi Chono, Jizheng Xu, and Minhua Zhou, “JCT-VC AHG report: Lossless Coding,” JCT-VC Doc JCTVC-G022, Nov. 2011.

- Oscar C. Au, Jizheng Xu, and Haoping Yu, “JCT-VC AHG report: Screen Content Coding (SCC),” JCT-VC Doc JCTVC-F013, July 2011.

- C. Lan, X. Peng, J. Xu and F. Wu, “Intra and inter coding tools for screen contents,” JCT-VC Doc JCTVC-E145, Mar. 2011.

- Shuhui Wang and Tao Lin, “A Unified LZ and Hybrid Coding for Compound Image Partial-Lossless Compression,” in proc. IEEE int. conf. Image and Signal Processing, pp. 1-5, Oct. 2009.

- Shuhui Wang and Tao Lin, “United Coding for Compound Image Compression,” in proc. IEEE int. conf. Image and Signal Processing, pp. 566-570, Oct. 2010.

- Tao Lin and Shuhui Wang, “Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment,” JCT-VC Doc JCTVC-H0065, Feb. 2012.

- Shuhui Wang and Tao Lin, “4:4:4 screen content coding using Macroblock-Adaptive Mixed Chroma-Sampling-Rate,” JCT-VC Doc JCTVC-H0073, Feb. 2012.

- J. Cao, K. Zhou, S. Wang, T. Lin, “YUV4:4:4 test sequences for screen content coding,” JCT-VC Doc JCTVC-H0294, Feb. 2012.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).