| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0273 |
| --- | --- |

| Title: | A proposal to remove redundancy in the context selection of significant_coeff_flag. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports that WD6 has redundancy in the derivation of sigCtx for the syntax element significant_coeff_flag. This contribution proposes a modification in the derivation process in WD6 to remove the redundancy and to reduce computation in the HM6 software with exactly the same coding efficiency. A formal proof is provided in the appendix. Experiment showed that the BDR for AI_Main and AI_HE10 is 0.000000%.

# Introduction

For transform block larger than 8x8, the context index increment of significant_coeff_flag is computed with a template. As shown in Figure 1, the template consists of 5 points {I, H, F, E, B}. Conceptually in WD6 [1] and HM5, the computation of the context index increment can be separated into three stages.

- In the first stage, a cnt is first computed from the four positions {I, H, F, E} within the current picture. In this stage, cnt has a range of [0,4].

- In the second stage, if the cnt resulted from the first stage is less than 4, than the significance of the position B is also added to cnt.

- In the last stage, the final value of the context increment is ( (cnt + 1) >> 1 ).

Figure 1: Template for the selection of signifcant map context.

# A redundancy in WD6 and HM6

This contribution presents a redundancy in the derivation process of the context increment in the second stage. The redundancy is best explained by the following example.

For example, after stage 1, the cnt has a value of 3. Since cnt is less than 4, stage 2 requires the significance of the position B to be added to cnt. The output of the stage 2 has either a value of 3 or a value of 4. In either case, the context increment resulted from the last stage is 2. In other words, the calculation of the significance of the position B is redundant and it has no influence on the outcome of the context increment.

# A proposal to streamline WD6 and HM6

To streamline WD6 and HM6, this contribution proposes to replace the value 4 in stage 2 by a value of 3 as in the following:

- In the second stage, if the cnt resulted from the first stage is less than 3, than the significance of the position B is also added to cnt.

# Propose WD text

In 9.2.3.1.4  of JCTVC-H1003_dk, replace the following text

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-1)

by

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 3, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-2)

# Results

A formal proof that the propose WD text results in exactly the same results as the WD6 [1] is in the appendix. To further demonstration that the results are the same, Table xxx shows that the BDR of the propose changes is 0.000000% for All Intra Main and All Intra HE10.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class B | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class C | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class D | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class E | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Overall | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
|  | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class F | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 1: BDR resulted from the proposed changes in WD6.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK,  8th Meeting: San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

This is a formal proof that the propose WD text results in the same sigCtx as the HM6 [1].

In 9.2.3.1.4 HM6 JCTVC-H1003_dk, we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-4)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-5)

Error: Reference source not found and Table 3: Results from WD6 text when yC  is not less than (( 1 << log2TrafoHeight ) – 2). show the results of sigCtx at equation (9-26) and the corresponding results of ( (sigCtx + 1) >> 1 ) in equation (9-27) and (9-28) when yC is less than and not less than ( 1 << log2TrafoHeight ) – 2 respectively.

| sigCtx just before (9-26) | significant_coeff_flag[ xC ][ yC + 2 ] if input to (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 2 | 1 |
| 2 | 0 | 2 | 1 |
| 2 | 1 | 3 | 2 |
| 3 | 0 | 3 | 2 |
| 3 | 1 | 4 | 2 |
| 4 | 0 | 4 | 2 |
| 4 | 1 | 4 | 2 |

Table 2: Results from WD6 text when yC is less than (( 1 << log2TrafoHeight ) – 2).

| sigCtx just before (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 1 |
| 3 | 3 | 2 |
| 4 | 4 | 2 |

Table 3: Results from WD6 text when yC  is not less than (( 1 << log2TrafoHeight ) – 2).

In the propose WD text, we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 3, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-6)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-7)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-8)

Table 4 and Table 5 show the results of sigCtx at equation (9-26) and the corresponding results of ( (sigCtx + 1) >> 1 ) in equation (9-27) and (9-28) when yC is less than and not less than ( 1 << log2TrafoHeight ) – 2 respectively.

| sigCtx just before (9-26) | significant_coeff_flag[ xC ][ yC + 2 ] if input to (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 2 | 1 |
| 2 | 0 | 2 | 1 |
| 2 | 1 | 3 | 2 |
| 3 | 0 | 3 | 2 |
| 3 | 1 | 3 | 2 |
| 4 | 0 | 4 | 2 |
| 4 | 1 | 4 | 2 |

Table 4: Results from proposed WD text when yC is less than (( 1 << log2TrafoHeight ) – 2).

| sigCtx just before (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 1 |
| 3 | 3 | 2 |
| 4 | 4 | 2 |

Table 5: Results from proposed WD text when yC is not less than (( 1 << log2TrafoHeight ) – 2).

As shown in Table 2 and Table 4, when yC is less than (( 1 << log2TrafoHeight ) – 2), the  WD6 and the proposed WD text result in the same results of  ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) .

As shown in Table 3 and Table 5 Table 3, when yC is not less than (( 1 << log2TrafoHeight ) – 2), the  WD6 and the proposed WD text result in the same results of  ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) .

Therefore by mathematical logic, the WD6 [1] and the proposed WD text always result in the same results of  ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28).