| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0274 |
| --- | --- |

| Title: | A proposal to reduce data dependency in the context selection of significant_coeff_flag. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove a data dependency in WD6 in the derivation of sigCtx for the syntax element significant_coeff_flag for transform blocks larger than 8x8. This proposal facilitates implementation with pipelining and parallel processing. Furthermore, it results in the same results as WD6. A formal proof is provided in the Appendix, and experiments demonstrated that the intra BDR is 0.000000% for All Intra Main and All Intra HE10.

# Introduction

For transform block larger than 8x8, the context index increment of significant_coeff_flag is computed with a template. As shown in Figure 1s, the template consist of 5 points {I, H, F, E, B}.  In WD6 [1] and HM6, the computation of the context index increment can be conceptually separated in three stages.

- In the first stage, a cnt is first computed from the four positions {I, H, F, E} inside the current picture. In this stage, cnt has a range of [0,4].

- In the second stage, if the cnt resulted from the first stage is less than 4, than the significance of the position B is also added to cnt.

- In the last stage, the final value of the context increment is ( (cnt + 1) >> 1 ).

Figure 1: Template for the selection of significant map context

# A proposal to remove data dependency

In the second stage, whether the significance of the position B is added to cnt depends on the prior value of cnt. In other words, whether this addition operation is executed depends on the data value of cnt. This data dependency is not friendly to pipelining and to parallel processing.

For pipelining and parallel processing, this contribution proposew to remove the dependency of the execution of the addition operation on the prior value of cnt in stage 2.

# WD text

In 9.2.3.1.4 HM6 [1], we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-1)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-2)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-3)

# Propose WD text

In the propose text, we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-4)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx =  ( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 )  + 24	(9-5)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 )  + ( (cIdx > 0) ? 18 : 21 )	(9-6)

In particular, as shown in Appendix A, for sigCtx in [0,5], the operation

( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 )

in (9-27) and (9-28) of the proposed WD text is equivalent to

max( 2, (sigCtx + 1) >> 1 )

As shown in the proposed WD text, the execution of (9-26) is no longer depends on the value of sigCtx.

# Results

A formal proof that the propose WD text results in exactly the same results as the WD6 [1] is provided in the appendix. To further demonstration that the results are the same, Table 1shows that the BDR of the propose changes is 0.000000% for All Intra Main and All Intra HE10.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class B | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class C | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class D | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class E | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Overall | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
|  | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Class F | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% | 0.000000% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 1: BDR resulted from the proposed changes in WD6.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

As shown in Table 2, for sigCtx in [0,5], we have

min( 2, (sigCtx + 1) >> 1 )  = ( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 )

| sigCtx | sigCtx + 1 | ((sigCtx + 3) >> 3) | ( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 ) | min( 2, (sigCtx + 1) >> 1 ) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 |
| 1 | 2 | 0 | 1 | 1 |
| 2 | 3 | 0 | 1 | 1 |
| 3 | 4 | 0 | 2 | 2 |
| 4 | 5 | 0 | 2 | 2 |
| 5 | 6 | 1 | 2 | 2 |

Table 2:   The equation min( 2, (sigCtx + 1) >> 1 )  and the equation ( (sigCtx + 1 − ((sigCtx + 3) >> 3) ) >> 1 ) has the same value for sigCtx in [0,5].

# Appendix B

This is a formal proof that the propose WD texts result in the same sigCtx as the HM6 [1].

In 9.2.3.1.4 of HM6 [1], we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-7)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-8)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-9)

Table 3 and Table 4 shows the results of sigCtx at equation (9-26) and the corresponding results of ( (sigCtx + 1) >> 1 ) in equation (9-27) and (9-28) when yC is less than and not less than ( 1 << log2TrafoHeight ) – 2 respectively.

| sigCtx just before (9-26) | significant_coeff_flag[ xC ][ yC + 2 ] if input to (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 2 | 1 |
| 2 | 0 | 2 | 1 |
| 2 | 1 | 3 | 2 |
| 3 | 0 | 3 | 2 |
| 3 | 1 | 4 | 2 |
| 4 | 0 | 4 | 2 |
| 4 | 1 | 4 | 2 |

Table 3: Results from WD6 text when yC is less than (( 1 << log2TrafoHeight ) – 2).

| sigCtx just before (9-26) | sigCtx just after (9-26) | ( (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 1 |
| 3 | 3 | 2 |
| 4 | 4 | 2 |

Table 4: Results from WD6 text when yC  is not less than (( 1 << log2TrafoHeight ) – 2).

In the propose text, we have

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx =  min( 2,  (sigCtx + 1) >> 1 )  + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = min( 2, (sigCtx + 1 ) >> 1 )  + ( (cIdx > 0) ? 18 : 21 )	(9-12)

Table 5 and Table 6 shows the results of sigCtx at equation (9-26) and the corresponding results of ( (sigCtx + 1) >> 1 ) in equation (9-27) and (9-28) when yC is less than and not less than ( 1 << log2TrafoHeight ) – 2 respectively.

| sigCtx just before (9-26) | significant_coeff_flag[ xC ][ yC + 2 ] if input to (9-26) | sigCtx just after (9-26) | min(2, (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 2 | 1 |
| 2 | 0 | 2 | 1 |
| 2 | 1 | 3 | 2 |
| 3 | 0 | 3 | 2 |
| 3 | 1 | 4 | 2 |
| 4 | 0 | 4 | 2 |
| 4 | 1 | 5 | 2 |

Table 5: Results from proposed WD text when yC is less than (( 1 << log2TrafoHeight ) – 2).

| sigCtx just before (9-26) | sigCtx just after (9-26) | max( 2, (sigCtx + 1) >> 1 ) in (9-27) and (9-28) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 1 |
| 3 | 3 | 2 |
| 4 | 4 | 2 |

Table 6: Results from proposed WD text when yC is not less than (( 1 << log2TrafoHeight ) – 2).

As shown in Table 3 and Table 5, when yC is less than (( 1 << log2TrafoHeight ) – 2), the  WD6 and the proposed WD text result in the same results in (9-27) and (9-28) .

As shown in Table 6 and Table 6, when yC is not less than (( 1 << log2TrafoHeight ) – 2), the  WD6 and the proposed WD text result in the same results in (9-27) and (9-28) .

Therefore, the WD6 and the proposed WD text always result in the same results in (9-27) and (9-28).