| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0275 WG11 Number: m24522 |
| --- | --- |

| Title: | On Leading Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes explicit signaling of leading pictures that follow a CRA picture in decoding order and precede it in output order. A leading pictures that follow a CRA picture (LFC) NAL unit type is proposed. It is asserted that this LFC NAL Unit type can allow a server / network node to easily discard leading pictures when sending a bitstream starting at the corresponding CRA picture. The Proposed LFC NAL unit type does not require changes to the decoding process of pictures.

# Introduction

JCTVC-H0496 [1] proposed bitstreams starting with CRA pictures. This functionality has been incorporated in HEVC CD [2].

We propose explicit signaling of leading pictures that follow a CRA picture in decoding order and precede it in output order.

We propose a new - leading pictures that follow a CRA picture (LFC) NAL Unit type to indicate these leading pictures. This will allow server/ network node to easily discard these leading pictures by examining its NAL unit type without requiring decoding of Sequence Parameter Set (SPS) and Picture Parameter Set (PPS). Without the proposed approach the server/ network node needs to decode SPS, PPS and compute the current picture’s POC value and compare it to the POC value of the corresponding CRA picture to deduce that a picture is a leading picture. Using our proposed approach these steps are not required to identify a leading picture.

Leading pictures may be discarded from the bitstream as they are not specified for output and have no effect on the decoding process of any other pictures that are specified for output.

# Proposed Change

We propose following syntax and semantics. The proposed changes are highlighted below compared to HEVC CD [2].

## Syntax and Semantics:

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that use nal_unit_type equal to 0 or in the range of 24..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 24..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 24..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA, non-TLA and non-LFC picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 15 | Coded slice of a leading picture following a CRA (LFC) picture slice_layer_rbsp( ) | VCL |
| 16-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

In the text, coded slice NAL unit collectively refers to a coded slice of a non-IDR picture NAL unit or to a coded slice of an IDR picture NAL unit. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  5 )  ?  1  :  0 )		(7-1)

When the value of nal_unit_type is equal to 3 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 3. Such a picture is referred to as a TLA picture.

When the value of nal_unit_type is equal to 4 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 4. Such a picture is referred to as a CRA picture.

When the value of nal_unit_type is equal to 5 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 5. Such a picture is referred to as an IDR picture.

When the value of nal_unit_type is equal to 15 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 15. Such a picture is referred to as a leading picture following a CRA (LFC) picture.

# Conclusion

The document proposed a new NAL unit type to signal leading pictures following a CRA picture. We believe that signaling these pictures in NAL unit header can allow easily discarding these pictures in the server/ network node when random access at the corresponding CRA picture is desired. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Y.-K. Wang, Y. Chen, M. Karczewicz, J. Chen, "On bitstreams starting with CRA pictures," JCTVC-H0496, 8th Meeting: San Jose, USA, 1-10 February, 2012.

[2] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003_dK, San Jose, February, 2012.