| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0276 |
| --- | --- |

| Title: | Updated design of the probability estimator and the coding engine of the M Coder |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Jan Stegemann Heiner Kirchhoffer Christian Bartnik Detlev Marpe Mischa Siekmann Thomas Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 183 jan.stegemann@hhi.fraunhofer.de |  |
| Source: | Fraunhofer HHI |  |  |  |

_____________________________

# Abstract

This contribution proposes a set of modifications to the current design of the probability estimator and the entropy coding engine of HEVC. The state-based probability estimator is extended from 63 states to 112 states and the table for the interval subdivision of the arithmetic coding engine is adjusted to conform with the internal state of the probability models. While each probability model requires 1 additional bit of memory the average gains in coding efficiency amount from 0.3 to 0.5%.

# Introduction

Recent contributions[1][2] have shown that probability estimators with larger window sizes outperform the M Coders probability estimation in terms of coding efficiency. The superior performance of probability estimators with larger window sizes depends on the representation of probabilities at a higher accuracy. Accordingly, in an implementation a greater number of bits have to be used for each probability model and the memory requirements of the entropy coding stage is increased. In this contribution a solution is presented that requires 8 bits for each probability model but also offers a scalable configuration to reduce the overall memory consumption of the arithmetic coding engine.

# Probability Estimation of the M Coder

Each probability model is equipped with a probability estimator, which keeps track of the underlying source statistics. The length of the sequence of past bins that is used for the estimation is embodied byα. After the encoding and decoding of a bin the probability model is updated dependent on the value of the bin equalling a MPS or a LPS. The probability for the next bin being a LPS is denoted by pt+1 and is calculated using the following equation.														pt+1t+1=α∗pt,                 if MPS occurs				(1)

In the current design the LPS probability is represented by a state index and the bin value of the MPS[4]. If the value of bin being coded equals a MPS the state is simply increased by one unless the maximum state is already set. In this case the state index remains on the maximum value. If the bin being coded is a LPS the probability model is set to the internal state index that represents the probability as calculated by equation 1. To avoid the need of computing new probabilities after every bin the update of the probability models is done using transition tables that contain the resulting state index for each state and for both LPS and MPS.

# Proposed Probability Estimator

In the proposed scheme the probability estimator operates with 112 state indexes instead of the 63 state indexes of the M Coder[4] as it used in the current version of HEVC. The update of the probability model is done using transition tables derived in the same manner as explained in the previous section but featuring 112 state indexes and α=0.969375. The LPS and MPS transition tables for the update of the internal state index of each probability model are depicted as follows:

Table 1 LPS transition table for 112 states

| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 16 | 17 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 24 | 25 | 26 | 27 |
| 28 | 29 | 30 | 30 | 31 | 32 | 33 | 34 | 35 | 35 | 36 | 37 | 38 | 39 | 39 | 40 |
| 41 | 42 | 43 | 43 | 44 | 45 | 46 | 46 | 47 | 48 | 49 | 49 | 50 | 51 | 51 | 52 |
| 53 | 53 | 54 | 55 | 55 | 56 | 57 | 57 | 58 | 59 | 59 | 60 | 60 | 61 | 62 | 62 |
| 63 | 63 | 64 | 64 | 65 | 66 | 66 | 67 | 67 | 68 | 68 | 69 | 69 | 70 | 70 | 70 |
| 71 | 71 | 72 | 72 | 73 | 73 | 73 | 74 | 74 | 75 | 75 | 75 | 76 | 76 | 76 | 77 |

Table 2 MPS transition table for 112 states

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |
| 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |
| 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 |
| 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 111 |

Equal to the update of the M Coder the internal state index of the probability model is used to address the LPS and MPS transition table and set the new internal state index. Toggling the valMPS also remains the same and only happens if a LPS occurs and the internal state of the probability model equals 0.

# Adjustments to the coding engine

The interval subdivision of the arithmetic coding engine is done using a table of pre-calculated range values. It is addressed by simple bit-shift operations of the internal state index of the probability model. In this proposal the arithmetic coding engine is presented in two different layouts. The first layout consists of a table with 28x8 pre-calculated range values. This table is designed to yield a maximum coding efficiency.  The second layout features a table with 28x4 pre-calculated range values and therefore consumes less then half the amount of memory compared to the current table in HEVC. For both layouts each value is set to the value that produces the minimum bit-rate for encoding and decoding. Their performance in terms of BD-rate is shown in the results section.

# Probability model initialization

The probability model initialization is updated to cover the range of 112 states instead of 63 for the initial probability of the LPS. Hence a new set of initialization values is provided for each probability model to match the extended range of internal state indexes. Note the basic mode of operation is identical to the initialization of the HM-6.0 anchor as it is proposed in [6]. To evaluate the impact of recalculating a new set of init values the following table shows the results for applying the equivalent recalculation procedure on the anchor version of HEVC.

# Memory consumption

Table 3 contains a comparison of the memory consumption of the two layouts of this proposal and the current version of HEVC. The MPS transition table is not taken into account as it can be realized without a table.

Table 3 Comparison of memory consumption

|  | M Coder | JCTV-I0276(28x4) | JCTV-I0276(28x8) |
| --- | --- | --- | --- |
| LPS Transition Table (Bits): | 64x6 | 112x7 | 112x7 |
| Range Table (Bits): | 64x4x8 | 28x4x8 | 28x8x8 |
| Sum (Bits): | 2432 | 1680 | 2576 |
| Bits per Context Model: | 7 | 8 | 8 |

- Results

The design is implemented into the HM6.0 software and is included as a patch file with this proposal. The code is activated by setting the define MCODE112_I0276 to 1. The define USE_RTAB_28x4 is used to activate the range table with 28x4 elements. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The simulations are run using the common test conditions for HM6.0[5].

## Hardware

The type of computer that has been used to run the simulations all running a 64 bit Ubuntu 10.10 Linux operating system are listed in Table 4. Turbo-Mode and Hyper-Threading are deactivated.

Table 4	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Encoding and Decoding | 2x6 | 2 x Intel(R) Xeon(R) CPU X5675 @ 3.07GHz | 64 GB |

## Results using the 28x8 range table

## Results using the 28x4 range table

# Conclusion

Both configurations in this proposal outperform the HM6.0 anchor in terms of coding efficiency while the amount of arithmetic operations remains the same. Furthermore overall memory consumption is reduced drastically when comparing the configuration with the 28x4 range table with HM 6.0 anchor.

# References

- A. Alshin, E. Alshina, “Multi-parameter probability update for CABAC”, JCTVC-F254, Torino, July, 2011.

- J. Stegemann, H. Kirchhoffer, D. Marpe, T.Wiegand, “Non-CE1: Counter-based probability model update with adapted arithmetic coding engine”, JCTVC-G547, Geneva, November, 2011.

- J. Stegemann, H. Kirchhoffer, D. Marpe, T.Wiegand, “Non-CE1: Dimension reduced rangeTabLPS table for the interval subdivision of the arithmetic coding engine”, JCTVC-H0265, San Jose, February 2012.

- D. Marpe, H. Schwarz, and T. Wiegand: Context-Based Adaptive Binary Arithmetic Coding in the H.264 / AVC Video Compression Standard, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, San Jose, February, 2012.

- Liwei Guo, “CE1 B3: 8-bit Linear Initialization for CABAC”, JCTVC-H0535, San Jose, February, 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).