| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011 | Document: JCTVC-H1003 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 6 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 6 of High efficiency video coding.

# Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply:

  - access unit: A set of NAL units that are consecutive in decoding order and contain exactly one coded picture. In addition to the coded slice NAL units of the coded picture, the access unit may also contain other NAL units not containing slices of the coded picture. The decoding of an access unit always results in a decoded picture.

  - AC transform coefficient: Any transform coefficient for which the frequency index in one or both dimensions is non-zero.

  - adaptive binary arithmetic decoding process: An entropy decoding process that derives the values of bins from a bitstream produced by an adaptive binary arithmetic encoding process.

  - adaptive binary arithmetic encoding process: An entropy encoding process, not normatively specified in this Recommendation | International Standard, that codes a sequence of bins and produces a bitstream that can be decoded using the adaptive binary arithmetic decoding process.

  - B slice: A slice that may be decoded using intra prediction or inter prediction using at most two motion vectors and reference indices to predict the sample values of each block.

  - bin: One bit of a bin string.

  - binarization: A set of bin strings for all possible values of a syntax element.

  - binarization process: A unique mapping process of all possible values of a syntax element onto a set of bin strings.

  - bin string: A string of bins. A bin string is an intermediate binary representation of values of syntax elements from the binarization of the syntax element.

  - bi-predictive slice: See B slice.

  - bitstream: A sequence of bits that forms the representation of coded pictures and associated data forming one or more coded video sequences. Bitstream is a collective term used to refer either to a NAL unit stream or a byte stream.

  - block: An MxN (M-column by N-row) array of samples, or an MxN array of transform coefficients.

  - broken link: A location in a bitstream at which it is indicated that some subsequent pictures in decoding order may contain serious visual artefacts due to unspecified operations performed in the generation of the bitstream.

  - byte: A sequence of 8 bits, written and read with the most significant bit on the left and the least significant bit on the right. When represented in a sequence of data bits, the most significant bit of a byte is first.

  - byte-aligned: A position in a bitstream is byte-aligned when the position is an integer multiple of 8 bits from the position of the first bit in the bitstream. A bit or byte or syntax element is said to be byte-aligned when the position at which it appears in a bitstream is byte-aligned.

  - byte stream: An encapsulation of a NAL unit stream containing start code prefixes and NAL units as specified in Annex Error: Reference source not found.

  - can: A term used to refer to behaviour that is allowed, but not necessarily required.

  - chroma: An adjective specifying that a sample array or single sample is representing one of the two colour difference signals related to the primary colours. The symbols used for a chroma array or sample are Cb and Cr.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

  - clean random access unit with leading pictures (CWL): An access unit in which the coded picture is a CRA picture for which the associated leading pictures are present.

  - clean random access unit without leading pictures (CNL): An access unit in which the coded picture is a CRA picture for which the associated leading pictures are not pressent.

  - clean random access (CRA) picture: A coded picture containing only I slices and for which all coded pictures that follow the CRA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the CRA picture either in decoding order or output order; and any picture that precedes the CRA picture in decoding order also precedes the CRA picture in output order.

  - coded picture: A coded representation of a picture containing all treeblocks of the picture.

  - coded picture buffer (CPB): A first-in first-out buffer containing access units in decoding order specified in the hypothetical reference decoder in Annex A.

  - coded representation: A data element as represented in its coded form.

  - coded slice NAL unit: A NAL unit containing a slice.

  - coded video sequence: A sequence of access units that consists, in decoding order, of an IDR access unit followed by zero or more non-IDR access units including all subsequent access units up to but not including any subsequent IDR access unit.

  - column: An integer number of treeblocks. Columns are delineated from one another by vertical boundaries that extend from the top boundary to the bottom boundary of the picture and are ordered consecutively from left to right in the picture. The division of each picture into columns is a partitioning.

  - component: An array or single sample from one of the three arrays (luma and two chroma) that make up a picture in 4:2:0, 4:2:2, or 4:4:4 colour format or the array or a single sample of the array that make up a picture in monochrome format.

  - context variable: A variable specified for the adaptive binary arithmetic decoding process of a bin by an equation containing recently decoded bins.

  - DC transform coefficient: A transform coefficient for which the frequency index is zero in all dimensions.

  - decoded picture: A decoded picture is derived by decoding a coded picture.

  - decoded picture buffer (DPB): A buffer holding decoded pictures for reference, output reordering, or output delay specified for the hypothetical reference decoder in Annex A.

  - decoder: An embodiment of a decoding process.

  - decoder under test (DUT): A decoder that is tested for conformance to this Recommendation | International Standard by operating the hypothetical stream scheduler to deliver a conforming bitstream to the decoder and to the hypothetical reference decoder and comparing the values and timing of the output of the two decoders.

  - decoding order: The order in which syntax elements are processed by the decoding process.

  - decoding process: The process specified in this Recommendation | International Standard that reads a bitstream and derives decoded pictures from it.

  - display process: A process not specified in this Recommendation | International Standard having, as its input, the cropped decoded pictures that are the output of the decoding process.

  - emulation prevention byte: A byte equal to 0x03 that may be present within a NAL unit. The presence of emulation prevention bytes ensures that no sequence of consecutive byte-aligned bytes in the NAL unit contains a start code prefix.

  - encoder: An embodiment of an encoding process.

  - encoding process: A process, not specified in this Recommendation | International Standard, that produces a bitstream conforming to this Recommendation | International Standard.

  - flag: A variable that can take one of the two possible values 0 and 1.

  - frequency index: A one-dimensional or two-dimensional index associated with a transform coefficient prior to an inverse transform part of the decoding process.

  - hypothetical reference decoder (HRD): A hypothetical decoder model that specifies constraints on the variability of conforming NAL unit streams or conforming byte streams that an encoding process may produce.

  - hypothetical stream scheduler (HSS): A hypothetical delivery mechanism for the timing and data flow of the input of a bitstream into the hypothetical reference decoder. The HSS is used for checking the conformance of a bitstream or a decoder.

  - I slice: A slice that is decoded using intra prediction only.

  - informative: A term used to refer to content provided in this Recommendation | International Standard that is not an integral part of this Recommendation | International Standard. Informative content does not establish any mandatory requirements for conformance to this Recommendation | International Standard.

  - instantaneous decoding refresh (IDR) access unit: An access unit in which the coded picture is an IDR picture.

  - instantaneous decoding refresh (IDR) picture: A coded picture for which the variable IdrPicFlag is equal to 1. An IDR picture causes the decoding process to mark all reference pictures as "unused for reference". All coded pictures that follow an IDR picture in decoding order can be decoded without inter prediction from any picture that precedes the IDR picture in decoding order. The first picture of each coded video sequence in decoding order is an IDR picture.

  - inter coding: Coding of a block, macroblock, slice, or picture that uses inter prediction.

  - inter prediction: A prediction derived from only data elements (e.g. sample value or motion vector) of reference pictures other than the current decoded picture.

  - intra coding: Coding of a block, macroblock, slice, or picture that uses intra prediction.

  - intra prediction: A prediction derived from only data elements (e.g. sample value) of the same decoded slice.

  - intra slice: See I slice.

  - inverse transform: A part of the decoding process by which a set of transform coefficients are converted into spatial-domain values, or by which a set of transform coefficients are converted into DC transform coefficients.

  - layer: One of a set of syntactical structures in a non-branching hierarchical relationship. Higher layers contain lower layers. The coding layers are the coded video sequence, picture, slice, and treeblock layers.

  - leading picture: A coded picture that follows some other particular picture in decoding order and precedes it in output order.

  - leaf: A terminating node of a tree that is a root node of a tree of depth 0.

  - level: A defined set of constraints on the values that may be taken by the syntax elements and variables of this Recommendation | International Standard. The same set of levels is defined for all profiles, with most aspects of the definition of each level being in common across different profiles. Individual implementations may, within specified constraints, support a different level for each supported profile. In a different context, level is the value of a transform coefficient prior to scaling.

  - list 0 (list 1) motion vector: A motion vector associated with a reference index pointing into reference picture list 0 (list 1).

  - list 0 (list 1, list combination) prediction: Inter prediction of the content of a slice using a reference index pointing into reference picture list 0 (list 1, combination).

  - luma: An adjective specifying that a sample array or single sample is representing the monochrome signal related to the primary colours. The symbol or subscript used for luma is Y or L.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance. The symbol L is sometimes used instead of the symbol Y to avoid confusion with the symbol y as used for vertical location.

  - may: A term used to refer to behaviour that is allowed, but not necessarily required. In some places where the optional nature of the described behaviour is intended to be emphasized, the phrase "may or may not" is used to provide emphasis.

  - motion vector: A two-dimensional vector used for inter prediction that provides an offset from the coordinates in the decoded picture to the coordinates in a reference picture.

  - must: A term used in expressing an observation about a requirement or an implication of a requirement that is specified elsewhere in this Recommendation | International Standard. This term is used exclusively in an informative context.

  - NAL unit: A syntax structure containing an indication of the type of data to follow and bytes containing that data in the form of an RBSP interspersed as necessary with emulation prevention bytes.

  - NAL unit stream: A sequence of NAL units.

  - non-reference picture: A picture coded with nal_ref_flag equal to 0. A non-reference picture is not used for inter prediction of any other pictures.

  - note: A term used to prefix informative remarks. This term is used exclusively in an informative context.

  - output order: The order in which the decoded pictures are output from the decoded picture buffer in case the decoded pictures are to be output from the decoded picture buffer. The output order of a picture is specified by the picture order count value, regardless of whether the picture is to be output.

  - P slice: A slice that may be decoded using intra prediction or inter prediction using at most one motion vector and reference index to predict the sample values of each block.

  - parameter: A syntax element of a sequence parameter set or a picture parameter set. Parameter is also used as part of the defined term quantization parameter.

  - partitioning: The division of a set into subsets such that each element of the set is in exactly one of the subsets.

  - picture:

  - picture parameter set: A syntax structure containing syntax elements that apply to zero or more entire coded pictures as determined by the pic_parameter_set_id syntax element found in each slice header.

  - picture order count: A variable that is associated with each coded picture and has a value that is increasing with increasing picture position in output order relative to the previous IDR picture in decoding order, if any.

  - prediction: An embodiment of the prediction process.

  - prediction process: The use of a predictor to provide an estimate of the data element (e.g. sample value or motion vector) currently being decoded.

  - predictive slice: See P slice.

  - predictor: A combination of specified values or previously decoded data elements (e.g. sample value or motion vector) used in the decoding process of subsequent data elements.

  - profile: A specified subset of the syntax of this Recommendation | International Standard.

  - quadtree: A tree in which a parent node can be split into four child nodes. A child node may become parent node for another plit into four child nodes.

  - quantization parameter: A variable used by the decoding process for scaling of transform coefficient levels.

  - random access: The act of starting the decoding process for a bitstream at a point other than the beginning of the stream.

  - raster scan: A mapping of a rectangular two-dimensional pattern to a one-dimensional pattern such that the first entries in the one-dimensional pattern are from the first top row of the two-dimensional pattern scanned from left to right, followed similarly by the second, third, etc., rows of the pattern (going down) each scanned from left to right.

  - raw byte sequence payload (RBSP): A syntax structure containing an integer number of bytes that is encapsulated in a NAL unit. An RBSP is either empty or has the form of a string of data bits containing syntax elements followed by an RBSP stop bit and followed by zero or more subsequent bits equal to 0.

  - raw byte sequence payload (RBSP) stop bit: A bit equal to 1 present within a raw byte sequence payload (RBSP) after a string of data bits. The location of the end of the string of data bits within an RBSP can be identified by searching from the end of the RBSP for the RBSP stop bit, which is the last non-zero bit in the RBSP.

  - recovery point: A point in the bitstream at which the recovery of an exact or an approximate representation of the decoded pictures represented by the bitstream is achieved after a random access or broken link.

  - reference index: An index into a reference picture list.

  - reference picture: A picture with nal_ref_flag equal to 1. A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture list: A list of reference pictures that is used for uni-prediction of a P or B slice. For the decoding process of a P slice, there is one reference picture list. For the decoding process of a B slice, there are two reference picture lists (list 0 and list 1) and a reference picture lists combination.

  - reference picture list 0: A reference picture list used for inter prediction of a P or B slice. All inter prediction used for P slices uses reference picture list 0. Reference picture list 0 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 1.

  - reference picture list 1: A reference picture list used for bi-prediction of a B slice. Reference picture list 1 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 0.

  - reference picture lists combination: A reference picture list used for uni-prediction of a B slice. Reference picture lists combination is derived from the entries of the reference picture lists 0 and reference picture list 1.

  - reference picture set: A set of reference pictures associated with a picture, consisting of all reference pictures that are prior to the associated picture in decoding order, that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order.

  - reserved: The term reserved, when used in the clauses specifying some values of a particular syntax element, are for future use by ITU-T | ISO/IEC. These values shall not be used in bitstreams conforming to this Recommendation | International Standard, but may be used in future extensions of this Recommendation | International Standard by ITU-T | ISO/IEC.

  - residual: The decoded difference between a prediction of a sample or data element and its decoded value.

  - row: An integer number of treeblocks. Rows are delineated from one another by horizontal boundaries that extend from the left boundary to the right boundary of the picture and are ordered consecutively from top to bottom in the picture. The division of each picture into rows is a partitioning.

  - sample aspect ratio: Specifies, for assisting the display process, which is not specified in this Recommendation | International Standard, the ratio between the intended horizontal distance between the columns and the intended vertical distance between the rows of the luma sample array in a picture. Sample aspect ratio is expressed as h:v, where h is horizontal width and v is vertical height (in arbitrary units of spatial distance).

  - scaling: The process of multiplying transform coefficient levels by a factor, resulting in transform coefficients.

  - sequence parameter set: A syntax structure containing syntax elements that apply to zero or more entire coded video sequences as determined by the content of a seq_parameter_set_id syntax element found in the picture parameter set referred to by the pic_parameter_set_id syntax element found in each slice header.

  - shall: A term used to express mandatory requirements for conformance to this Recommendation | International Standard. When used to express a mandatory constraint on the values of syntax elements or on the results obtained by operation of the specified decoding process, it is the responsibility of the encoder to ensure that the constraint is fulfilled. When used in reference to operations performed by the decoding process, any decoding process that produces identical results to the decoding process described herein conforms to the decoding process requirements of this Recommendation | International Standard.

  - should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Recommendation | International Standard.

  - slice: An integer number of treeblocks ordered consecutively in the raster scan. The division of each picture into slices is a partitioning. The treeblock addresses are derived from the first treeblock address in a slice (as represented in the slice header).

  - slice header: A part of a coded slice containing the data elements pertaining to the first or all macroblocks represented in the slice.

  - source: Term used to describe the video material or some of its attributes before encoding.

  - start code prefix: A unique sequence of three bytes equal to 0x000001 embedded in the byte stream as a prefix to each NAL unit. The location of a start code prefix can be used by a decoder to identify the beginning of a new NAL unit and the end of a previous NAL unit. Emulation of start code prefixes is prevented within NAL units by the inclusion of emulation prevention bytes.

  - string of data bits (SODB): A sequence of some number of bits representing syntax elements present within a raw byte sequence payload prior to the raw byte sequence payload stop bit. Within an SODB, the left-most bit is considered to be the first and most significant bit, and the right-most bit is considered to be the last and least significant bit.

  - syntax element: An element of data represented in the bitstream.

  - syntax structure: Zero or more syntax elements present together in the bitstream in a specified order.

  - temporal layer access (TLA) unit: An access unit in which the coded picture is a TLA picture.

  - temporal layer access (TLA) picture: A coded picture for which each slice has nal_unit_type equal to 3; the TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order.

  - tile: An integer number of treeblocks co-occurring in one column and one row, ordered consecutively in treeblock raster scan of the tile. The division of each picture into tiles is a partitioning. Tiles in a picture are ordered consecutively in tile raster scan of the picture. Although a slice contains treeblocks that are consecutive in treeblock raster scan of a tile, these treeblocks are not necessarily consecutive in treeblock raster scan of the picture.

  - transform coefficient: A scalar quantity, considered to be in a frequency domain, that is associated with a particular one-dimensional or two-dimensional frequency index in an inverse transform part of the decoding process.

  - transform coefficient level: An integer quantity representing the value associated with a particular two-dimensional frequency index in the decoding process prior to scaling for computation of a transform coefficient value.

  - tree: A tree is a finite set of nodes with a unique root node. A terminating node is called a leaf.

  - treeblock: A NxN block of luma samples and two corresponding blocks of chroma samples of a picture that has three sample arrays, or a NxN block of samples of a monochrome picture or a picture that is coded using three separate colour planes. The division of a slice into treeblocks is a partitioning.

  - treeblock address: A treeblock address is the index of a treeblock in a treeblock raster scan of the picture starting with zero for the top-left treeblock in a picture.

  - treeblock location: The two-dimensional coordinates of a treeblock in a picture denoted by ( x, y ). For the top left treeblock of the picture ( x, y ) is equal to ( 0, 0 ). x is incremented by 1 for each treeblock column from left to right. The value of y is incremented by 1 for each treeblock row from top to bottom.

  - treeblock partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a treeblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a treeblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes.

  - universal unique identifier (UUID): An identifier that is unique with respect to the space of all universal unique identifiers.

  - unspecified: The term unspecified, when used in the clauses specifying some values of a particular syntax element, indicates that the values have no specified meaning in this Recommendation | International Standard and will not have a specified meaning in the future as an integral part of this Recommendation | International Standard.

  - variable length coding (VLC): A reversible procedure for entropy coding that assigns shorter bit strings to symbols expected to be more frequent and longer bit strings to symbols expected to be less frequent.

  - VCL NAL unit: A collective term for coded slice NAL units.

  - z-scan: A specific seqential ordering of blocks partitioning a picture. When the blocks are of the same size as treeblocks, the z-scan order is identicdal to treeblock raster scan of the picture. When the blocks are of a smaller size than treeblocks, i.e., treeblocks are further partitioned into smaller blocks, the z-scan order traverses from treeblock to treeblock in treeblock raster scan of the picture, and inside each treeblock, which may be divided into quadtrees hierachically to lower levels, the z-scan order traverses from quadree to quadree of a particular level in quadtree-of-the-particular-level raster scan of the quadtree of the immediately higher level.

  - zig-zag scan: A specific sequential ordering of transform coefficient levels from (approximately) the lowest spatial frequency to the highest.

## Semantics

Semantics associated with the syntax structures and with the syntax elements within these structures are specified in this subclause. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

### NAL unit semantics

NOTE 1 – The VCL is specified to efficiently represent the content of the video data. The NAL is specified to format that data and provide header information in a manner appropriate for conveyance on a variety of communication channels or storage media. All data are contained in NAL units, each of which contains an integer number of bytes. A NAL unit specifies a generic format for use in both packet-oriented and bitstream systems. The format of NAL units for both packet-oriented transport and byte stream is identical except that each NAL unit can be preceded by a start code prefix and extra padding bytes in the byte stream format.

NumBytesInNALunit specifies the size of the NAL unit in bytes. This value is required for decoding of the NAL unit. Some form of demarcation of NAL unit boundaries is necessary to enable inference of NumBytesInNALunit. One such demarcation method is specified in Annex Error: Reference source not found for the byte stream format. Other methods of demarcation may be specified outside of this Recommendation | International Standard.

forbidden_zero_bit shall be equal to 0.

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture.

For coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

nal_ref_flag shall be equal to 1 for sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one NAL unit with nal_unit_type equal to 1 a particular picture, it shall be equal to 0 for all NAL units with nal_unit_type equal to 1 the picture.

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4, 5 or 16.

nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 6, 9, 10, 11, or 12.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that use nal_unit_type equal to 0 or in the range of 24..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 24..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 24..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA, non-CNL and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture for which the associated leading pictures are present slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 16 | Coded slice of a CRA picture for which associated leading pictures are not present slice_layer_rbsp( ) | VCL |
| 17-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

In the text, coded slice NAL unit collectively refers to a coded slice of a non-IDR picture NAL unit or to a coded slice of an IDR picture NAL unit. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  5 )  ?  1  :  0 )		(7-1)

When the value of nal_unit_type is equal to 3 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 3. Such a picture is referred to as a TLA picture.

When the value of nal_unit_type is equal to 4 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 4. Such a picture is referred to as a CRA picture for which the associated leading pictures are present.

When the value of nal_unit_type is equal to 5 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 5. Such a picture is referred to as an IDR picture.

When the value of nal_unit_type is equal to 16 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 16. Such a picture is referred to as a CRA picture for which the associated leading pictures are not present.

## Slice decoding process

### Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in temporal or spatial direct mode, to represent picture order differences between pictures for motion vector derivation, for implicit mode weighted prediction in B slices (see subclause Error: Reference source not found), and for decoder conformance checking (see subclause B.3).

Each coded picture is associated with one picture order count, called PicOrderCntVal. PicOrderCntVal indicates the picture order of the corresponding picture relative to the previous IDR picture in decoding order.

- The variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows.

- If the current picture is an IDR picture, both prevPicOrderCntLsb and prevPicOrderCntMsb are set equal to 0.

–	Otherwise (the current picture is not an IDR picture), the following applies.

–	If the first coded picture in the bitstream is a CRA picture and the current picture is the first coded picture in the bitstream, both prevPicOrderCntLsb and prevPicOrderCntMsb are set to 0.

–	Otherwise let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic.

PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-2)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as

PicOrderCntVal = PicOrderCntMsb + pic_order_cnt_lsb	(8-3)

NOTE 1 – All IDR pictures will have PicOrderCntVal equal to 0 since pic_order_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one coded video sequence, the PicOrderCntVal values for any two coded pictures shall be different.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-4)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-5)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that exceed the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

NOTE 3 – Many encoders assign PicOrderCntVal proportional to the sampling time of the corresponding picture relative to the sampling time of the previous IDR picture.

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

  - General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes B.3.1 and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes A, Error: Reference source not found, and Error: Reference source not found, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

Figure C-2 – HRD buffer model

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of picture storage buffers) for temporal layer X is max_dec_pic_buffering[ X ] + 1 for each X in the range of 0 to max_temporal_layers_minus1, inclusive.

The HRD operates as follows. Data associated with access units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each access unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB at its CPB removal time. When a picture is placed in the DPB it is removed from the DPB at the later of the DPB output time or the time that it is marked as "unused for reference".

The operation of the CPB is specified in subclause C.1. The instantaneous decoder operation is specified in clauses 2-9. The operation of the DPB is specified in subclause C.2. The output cropping is specified in subclause C.2.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses D.1.1 and D.2.1. The removal timing of access units from the CPB and output timing from the DPB are specified in the picture timing SEI message as specified in subclauses D.1.2 and D.2.2. All timing information relating to a specific access unit shall arrive prior to the CPB removal time of the access unit.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses C.3 and C.4, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of an access unit is not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the coded picture or the decoded picture of access unit n.

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

If the first access unit is a CNL access unit and the value of cra_info_present_flag of the associated buffering period SEI message is equal to 1, the variable UseInitialCpbRemovalDelay[ SchedSelIdx ]  is set to the value of initial_cra_cpb_removal_delay[ SchedSelIdx ] of the associated buffering period SEI message, otherwise, the variable UseInitialCpbRemovalDelay[ SchedSelIdx ]  is set to the value of initial_cpb_removal_delay[ SchedSelIdx ] of the associated buffering period SEI message.

If the first access unit is a CNL access unit and the value of cra_info_present_flag of the associated buffering period SEI message is equal to 1, the variable UseInitialCpbRemovalDelayOffset[ SchedSelIdx ]  is set to the value of initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] of the associated buffering period SEI message, otherwise, the variable UseInitialCpbRemovalDelayOffset[ SchedSelIdx ]  is set to the value of initialcpb_removal_delay_offset[ SchedSelIdx ] of the associated buffering period SEI message.

The time at which the first bit of access unit n begins to enter the CPB is referred to as the initial arrival time tai( n ).

The initial arrival time of access units is derived as follows.

–	If the access unit is access unit 0, tai( 0 ) = 0,

–	Otherwise (the access unit is access unit n with n > 0), the following applies.

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit n, is equal to the final arrival time (which is derived below) of access unit n − 1, i.e.,

tai( n ) = taf( n − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit n is derived by

tai( n ) = Max( taf( n − 1 ), tai,earliest( n ) )		(C-3)

where tai,earliest( n ) is derived as follows.

–	If access unit n is not the first access unit of a subsequent buffering period, tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( UseInitialCpbRemovalDelay[ SchedSelIdx ] +
	UseInitialCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( n ) being the nominal removal time of access unit n from the CPB as specified in subclause C.1.2.

–	Otherwise (access unit n is the first access unit of a subsequent buffering period), tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( UseInitialCpbRemovalDelay[ SchedSelIdx ]  90000 )	(C-5)

The final arrival time for access unit n is derived by

taf( n ) = tai( n ) + b( n )  BitRate[ SchedSelIdx ]		(C-6)

- where b( n ) is the size in bits of access unit n, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows.

–	If the content of the active sequence parameter sets for access unit n and access unit n − 1 differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for access unit n that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for access unit n. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for access unit n − 1.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies.

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( n )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows.

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( n ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the time tr( n ).

    - Timing of coded picture removal

- When an access unit n is the access unit with n equal to 0 (the access unit that initialises the HRD), the nominal removal time of the access unit from the CPB is specified by

tr,n( 0 ) = UseInitialCpbRemovalDelay[ SchedSelIdx ]  90000	(C-7)

When an access unit n is the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the access unit from the CPB is specified by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-8)

where tr,n( nb ) is the nominal removal time of the first access unit of the previous buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

When an access unit n is the first access unit of a buffering period, nb is set equal to n at the removal time tr,n( n ) of the access unit n.

The nominal removal time tr,n(n) of an access unit n that is not the first access unit of a buffering period is given by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-9)

where tr,n( nb ) is the nominal removal time of the first access unit of the current buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

The removal time of access unit n is specified as follows.

–	If low_delay_hrd_flag is equal to 0 or tr,n( n ) >= taf( n ), the removal time of access unit n is specified by

tr( n ) = tr,n( n )		(C-10)

–	Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), the removal time of access unit n is specified by

tr( n ) = tr,n( n ) + tc * Ceil( ( taf( n ) − tr,n( n ) ) tc ) 		(C-11)

NOTE – The latter case indicates that the size of access unit n, b( n ), is so large that it prevents removal at the nominal removal time.

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. Prior to initialization, the DPB is empty (the DPB fullness is set to zero). The following steps of the subclauses of this subclause all happen instantaneously at tr( n ) and in the sequence listed.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

If the current picture is an IDR picture, the following applies:

- When the IDR picture is not the first IDR picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples or pic_height_in_luma_samples.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

All pictures m in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture m is marked as "unused for reference",

–	picture m has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the current picture n; i.e., to,dpb( m ) <= tr( n )

When a picture is removed from the DPB, the DPB fullness is decremented by one.

    - Picture decoding and output

Picture n is decoded.

The variable maxPicOrderCnt is set equal to the maximum of the PicOrderCntVal values for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output".

The variable minPicOrderCnt is set equal to the minimum of the PicOrderCntVal for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output".

It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

When picture n has PicOutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )		(C-12)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture is specified as follows.

–	If PicOutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause B.1.3.

–	Otherwise (PicOutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture is output later and will be stored in the DPB (as specified in subclause B.1.3) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

When output, the picture shall be cropped, using the cropping rectangle specified in the active sequence parameter set.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of Dto,dpb( n ) is defined as:

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-13)

where nn indicates the picture that follows after picture n in output order and has PicOutputFlag equal to 1.

    - Current decoded picture marking and storage

The current decoded picture is stored in the DPB in an empty picture storage buffer, and the DPB fullness is incremented by one. If the current picture is a reference picture, it is marked as "used for reference", otherwise it is marked as "unused for reference".

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard shall fulfil the following requirements.

It is a requirement of bitstream conformance that the bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

It is a requirement of bitstream conformance that first coded picture in a bitstream shall be an IDR picture or a CRA picture.

The bitstream is tested by the HRD for conformance as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of UseInitialCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a coded video sequence conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2-9, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item j) of subclause A.3.1 or item h) of subclause A.3.3 (depending on the profile in use) but also fall within the bounds set for VCL HRD parameters for profile conformance in item i) of subclause A.3.1 or item g) of subclause A.3.3 (depending on the profile in use), conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item i) of subclause A.3.1.

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-14)

the value of UseInitialCpbRemovalDelay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

UseInitialCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-15)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= 	UseInitialCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-16)

- NOTE 2 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which tr,n( n ) is less than taf( n ). When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses A.3.1 through A.3.3.

- After the decoding process for reference picture set as specified in subclause 8.2.2 has been invoked, the number of decoded pictures in the DPB, not including the current picture, with temporal_id lower than or equal to the temporal_id of the current picture, shall be less than or equal to max_dec_pic_buffering[ temporal_id ].

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in subclause C.2.

- The value of to,dpb( n ) as given by Equation C-13, which is the difference between the output time of a picture and that of the first picture following it in output order and having pic_output_flag equal to 1, shall satisfy the constraint expressed in subclause A.3.1 for the profile and level specified in the bitstream using the decoding process specified in clauses 2-9.

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard shall fulfil the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause C.3, provided that all sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified by subclause C.3 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex A for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex A. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-17)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-18)

f( r ) = UseInitialCpbRemovalDelay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  UseInitialCpbRemovalDelay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-19)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex A for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – initial_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex A. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbSize. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is described below.

    - Operation of the output order DPB

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. At HRD initialization, the DPB is empty. The following steps all happen instantaneously when an access unit is removed from the CPB, and in the order listed.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IDR picture, the following applies.

- When the IDR picture is not the first IDR picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle changes in the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering more gracefully than the HRD.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain.

–	Otherwise (the current picture is not an IDR picture), picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). When any of the following conditions is true, the "bumping" process specified in subclause B.3.2.1 is invoked repeatedly until there is an empty picture storage buffer to store the current decoded picture.

- The number of pictures in the DPB that are marked "needed for output" is greater than num_reorder_pics[ temporal_id ],

- The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to max_dec_pic_buffering[ temporal_id ] + 1.

When the current picture is an IDR picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output).

2.	All non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause B.3.2.1.

      - "Bumping" process

The "bumping" process is invoked in the following cases.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause B.3.2.

–	The number of pictures in the DPB that are marked "needed for output" is greater than num_reorder_pics[ temporal_id ], as specified in subclause B.3.2.

–	The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to max_dec_pic_buffering[ temporal_id ] + 1, as specified in subclause B.3.2.

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

    - Picture decoding, marking and storage

The current picture is decoded and stored in an empty picture storage buffer in the DPB, and the following applies.

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

- Annex D

Supplemental enhancement information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics for SEI message payloads.

SEI messages assist in processes related to decoding, display or other purposes. However, SEI messages are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex A for the specification of conformance). Some SEI message information is required to check bitstream conformance and for output timing decoder conformance.

In Annex B.3.1, specification for presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, SEI messages shall obey the syntax and semantics specified in subclauses Error: Reference source not found and this annex. When the content of an SEI message is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the SEI message is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

### Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| cra_info_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( cra_info_present_flag ) { |  |
| initial_cra_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if ( cra _info_present_flag ) { |  |
| initial_cra_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Buffering period SEI message semantics

When NalHrdBpPresentFlag or VclHrdBpPresentFlag are equal to 1, a buffering period SEI message can be associated

with any access unit in the bitstream, and a buffering period SEI message shall be associated with each IDR access unit

and with each access unit associated with a recovery point SEI message.

NOTE – For some applications, the frequent presence of a buffering period SEI message may be desirable.

A buffering period is specified as the set of access units between two instances of the buffering period SEI message in

decoding order.

seq_parameter_set_id  specifies the sequence parameter set that contains the sequence HRD attributes. The value of

seq_parameter_set_id shall be equal to the value of seq_parameter_set_id in the picture parameter set referenced by the

primary coded picture associated with the buffering period SEI message. The value of seq_parameter_set_id shall be in

the range of 0 to 31, inclusive.

initial_cpb_removal_delay [ SchedSelIdx ] specifies the delay for the SchedSelIdx-th CPB between the time of arrival

in the CPB of the first bit of the coded data associated with the access unit associated with the buffering period SEI

message and the time of removal from the CPB of the coded data associated with the same access unit, for the first

buffering period after HRD initialisation. The syntax element has a length in bits given by initial_cpb_removal_delay_length_minus1 + 1. It is in units of a 90 kHz clock.

initial_cpb_removal_delay[ SchedSelIdx ] shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ]

BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cpb_removal_delay_offset [ SchedSelIdx ] is used for the SchedSelIdx-th CPB in combination with the

cpb_removal_delay to specify the initial delivery time of coded access units to the CPB.

initial_cpb_removal_delay_offset[ SchedSelIdx ] is in units of a 90 kHz clock. The

initial_cpb_removal_delay_offset[ SchedSelIdx ] syntax element is a fixed length code whose length in bits is given by

initial_cpb_removal_delay_length_minus1 + 1. This syntax element is not used by decoders and is needed only for the

delivery scheduler (HSS) specified in Annex C.

Over the entire coded video sequence, the sum of initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.

The semantics of the new syntax elements are as follows.

cra_info_present_flag equal to 1 indicates the presence of the initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements. cra_info_present_flag equal to 0 indicates the absence of the initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ]  syntax elements. This flag shall not be 1 when the associated picture is neither a CRA picture for which the associated leading pictures are present nor a CRA picture for which the associated leading pictures are not present.

initial_cra_cpb_removal_delay[ SchedSelIdx ] specifies a delay for the SchedSelIdx-th CPB between the time of arrival in the CPB of the first bit of the coded data associated with the access unit associated with the buffering period SEI message and the time of removal from the CPB of the coded data associated with the same access unit, for the first buffering period after HRD initialization. The syntax element has a length in bits given by

initial_cpb_removal_delay_length_minus1 + 1. It is in units of a 90 kHz clock.

initial_cra_cpb_removal_delay[ SchedSelIdx ] shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ] BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cra_cpb_removal_delay_offset [ SchedSelIdx ] is used for the SchedSelIdx-th CPB in combination with the cpb_removal_delay to specify the initial delivery time of coded access units to the CPB. initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] is in units of a 90 kHz clock. The initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] syntax element is a fixed length code whose length in bits is given by initial_cpb_removal_delay_length_minus1 + 1. This syntax element is not used by decoders and is needed only for the delivery scheduler (HSS) specified in Annex C. Over the entire coded video sequence, the sum of initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.