| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0277 WG11 Number: m24524 |
| --- | --- |

| Title: | HRD Buffering for Bitstreams Starting with CRA Picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes modification to Buffering Period SEI message and to HRD for bitstreams starting with CRA pictures when leading pictures may be present. It is asserted that the proposed HRD modification can reduce the initial buffering latency when starting playback at CRA picture. Additionally it is shown that with the proposed modification HRD will not underflow when leading pictures after CRA are discarded.

# Introduction

JCTVC-H0496 [1] proposed bitstreams starting with CRA pictures. This functionality has been incorporated in HEVC CD [2].

When random access occurs at a CRA picture, there can be leading pictures that follow the CRA picture in decoding order and precede it in output order. It is possible to discard the leading pictures following the CRA picture without affecting the operation, as the leading pictures are not specified for output and have no effect on the decoding process of any other pictures that are specified for output.

We propose modification to Buffering Period SEI message and to HRD for bitstreams starting with CRA pictures when leading pictures may be present.

The proposed HRD modification

- Can reduce the initial buffering latency when starting playback at CRA picture.

- Will guarantee that HRD will not underflow when leading pictures after CRA picture are discarded.

# Proposed Change

We propose following syntax and semantics. The proposed changes are highlighted below compared to HEVC CD [2].

## Syntax and Semantics:

### Buffering period SEI message syntax

| buffering_period( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_id | 5 | ue(v) |
| cra_leadingpict_discard_flag | 5 | u(1) |
| if( NalHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| if ( cra_leadingpict_discard_flag ) { |  |  |
| initial_cra_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| if( VclHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| if ( cra_leadingpict_discard_flag ) { |  |  |
| initial_cra_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |

### Buffering period SEI message semantics

The semantics of the new syntax elements are as follows.

cra_leadingpict_discard_flag equal to 1 indicates the presence of the initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements. cra_leadingpict_discard_flag equal to 0 indicates the absence of the initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ]  syntax elements. This flag shall not be 1 when the associated picture is not a CRA picture.

initial_cra_cpb_removal_delay[ SchedSelIdx ] specifies the delay for the SchedSelIdx-th CPB between the time of arrival in the CPB of the first bit of the coded data associated with the access unit associated with the buffering period SEI message and the time of removal from the CPB of the coded data associated with the same access unit, for the first buffering period after HRD initialization at a starting CRA picture when access units containing leading pictures associated with the starting CRA picture are discarded. The syntax element has a length in bits given by

initial_cpb_removal_delay_length_minus1 + 1. It is in units of a 90 kHz clock.

initial_cra_cpb_removal_delay[ SchedSelIdx ] shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ] ** BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cra_cpb_removal_delay_offset [ SchedSelIdx ] is used for the SchedSelIdx-th CPB in combination with the cpb_removal_delay to specify the initial delivery time of coded access units to the CPB for bitstream starting with a CRA pictures when access units containing leading pictures associated with the starting CRA picture are discarded . initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] is in units of a 90 kHz clock. The initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] syntax element is a fixed length code whose length in bits is given by initial_cpb_removal_delay_length_minus1 + 1. This syntax element is not used by decoders and is needed only for the delivery scheduler (HSS) specified in Annex C. Over the entire coded video sequence, the sum of initial_cra_cpb_removal_delay[ SchedSelIdx ] and initial_cra_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.

### C.2 Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both Type I and Type II conformance.

#### C2.1 Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages.  Prior to initialisation, the CPB is empty.

NOTE - After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

The access unit that is associated with the buffering period SEI message that initializes the CPB is referred to as access unit 0. All other access units are referred to as access unit n with n being incremented by 1 for the next access unit in decoding order.

If the first access unit is a CRA access unit, and the leading pictures are not present and the cra_leadingpict_discard_flag is equal to 1, use_initial_cpb_removal_delay[ SchedSelIdx ]  is set to value of initial_cra_cpb_removal_delay[ SchedSelIdx ], otherwise, use_initial_cpb_removal_delay[ SchedSelIdx ]  is set to value of initialcpb_removal_delay[ SchedSelIdx ].

The time at which the first bit of access unit n begins to enter the CPB is referred to as the initial arrival time tai( n ).

The initial arrival time of access units is derived as follows.

- If the access unit is access unit 0, tai( 0 ) = 0,

- Otherwise (the access unit is access unit n with n > 0), the following applies.

- If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit n, is equal to the final arrival time (which is derived below) of access unit n - 1, i.e.

tai( n ) = taf( n – 1 )		(C-1)

- Otherwise, if cbr_flag[ SchedSelIdx ] is equal to 0 and access unit n is not the first access unit of a subsequent buffering period, the initial arrival time for access unit n is derived by

tai( n ) = Max( taf( n – 1 ), tai,earliest( n ) )		(C-2)

where tai,earliest( n ) is given as follows

tai,earliest( n ) = tr,n( n ) – 
( use_initial_cpb_removal_delay[ SchedSelIdx ] + use_initial_cpb_removal_delay_offset[ SchedSelIdx ] )  90000					(C-3)

with tr,n( n ) being the nominal removal time of access unit n from the CPB as specified in subclause Error: Reference source not found and use_initial_cpb_removal_delay[ SchedSelIdx ] and use_initial_cpb_removal_delay_offset[ SchedSelIdx ] being specified as described above based on the previous buffering period SEI message.

- Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0 and the subsequent access unit n is the first access unit of a subsequent buffering period), the initial arrival time for the access unit n is derived by

tai( n ) = tr,n( n ) – ( use_initial_cpb_removal_delay[ SchedSelIdx ]  90000 )	(C-4)

with use_initial_cpb_removal_delay[ SchedSelIdx ] being specified as described above based on the buffering period SEI message associated with access unit n.

The final arrival time for access unit n is derived by

taf( n ) = tai( n ) + b( n )  BitRate[ SchedSelIdx ]		(C-5)

where b( n ) is the size in bits of access unit n, counting the bits of the Type I bitstream for Type I conformance or the bits of the Type II bitstream for Type II conformance.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows.

- If access unit n and access unit n - 1 are part of different coded video sequences and the content of the active sequence parameter sets of the two coded video sequences differ, the HSS may select a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided for the coded video sequence containing access unit n that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the second of the two coded video sequences (which contains access unit n – 1) that differs from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx  that was in use for the coded video sequence containing access unit n - 1.

- Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies.

- the variable BitRate[ SchedSelIdx ] comes into effect at time tai( n )

- the variable CpbSize[ SchedSelIdx ] comes into effect as follows.

- If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( n ),

- Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the time tr( n ).

#### C2.2 Timing of coded picture removal

It is assumed that the nominal CPB removal time and the CPB removal time of a coded picture are calculated immediately after the previous coded picture is removed from the CPB, or, for access unit 0, when the HRD is initialised.

If the first access unit is a CRA access unit, and the leading pictures are not present and the cra_leadingpict_discard_flag is equal to 1, then for access unit 0, the nominal removal time of the access unit from the CPB is specified by

tr,n( 0 ) = initial_cra_cpb_removal_delay[ SchedSelIdx ] 90000

Else for access unit 0, the nominal removal time of the access unit from the CPB is specified by

tr,n( 0 ) = initial_cpb_removal_delay[ SchedSelIdx ] 90000	(C-6)

For the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the access unit from the CPB is specified by

tr,n( n ) = tr,n ( nb ) + tc * cpb_removal_delay( n )		(C-7)

where tr,n( nb ) is the nominal removal time of the first picture of the previous buffering period and cpb_removal_delay( n ) is specified in the picture timing SEI message associated with access unit n.

When an access unit n is the first access unit of a buffering period, nb is set equal to n at the removal time tr,n( n ) of access unit n.

The nominal removal time tr,n(n) of an access unit n that is not the first access unit of a buffering period is given by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-8)

The removal time of access unit n is specified as follows.

- If low_delay_hrd_flag is equal to 0 or tr,n( n ) >= taf( n ), the removal time of access unit n is specified by

tr( n ) = tr,n( n )			(C-9)

- Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), the removal time of access unit n is specified by

tr( n ) = tr,n( n ) + tc * Ceil( ( taf( n ) - tr,n( n ) ) tc ) 		(C-10)

NOTE – The latter case indicates that the size access unit n, b(n), is so large that it prevents removal at the nominal removal time.

#### C.4 Bitstream conformance

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-11)

the value of use_initial_cpb_removal_delay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

use_initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-12)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= use_	initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-13)

- NOTE 2 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

# Results

We illustrate an example of how use of Buffering Period SEI message parameters from CD (without our proposed modification) can result in a underflow of HRD when leading pictures are discarded. We encoded 500 frames of the video sequence BasketBallPass using HM6.1 RA main configuration (encoder_randomaccess_main.cfg ) with CRA frames at random access points at every 32 frames. We then decoded the bitstream by performing random access at frame 64.

We simulated two scenarios:

- Case1: All pictures starting at CRA random access point including subsequent leading pictures are transmitted by the server. The bitstream is decoded using HRD parameters from the corresponding Buffering Period SEI message as in current HEVC CD (CD Method) for this random access point. Figure 1 shows the HRD buffer occupancy in this case for one leaky bucket rate.

Figure 1: Buffer occupancy of HRD with leading pictures present (CD Method)

- Case2: Pictures starting at CRA random access point are transmitted by the server but the leading pictures subsequent to the CRA picture are discarded. The bitstream is decoded using HRD parameters from the corresponding HEVC CD Buffering Period SEI message as above (i.e. without our proposed method) for this random access point (CD Method). Figure 2 shows the HRD buffer occupancy in this case. As can be observed in this case the HRD buffer underflows.

Figure 2: Buffer occupancy of HRD with leading pictures discarded (CD Method)

The bitstream is decoded using HRD parameters from Buffering Period SEI message with our proposed method for this random access point (Proposed Method). Figure 3 shows the HRD buffer occupancy in this case for the same leaky bucket rate as in Figure 2. As can be observed in this case the HRD buffer underflow is eliminated using our proposed method.

Figure 3: Buffer occupancy of HRD with leading pictures discarded (Proposed Method)

# Conclusion

The document proposed syntax and semantics to modify the Buffering Period SEI message for bitstreams starting with CRA pictures when leading pictures are present. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Y.-K. Wang, Y. Checn, M. Karczewicz, J. Chen, "On bitstreams starting with CRA pictures," JCTVC-H0496, 8th Meeting: San Jose, USA, 1-10 February, 2012.

[2] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003_dK, San Jose, February, 2012.