| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0278 WG11 Number: m24525 |
| --- | --- |

| Title: | Signaling of CRA Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Louis Kerofsky 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes methods for signaling of CRA pictures. The proposed methods distinguish between: CRA pictures with subsequent leading pictures and CRA pictures without subsequent leading pictures. The CRA pictures without subsequent leading pictures can occur when a server or network node discards the leading pictures following the corresponding CRA picture. It is asserted that the proposed approach allows the decoder to know at the time of decoding a CRA picture if there are going to be any leading pictures arriving in the bitstream following this CRA picture.

# Introduction

JCTVC-H0496 [1] proposed bitstreams starting with CRA pictures. This functionality has been incorporated in HEVC CD [2].

We propose methods for signaling of CRA pictures. The proposed methods can be used to distinguish between: CRA pictures with subsequent leading pictures and CRA pictures without subsequent leading pictures. The CRA pictures without subsequent leading pictures can occur when a server or network node discards the leading pictures following the corresponding CRA picture. The proposed approach can allow the decoder to know at the time of decoding a CRA picture if there are going to be any leading pictures arriving in the bitstream following this CRA picture.

The proposed signaling can allow the decoder to decide earlier (at the time of decoding the CRA picture) the correct HRD parameters to use based on presence or absence of subsequent leading pictures following the CRA picture.

# Proposed Change

We propose following two signaling methods to indicate whether a CRA picture has or does not have following leading pictures:

- Proposal 1: A new NAL Unit type for CRA picture to indicate that there are no leading pictures following the CRA picture.

- Proposal 2: A flag in the Slice header for the CRA picture to indicate if leading pictures are present following the CRA picture or not.

We propose following syntax and semantics for the two proposals. The proposed changes are highlighted below compared to HEVC CD [2].

## Syntax and Semantics:

Proposal 1:  A New NALU Type

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture with subsequent leading pictures slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 16 | Coded slice of a CRA picture with no subsequent leading pictures slice_layer_rbsp( ) | VCL |
| 17-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

NAL units that use nal_unit_type equal to 0 or in the range of 24..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 24..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 24..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

In the text, coded slice NAL unit collectively refers to a coded slice of a non-IDR picture NAL unit or to a coded slice of an IDR picture NAL unit. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  5 )  ?  1  :  0 )		(7-1)

When the value of nal_unit_type is equal to 3 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 3. Such a picture is referred to as a TLA picture.

When the value of nal_unit_type is equal to 4 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 4. Such a picture is referred to as a CRA picture with subsequent leading pictures present following this CRA picture.

When the value of nal_unit_type is equal to 5 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 5. Such a picture is referred to as an IDR picture.

When the value of nal_unit_type is equal to 16 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 16. Such a picture is referred to as a CRA picture with no subsequent leading pictures present following this CRA picture.

Proposal 2: A New Flag

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | u(v) |
| cra_leadingpict_present_flag | u(1) |
| … |  |
| } |  |

cra_leadingpict_present_flag equal to 1 indicates that there are leading pictures present in the bitstream following this CRA picture. cra_leadingpict_present_flag equal to 0 indicates that there are no leading pictures present in the bitstream following this CRA picture.

NOTE  – A server could discard the leading pictures following a CRA picture and set the value of cra_leadingpict_present_flag to zero.

The above table shows one possible location in slice header for this flag. The flag can be sent at a different location in slice header and could for example be included only for the first slice in the picture or could be only included in slice header for CRA pictures.

# Conclusion

The document proposed methods for signaling of CRA pictures. The proposed methods can be used to distinguish between: CRA pictures with subsequent leading pictures and CRA pictures without subsequent leading pictures. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Y.-K. Wang, Y. Chen, M. Karczewicz, J. Chen, "On bitstreams starting with CRA pictures," JCTVC-H0496, 8th Meeting: San Jose, USA, 1-10 February, 2012.

[2] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003_dK, San Jose, February, 2012.