| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011 | Document: JCTVC-H1003 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 6 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 6 of High efficiency video coding.

## Semantics

Semantics associated with the syntax structures and with the syntax elements within these structures are specified in this subclause. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

### NAL unit semantics

NOTE 1 – The VCL is specified to efficiently represent the content of the video data. The NAL is specified to format that data and provide header information in a manner appropriate for conveyance on a variety of communication channels or storage media. All data are contained in NAL units, each of which contains an integer number of bytes. A NAL unit specifies a generic format for use in both packet-oriented and bitstream systems. The format of NAL units for both packet-oriented transport and byte stream is identical except that each NAL unit can be preceded by a start code prefix and extra padding bytes in the byte stream format.

NumBytesInNALunit specifies the size of the NAL unit in bytes. This value is required for decoding of the NAL unit. Some form of demarcation of NAL unit boundaries is necessary to enable inference of NumBytesInNALunit. One such demarcation method is specified in Annex Error: Reference source not found for the byte stream format. Other methods of demarcation may be specified outside of this Recommendation | International Standard.

forbidden_zero_bit shall be equal to 0.

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture.

For coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

nal_ref_flag shall be equal to 1 for sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one NAL unit with nal_unit_type equal to 1 or 4 of a particular picture, it shall be equal to 0 for all NAL units with nal_unit_type equal to 1 or 4 of the picture.

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 5.

nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 6, 9, 10, 11, or 12.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that use nal_unit_type equal to 0 or in the range of 24..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 24..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 24..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 24 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture with subsequent leading pictures slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 16 | Coded slice of a CRA picture with no subsequent leading pictures slice_layer_rbsp( ) | VCL |
| 17-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

In the text, coded slice NAL unit collectively refers to a coded slice of a non-IDR picture NAL unit or to a coded slice of an IDR picture NAL unit. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  5 )  ?  1  :  0 )		(7-1)

When the value of nal_unit_type is equal to 3 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 3. Such a picture is referred to as a TLA picture.

When the value of nal_unit_type is equal to 4 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 4. Such a picture is referred to as a CRA picture with subsequent leading pictures present following this CRA picture.

When the value of nal_unit_type is equal to 5 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 5. Such a picture is referred to as an IDR picture.

When the value of nal_unit_type is equal to 16 for a NAL unit containing a slice of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to 4. Such a picture is referred to as a CRA picture with no subsequent leading pictures present following this CRA picture.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| cra_leadingpict_present_flag | u(1) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, idr_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding treeblock in the picture.

slice_address specifies the address in slice granularity resolution in which the slice starts. The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

The variable SliceCtbAddrRS, specifying the coding treeblock in which the slice starts in coding treeblock raster scan order, is derived as follows.

SliceCtbAddrRS = ( slice_address >> SliceGranularity )	(7-2)

The variable SliceCbAddrZS, specifying the address of first coding block in the slice in minimum coding block granularity in z-scan order, is derived as follows.

SliceCbAddrZS = slice_address	<< ( ( log2_diff_max_min_coding_block_size − SliceGranularity ) <<1 )	(7-3)

The slice decoding starts with the largest coding unit possible at the slice starting coordinate.

slice_type specifies the coding type of the slice according to Table 7-2.

Table 7-2 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - P (P slice) |
| - 1 | - B (B slice) |
| - 2 | - I (I slice) |

When nal_unit_type is equal to 5 (IDR picture), slice_type shall be equal to 2.

When max_dec_pic_buffering[ max_temporal_layers_minus1 ] is equal to 0, slice_type shall be equal to 2.

cra_leadingpict_present_flag equal to 1 indicates that there are leading pictures present in the bitstream following this CRA picture. cra_leadingpict_present_flag equal to 0 indicates that there are no leading pictures present in the bitstream following this CRA picture.

NOTE  – A server could discard the leading pictures following a CRA picture and set the value of cra_leadingpict_present_flag to zero.