| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0279 |
| --- | --- |

| Title: | Proposed modifications of explicit weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In the current HEVC text, there are some issues on the case of the combination with the explicit weighted prediction (WP) and reference list combination process, which means WP parameter signalling and those mapping process for list Lx or LC when ref_pic_list_combination_flag is 0 or 1. In this contribution, the proposed modifications of the explicit WP are proposed with the goal to fix some remaining issues in the current HEVC text. Those modifications do not affect the experimental results of WP scheme on the common test conditions.

# Problem statement

At 6th JCT-VC Torino meeting, AVC based weighted prediction (AVCWP) scheme [1][2] was adopted in HM4 and WD4 [4]. At 7th JCT-VC meeting, the redundancy reduction of WP parameter signalling Error: Reference source not found was adopted to combine the signalling of explicit WP parameters for different reference picture lists in B slices. This scheme can remove redundant signalling of WP parameters by using the scheme of the combined reference list management scheme [3]. When ref_pic_list_combination_flag is 1, WP parameters are signalled according to the LC indices, and the weighting factors and offsets used in bi-prediction are derived by mapping a given L0 or L1 index to the corresponding LC index and retrieving the WP parameters sent for that LC index. However, when ref_pic_list_combination_flag is equal to 0, WP parameters are signalled for L0 and L1 indices but LC indices. There are some issues of the current scheme as follows:

- When ref_pic_list_combination_flag is 0, WP parameters for both L0 and L1 indices are signalled similar as H.264 WP. However, at that case list L0 and L1 are identical; we don’t need to signal the WP parameter for list L1.

- When ref_pic_list_combination_flag is 1, in the current HEVC text, the ref_pic_list_combination() process allows list LC to be smaller than the combination of list L0 and list L1 (That case is not defined in common test conditions [6]). At that case, since WP parameters signalling in current text of explicit weighted prediction is not fully defined, some WP parameters may be missing entries for L0 and L1 indices.

- There are some editorial issues on the decoding process in 8.5.2.2.3.2 weighted sample prediction process. It uses the array RefIdxLCToRefIdxLx[refIdxL0/refIdxL1] to map L0 or L1 index to LC index and to find the corresponding WP parameters. However, this mapping is not appropriate, since RefIdxLCToRefIdxLx[ ] maps LC index to an L0 or L1 index, not in the reverse direction. Then, additional arrays RefIdxLxToRefIdxLC[ ] must be defined in order to make the current decoding process in 8.5.2.2.3.2 work correctly.

# Proposal

In order to fix these issues, this contribution proposes revised text of explicit weighted prediction. Proposed syntax, semantics and the decoding process are shown in Appendix.

To fix the point 1, the WP parameters for list L1 in B-slices are unified to the WP parameters for list LC. If num_ref_idx_lc_active_minus1 is not present, which means ref_pic_list_combination_flag is 0, this value is inferred to be (num_ref_idx_l0_active_minus1), because at that case, reference picture list l0 and reference picture list l1 are identical and reference picture list l0 is used as the combined reference picture list. By introducing this modification, the pred_weight_table syntax can be simplified.

To fix the point 2, the revised decoding process in subclause 8.5.2.2.3.2 and subclause 8.3.4.3 are proposed. If some missing entries of WP parameters are happened, the default WP parameters are filled in that reference picture. When we’d like to use the arbitrary WP parameters for all reference pictures, then we set an appropriate value of num_ref_idx_lc_active_minus1 to cover all list L0 and L1 entries or we may set ref_pic_list_combination_flag to 0. The default WP parameters are defined as follows;

wXC = ( 2 << logWDc )

oXC = 0

where, wXC is the default weighting factor and oXC is the default offset.

To fix the point 3, the new array RefIdxL0ToRefIdxLC and RefIdxL0ToRefIdxLC are introduced in subclause 8.3.4.3. Using those new arrays, weighted sample prediction process in subclause 8.5.2.2.3.2 is modified.

The full version of the proposed modifications in the latest HEVC WD6 text is attached in this contribution.

# Conclusion

In this contribution, the modifications of explicit weighted prediction to solve some issues are provided. Those modifications do not affect current simulation results of the weighted prediction scheme based on common test conditions, because ref_pic_list_combination_flag is always equal to 1 for B-slices on those conditions and the reference pictures for LC is same entries as those for L0 and L1. To introduce these modifications, some issues on the current text of weighted prediction can be fixed.

It is suggested that this revised text is reflected in the next version of HEVC text.

# References

- P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

- A. Tanizawa, "Explicit Weighted Prediction with simple WP parameter estimation," Joint Collaborative Team on Video Coding, JCTVC-F326, Torino, July 2011.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD6: Working Draft 6 of High-Efficiency Video Coding”, Document no JCTVC-H1003. February 2012.

- Y. Suzuki and TK Tan, " Extension of uni-prediction simplification in B slices," Joint Collaborative Team on Video Coding, JCTVC-D421, Daegu, Jan. 2011.

- A. Tanizawa, T. Chujoh, T. Yamakage, “Redundancy removal of explicit weighted prediction syntax”, Document no JCTVC-G441, Nov 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-G1200. November 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed weighted prediction syntax change

Table 1. pred_weight_table() syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ ( slice_type = = B && ref_pic_list_combination_flag = = 0 ) ) { |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| if( ref_pic_list_combination_flag = = 0 ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| luma_weight_lc_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_lc[ i ] | se(v) |
| luma_offset_lc[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_lc_flag | u(1) |
| if( chroma_weight_lc_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_lc[ i ][ j ] | se(v) |
| delta_chroma_offset_lc[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed weighted prediction semantics change

#### 7.4.3.3. Reference picture lists combination semantics

ref_pic_list_combination_flag equal to 1 indicates that the reference picture list 0 and the reference picture list 1 are combined to be an additional reference picture lists combination used for the prediction units being uni-directional predicted. This flag equal to 0 indicates that the reference picture list 0 and reference picture list 1 are identical thus reference picture list 0 is used as the reference picture lists combination. The reference picture lists combination is set to be empty at the start of the loop defined in this table.

num_ref_idx_lc_active_minus1+1 specifies the number of reference pictures selected from reference picture list 0 or reference picture list 1 in the reference picture lists combination. When num_ref_idx_lc_active_minus1 is not present, it is inferred to be num_ref_idx_l0_active_minus1.

#### 7.4.3.8. Weighted prediction parameters semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag is equal to 1, the value of LumaWeightL0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag is equal to 1, the value of ChromaWeightL0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )	 (7-1)

The variable ChromaOffsetL0[ i ][ j ] shall be in the range of −127 to 128, inclusive. When chroma_weight_l0_flag is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

luma_weight_lc_flag, delta_luma_weight_lc, luma_offset_lc, chroma_weight_lc_flag, delta_chroma_weight_lc, delta_chroma_offset_lc have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by lc, list combination, and List combination, respectively.

# Proposed decoding process change

- 8.3.4.3. Mapping process for reference picture lists combination in B slices

This initialization process is invoked when decoding a B slice header.

Input to this process are the reference picture list RefPicListX and num_ref_idx_lX_active_minus1 with X being 0 or 1.

Outputs of this process are arrays PredLCToPredLx, RefIdxLCToRefIdxLx, RefIdxL0ToRefIdxLC and RefIdxL1ToRefIdxLC.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 0, the following ordered steps apply:

      - Let refIdxL0 and refIdxL1 be indices into the reference picture lists RefPicListL0 and RefPicListL1. They are initially set equal to 0.

      - Let refIdxLC be an index into PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

      - All elements of arrays RefIdxL0ToRefIdxLC and RefIdxL1ToRefIdxLC are initially set equal to -1.

      - The following process is repeated until refIdxL0 and refIdxL1 are both greater than num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively:

–	If refIdxL0 is less than or equal to num_ref_idx_l0_active_minus1,

–	If the entry RefPicListL0[ refIdxL0 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-2)

RefIdxL0ToRefIdxLC[ refIdxL0 ] = refIdxLC,

RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL0.

–	refIdxL0++.

–	If refIdxL1 is less than or equal to num_ref_idx_l1_active_minus1 and ref_pic_list_combination_flag equal to 1,

–	If the entry RefPicListL1[ refIdxL1 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L1, 	(8-3)

RefIdxL1ToRefIdxLC[ refIdxL1 ] = refIdxLC,

RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL1.

–	refIdxL1++.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 1, the following ordered steps apply:

- Let refIdxLC be an index into the reference picture list PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

- All elements of arrays RefIdxL0ToRefIdxLC and RefIdxL1ToRefIdxLC are initially set equal to -1.

- The corresponding syntax elements pic_from_list_0_flag and ref_idx_list_curr are processed in the order they occur in the bitstream. For each of these syntax elements pairs, the following applies.

–	If pic_from_list_0_flag is equal to 1,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 		(8-4)

RefIdxL0ToRefIdxLC[ ref_idx_list_curr ] = refIdxLC.

–	Otherwise,

PredLCToPredLx[ refIdxLC ] = Pred_L1		(8-5)

RefIdxL1ToRefIdxLC[ ref_idx_list_curr] = refIdxLC.

–	RefIdxLCToRefIdxLx[ refIdxLC++ ] = ref_idx_list_curr,

When refIdxLC is greater than num_com_ref_list_active_minus1+ 1, the extra entries past position num_com_ref_list_active_minus1 are discarded from PredLCToPredLx and RefIdxLCToRefIdxLx.

When refIdxLC is less than num_com_ref_list_active_minus1 + 1, the remaining entries in PredLCToPredLx and RefIdxLCToRefIdxLx are set equal to Pred_L0 and 0, respectively.

###### 8.5.2.2.3.2 Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction logWDC, logWDC, w0C, w1C, o0C, o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)−1 and y = 0..(nPSH)−1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * w0C + 2logWDC − 1 ) >> logWDC ) + o0C ) 	 (8-6)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * w0C + o0C ) 	(8-7)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * w1C + 2logWDC − 1 ) >> logWDC ) + o1C ) 	(8-8)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * w1C + o1C ) 	(8-9)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * ( w0C + w1C ) + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-10)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * w0C + predSamplesL1 [ x, y ] * w1C + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-11)

Where the variables logWDC, o0C, o1C, and w0C, w1C are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

logWDc = 5+shift1		(8-12)

o0C = 0				(8-13)

o1C = 0				(8-14)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-15)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-16)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-17)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-18)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

w0C=32				(8-19)

w1C=32				(8-20)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)		(8-21)

w1C = WeightScaleFactor >> 2		(8-22)

–	Otherwise, if weighted_bipred_idc are equal to 1 in B slice, combined explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-23)

  - If ( RefIdxL0ToRefIdxLC[ refIdxLC ] >= 0 )

w0C = LumaWeightLC[ RefIdxL0ToRefIdxLC[ refIdxLC ] ]	(8-24)

o0C = luma_offset_lc[ RefIdxL0ToRefIdxLC[ refIdxLC ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-25)

  - Otherwise, ( RefIdxL0ToRefIdxLC[ refIdxLC ] <0 )

w0C = ( 2 << logWDc )		(8-26)

o0C = 0		(8-27)

  - If ( RefIdxL1ToRefIdxLC[ refIdxLC ] >= 0 )

w1C = LumaWeightLC[ RefIdxL1ToRefIdxLC[ refIdxLC ] ] 	(8-28)

o1C = luma_offset_lc[ RefIdxL1ToRefIdxLC[ refIdxLC ] ] * ( 1 << ( BitDepthY − 8 ) ) 	(8-29)

  - Otherwise, ( RefIdxL1ToRefIdxLC[ refIdxLC ] <0 )

w1C = ( 2 << logWDc )		(8-30)

o1C = 0		(8-31)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-32)

  - If ( RefIdxL0ToRefIdxLC[ refIdxLC ] >= 0 )

w0C = ChromaWeightLC[ RefIdxL0ToRefIdxLC[ refIdxLC ] ][ iCbCr ]	(8-33)

o0C = ChromaOffsetLC[ RefIdxL0ToRefIdxLC[ refIdxLC ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-34)

  - Otherwise, ( RefIdxL0ToRefIdxLC[ refIdxLC ] <0 )

w0C = ( 2 << logWDc )		(8-35)

o0C = 0		(8-36)

  - If ( RefIdxL1ToRefIdxLC[ refIdxLC ] >= 0 )

w1C = ChromaWeightLC[ RefIdxL1ToRefIdxLC[ refIdxLC ] ][ iCbCr ]	(8-37)

o1C = ChromaOffsetLC[ RefIdxL1ToRefIdxLC[ refIdxLC ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-38)

  - Otherwise, ( RefIdxL1ToRefIdxLC[ refIdxLC ] <0 )

w1C = ( 2 << logWDc )		(8-39)

o1C = 0		(8-40)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-41)

w0C = LumaWeightL0[refIdxL0]		(8-42)

o0C = luma_offset_l0[refIdxL0] * ( 1 << ( BitDepthY − 8 ) )	(8-43)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-44)

w0C = ChromaWeightL0[refIdxL0][ iCbCr ]		(8-45)

o0C = ChromaOffsetL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-46)