| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0280_r1 |
| --- | --- |

| Title: | AHG6: Quantization Matrices and Deblocking Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qualcomm.com rajanj@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

HEVC includes the signalling of quantization matrices or scaling lists in the APS. These matrices can significantly modify the QP values used by the scaling process for various use cases. On the other hand, the HEVC deblocking filter determines its filtering strength based on QP values without considering the potentially significant QP changes introduced by the quantization matrices, which can lead to either too weak or too strong deblocking strength. Therefore, it is proposed to provide signalling of one “equivalent QP change” per signalled quantization matrix in order to appropriately modify the deblocking strength. The method to determine the equivalent QP change is encoder-side only and can be optimized together with the quantization matrices. The concept is illustrated and the BD-rate results for the default quantization matrices are studied. The objective quality and intended subjective quality of the default matrices is unaffected by the proposed method.

# Problem Statement

The HEVC committee draft (CD) [1] supports the signalling of scaling lists or quantization matrices (Q-matrices) in the APS per transform size, color component, and intra/inter mode. Let M[i][j] represent elements from the array corresponding to one of the Q-matrices. The typical value range of M[i][j] is 1 to 255 (8-bit) with the value 16 representing no change to the quantization for frequency index (i,j). Table 1 enumerates the equivalent QP change in case the normalized value of M[i][j] / 16 doubles or halves, which corresponds to the doubling or halving of the quantization step size, or equivalently, a QP change of +6 or -6. Intermediate values are also allowed.

Table 1 Illustration of QP change as a function of the doubling or halving of the Q-matrix M[i][j] element.

| M[i][j] / 16 | QP Change |
| --- | --- |
| 1/16 | -24 |
| 1/8 | -18 |
| 1/4 | -12 |
| 1/2 | -6 |
| 1 | 0 |
| 2 | +6 |
| 4 | +12 |
| 8 | +18 |
| 16 | +24 |

Any Q-matrix can be signalled that significantly modifies the QP per transform size, per frequency, etc. The problem is that the HEVC deblocking filter [1] is not aware of the QP change introduced by a Q-matrix. Therefore, deblocking filtering strength for luma or chroma blocking artifact edges can be too weak or too strong.

# Proposal

To adjust the deblocking filter strength for the current edge in between two blocks P and Q based on Q-matrix QP changes for blocks P and Q, the deblocking filter first needs to determine the sizes of the TUs that contain the two blocks, as well as the color component type and intra or inter mode. Based on this information, the Q-matrices that correspond with blocks P and Q are determined.

In this proposal, the encoder computes an “equivalent QP change” per Q-matrix that is signalled in the APS and updates the equivalent QP changes when Q-matrices are modified through signalling. This is an encoder-side only method; hence, the encoder has the freedom to optimize the equivalent QP changes. In this proposal, the average of the M[i][j] array elements (low-frequency quadrant) is computed for each Q-matrix and subsequently, the equivalent QP change is determined by using a lookup table, illustrated in Table 2 (encoder-side table only). This results in one equivalent QP change value per Q-matrix, for a total of 20 values. The deblocking filter will adjust the deblocking parameters (table look-up of β and tC parameters) based on the equivalent QP changes for blocks P and Q.

Table 2 Encoder-side only table, as an illustration of deriving an equivalent QP change (QPQM) based on the average of the QM[i][j] array elements.

| Avg QM[i][j] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPQM | -24 | -18 | -14 | -12 | -10 | -8 | -7 | -6 | -5 | -4 | -3 | -2 | -2 | -1 | -1 | 0 |
| Avg QM[i][j] | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| QPQM | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 6 |
|  | … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avg QM[i][j] | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| QPQM | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Avg QM[i][j] | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |
| QPQM | 23 | 23 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |

The equivalent QP changes are signaled together with the Q-matrices in the APS. The high-level syntax changes for the APS are illustrated in Table 3. The semantics of the introduced syntax elements is as follows:

scaling_list_deblocking_flag equal to 1 specifies that scaling list parameters for deblocking filter adjustment are present in the APS, while value 0 specifies that no parameters are present and that the deblocking filter shall infer all scaling_list_deblocking[sizeID][matrixID] parameters equal to 0

scaling_list_deblocking[sizeID][matrixID] is equal to the scaling list parameter for deblocking filter adjustment corresponding to sizeID and matrixID

Table 3 scaling_list_param syntax (APS) and signalling of proposed equivalent QP changes per Q-matrix for deblocking strength adjustment

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| scaling_list_deblocking_flag | u(1) |
| if ( scaling_list_deblocking_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_deblocking[ sizeID ][ matrixID ] | se(v) |
| } |  |
| } |  |

The deblocking filter in encoder and decoder determines what equivalent QP change, QPQM, to apply to the QPY values of the P and Q blocks using the signalled parameters (scaling_list_deblocking[sizeID][matrixID]). The QPL value for deblocking a luma edge is computed as follows:

QPL = ( QPY,P + QPQM,P + QPY,Q + QPQM,Q + 1 ) / 2

Similarly, for a chroma edge:

QPC = ( QPCr,P + QPQM,P + QPCr,Q + QPQM,Q + 1 ) / 2

QPC = ( QPcb,P + QPQM,P + QPCb,Q + QPQM,Q + 1 ) / 2

With QPCr = QP_UV( QPY ) and QPCb = QP_UV( QPY ). QP_UV() is the function or table that converts the luma QPY into QPCr and QPCb for chrominance.

As an illustration of the concept, Fig. 1 depicts the BasketballDrill sequence coded with HM6.0 with the All-Intra test conditions and QP=13, while the Q-matrices are configured to result in a QP change of +24. Therefore, the actual QP used for coding is 37, while the HM6.0 deblocking filter strength is configured for QP=13. Fig. 2 illustrates the proposed approach where the deblocking filter is aware of the equivalent QP changes and is configured with the correct deblocking strength, which is QP=37.

Figure 1 HM6.0, AI-Main, QP=13, frame 1. Coding is performed with QP=37, while deblocking filter is configured with strength based on QP=13.

Figure 2 Proposed, AI-Main, QP=13+24=37 (frame 1) for coding and for deblocking filter strength

# Objective Results

The experiments are performed according to the common test conditions described in JCTVC-H1100, with the default Q-matrices enabled in the configuration files (ScalingList=1). First, the BD-rate impact of the default Q-matrices is compared with the HM6.0 anchor in Table 4. The encoding times  are measured in a cluster computing environment and are approximate only. The decoding times are measured on a single CPU.

Table 4

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.7% | 1.3% | 0.1% | -0.4% | 0.2% |
| Class B | 2.3% | -2.7% | -2.7% | 1.7% | -3.4% | -3.5% |
| Class C | 4.5% | -1.9% | -2.3% | 3.4% | -3.4% | -4.0% |
| Class D | 5.2% | -3.5% | -2.6% | 4.3% | -4.4% | -3.8% |
| Class E | 1.1% | -1.6% | -1.7% | 0.5% | -1.9% | -1.8% |
| Class F | 6.1% | 0.1% | 2.5% | 5.4% | -2.3% | 0.0% |
| Overall | 3.3% | -1.5% | -1.0% | 2.6% | -2.7% | -2.2% |
|  | 3.2% | -1.4% | -0.9% | 2.5% | -2.6% | -2.1% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 3.1% | 4.8% | -0.2% | 1.7% | 3.0% |
| Class B | 0.7% | -2.1% | -2.5% | 0.4% | -2.3% | -2.8% |
| Class C | 3.7% | 0.3% | -0.5% | 3.4% | -0.4% | -1.1% |
| Class D | 3.9% | -2.4% | -2.3% | 3.8% | -2.4% | -2.3% |
| Class E |  |  |  |  |  |  |
| Class F | 6.8% | 4.2% | 7.3% | 6.3% | 2.4% | 5.0% |
| Overall | 2.9% | 0.5% | 1.2% | 2.7% | -0.3% | 0.2% |
|  | 3.0% | 0.6% | 1.2% | 2.7% | -0.2% | 0.2% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | -3.5% | -3.9% | 0.1% | -3.1% | -3.4% |
| Class C | 2.4% | -2.0% | -2.5% | 1.8% | -2.0% | -2.3% |
| Class D | 2.3% | -4.0% | -4.1% | 1.7% | -4.2% | -4.1% |
| Class E | 1.4% | -0.4% | -0.7% | 0.9% | -0.1% | -0.8% |
| Class F | 4.8% | 1.5% | 5.1% | 4.1% | 0.1% | 5.8% |
| Overall | 2.2% | -1.8% | -1.4% | 1.7% | -2.0% | -1.1% |
|  | 2.2% | -1.7% | -1.2% | 1.6% | -2.0% | -1.1% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -3.3% | -4.0% | 0.0% | -3.1% | -3.6% |
| Class C | 2.5% | -2.3% | -2.6% | 2.0% | -2.1% | -2.5% |
| Class D | 2.6% | -4.0% | -4.3% | 2.2% | -3.6% | -3.6% |
| Class E | 1.5% | -0.2% | -1.0% | 1.0% | 0.3% | 0.3% |
| Class F | 5.0% | 2.3% | 5.3% | 4.1% | 1.1% | 4.1% |
| Overall | 2.3% | -1.6% | -1.5% | 1.8% | -1.6% | -1.3% |
|  | 2.2% | -1.6% | -1.4% | 1.7% | -1.7% | -1.2% |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Second, in Table 5 the proposed deblocking filter Q-matrix adaptation is enabled based on the equivalent QP changes introduced by the default Q-matrices and its BD-rate efficiency is compared with HM6.0 with default Q-matrices enabled (Table 4 results). There is no observable BD-rate difference introduced by the proposed deblocking filter adaptation for the default Q-matrices and there is no subjective quality difference introduced for these matrices in order to preserve their intended subjective improvements.

Table 5

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# Conclusion

Quantization matrices can substantially change the QP values used by the coding process, while the HEVC deblocking filter does not adjust its filtering strength based on the QP values that are modified by quantization matrices. It is proposed to signal one equivalent QP change per quantization matrix in the APS, to be used by the deblocking filter to adjust the filtering strength appropriately. Optimally determining the equivalent QP changes is an encoder-only process.

# References

[1] Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, CA, USA, Feb. 2012

# HEVC Draft Text Changes

Table 3 above represents the changes to the scaling_list_param( ) syntax in the APS (see highlights). The semantics of the proposed syntax elements is repeated for convenience:

scaling_list_deblocking_flag equal to 1 specifies that scaling list parameters for deblocking filter adjustment are present in the APS, while value 0 specifies that no parameters are present and that the deblocking filter shall infer all scaling_list_deblocking[sizeID][matrixID] parameters equal to 0

scaling_list_deblocking[sizeID][matrixID] is equal to the scaling list parameter for deblocking filter adjustment corresponding to sizeID and matrixID

The following HEVC draft text changes are based on the document attached to HM Text Ticket #492.

##### 8.7.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

If verticalEdgeFlag is equal to 1, the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +i, yC + yB + k ]

pi,k = recPictureL[ xC + xB – i − 1, yC + yB + k ]

Otherwise (verticalEdgeFlag is equal to 0), the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +k, yC + yB + i ]

pi,k = recPictureL[ xC + xB +k, yC + yB – i − 1 ]

The variables log2TrafoWidthQ and log2TrafoHeightQ are set equal to the width and the height of the transform unit that includes the luma sample value q0,0. The variable log2TrafoSizeQ is set equal to (log2TrafoWidthQ + log2TrafoHeightQ) / 2. The variable SizeIDQ is set equal to (log2TrafoSizeQ – 2).

The variables log2TrafoWidthP and log2TrafoHeightP are set equal to the width and the height of the transform unit that includes the luma sample value p0,0. The variable log2TrafoSizeP is set equal to (log2TrafoWidthP + log2TrafoHeightP) / 2. The variable SizeIDP is set equal to (log2TrafoSizeP – 2).

If the luma sample q0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDQ is set equal to 0; otherwise, if SizeIDQ is equal to 3, then MatrixIDQ is set equal to 1, otherwise 3.

If the luma sample p0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDP is set equal to 0; otherwise, if SizeIDP is equal to 3, then MatrixIDP is set equal to 1, otherwise 3.

The variable QPQ,QM is set equal to scaling_list_deblocking[SizeIDQ][MatrixIDQ].

The variable QPP,QM is set equal to scaling_list_deblocking[SizeIDP][MatrixIDP].

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

The variable QPQ is set equal to the QPY value of the coding unit containing the sample q0,0 plus the variable QPQ,QM.

The variable QPP is set equal to the QPY value of the coding unit containing the sample p0,0 plus the variable QPP,QM.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	When pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- When pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

….

##### 8.7.1.4.3 Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is greater than 1.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

If verticalEdgeFlag is equal to 1, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]

Otherwise (verticalEdgeFlag is equal to 0), the following applies.

- The parameters ks and ke are derived as follows.

–	If xPOS is equal to 1, the parameters ks and ke are set to −1 and 2 respectively.

–	Otherwise if xPOS is equal to 2, the parameters ks and ke are set to 0 and 2 respectively.

–	Otherwise, ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]

The variables log2TrafoWidthQ and log2TrafoHeightQ are set equal to the width and the height of the transform unit that includes the chroma sample value q0,0. The variable log2TrafoSizeQ is set equal to (log2TrafoWidthQ + log2TrafoHeightQ) / 2. The variable SizeIDQ is set equal to (log2TrafoSizeQ – 2).

The variables log2TrafoWidthP and log2TrafoHeightP are set equal to the width and the height of the transform unit that includes the chroma sample value p0,0. The variable log2TrafoSizeP is set equal to (log2TrafoWidthP + log2TrafoHeightP) / 2. The variable SizeIDP is set equal to (log2TrafoSizeP – 2).

If the variable s’ represents the chroma sample array recPictureCb of the current picture:

- If the chroma sample q0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDQ is set equal to 1; otherwise 4.

- If the chroma sample p0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDP is set equal to 1; otherwise 4.

If the variable s’ represents the chroma sample array recPictureCr of the current picture:

- If the chroma sample q0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDQ is set equal to 2; otherwise 5.

- If the chroma sample p0,0 is in a coding unit coded with intra prediction mode, the variable MatrixIDP is set equal to 2; otherwise 5.

The variable QPQ,QM is set equal to scaling_list_deblocking[SizeIDQ][MatrixIDQ].

The variable QPP,QM is set equal to scaling_list_deblocking[SizeIDP][MatrixIDP].

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

The variable QPQ is specified in Table 8-11 based on the index qPI that is set equal to QPY of the coding unit containing the sample q0,0, plus the variable QPQ,QM.

The variable QPP is specified in Table 8-11 based on the index qPI that is set equal to QPY value of the coding unit containing the sample p0,0, plus the variable QPP,QM.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	When pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- When pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

The variable QPC is specified in Table 8-6 based on an index qPI derived as follows:

qPI = ( ( QPQ + QPP + 1 ) >> 1 )

A variable QPC is derived as follows:

QPC = ( ( QPQ + QPP + 1 ) >> 1 )

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) ) as input.

….

- Table 8-6 – Specification of QPC as a function of qPI

| qPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | = qPI | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).