| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0282_r1 |
| --- | --- |

| Title: | AHG6: Lossless Coding and Deblocking Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

A CU is losslessly coded if the qpprime_y_zero_transquant_bypass_flag in the SPS is equal to 1 and if QP′Y is equal to 0 for the CU. The problem is that the QPL quantity used by the deblocking filter results in too weak deblocking filtering for the boundary discontinuity between a lossy and losslessly coded region. It is proposed to assign the QPY value of the lossy side of the boundary to QPL. It is demonstrated that the deblocking quality of the boundary is improved. The HM6.1 anchor is reproduced under common test conditions.

# Problem Statement

During the 8th JCT-VC meeting in San Jose, a lossless coding mode was adopted into the HEVC committee draft (CD) [1]. The qpprime_y_zero_transquant_bypass_flag in the SPS equal to 1 specifies that if QP′Y is equal to 0, then a lossless coding process shall be applied. In lossless coding operation, the scaling and transform process and the in-loop filter processes are bypassed. Currently, the lossless coding mechanism is integrated into the HM6.1 software.

In the CD text, the luma quantization parameter QP′Y is defined as follows (see section 8.6.1):

QP’Y = QPY + QpBdOffsetY								(Eq. 1)

with QpBdOffsetY = 6 * bit_depth_luma_minus8.

If the bitdepth is 8 bit then QpBdOffsetY equals 0, or if the bitdepth is 10 bit then QpBdOffsetY equals 12. The range of QPY is -QpBdOffsetY to 51 and the range of QP’Y is 0 to (51 + QpBdOffsetY).

According to the CD text [1], the in-loop deblocking filter will skip processing of the current CU if QP’Y=0 and qpprime_y_zero_transquant_bypass_flag=1. However, if the current CU is surrounded by CUs that are lossy coded (QP’Y>0), then the deblocking filter will skip processing the left and top edges of the current CU while the right and bottom edges may be deblocking filtered, which is illustrated in Figure 1(a). The problem is that the deblocking filter modifies the lossless samples along the right and bottom edges. This behavior was not intended and, therefore, the Tickets #472 and #476 were issued to fix the bugs in the HM and in the CD text. The corrected deblocking filtering along the losslessly coded CU boundaries is illustrated in Fig. 1(b).

The deblocking filter parameters, β and tC, are determined by QPL, which is the average of the QPY values on both sides of the current edge between blocks P and Q:

QPL = ( QPY,P + QPY,Q + 1) >> 1								(Eq. 2)

In case the P block side of the edge is losslessly coded (QP’Y,P=0 or QPY,P=- QpBdOffsetY), then the following QPL is computed:

QPL = ( -QpBdOffsetY + QPY,Q + 1) >> 1							(Eq. 3)

The problem is that this QPL value is too small in case the lossy samples adjacent to the losslessly coded CU are modified as is illustrated in Fig. 1(b), and as a result the deblocking filtering strength will be too weak.

QP’Y>0

QP’Y>0

QP’Y>0

QP’Y>0

Lossless

QP’Y=0

QP’Y>0

(a)

QP’Y>0

QP’Y>0

QP’Y>0

(b)

Lossless

QP’Y=0

Figure 1(a) Current HM6.1 deblocking of the edges of a losslessly coded CU. Bug fixes were issued in Tickets #472 and #476 to obtain the intended deblocking filtering in (b), which modifies the samples adjacent to the losslessly coded CU while leaving the lossless CU samples unmodified.

# Proposal

As illustrated in Fig. 1(b), the deblocking filter can modify the samples that are adjacent to the losslessly coded CU while leaving the lossless CU samples unmodified. In this case, the QPL value for looking up the deblocking filter parameters β and tC needs to be modified, because the current value (see Eq. 3) is inadequate for providing visually pleasing integration of the losslessly coded CUs into the surrounding lossy coded region. The proposed solution is to use the QPY value of the CU that is on the lossy coded side of the edge between blocks P and Q as is illustrated in Table 1. This is equivalent to determining the maximum of the QPY values of the CUs containing blocks P and Q. The following pseudo-code can be used to obtain the proposed QPL result based on Eq. 2, without the need for determining special conditions (lossless mode determination is already available):

if( CU_Q->isLosslessCoded(PartQIdx) )

{

QP_Q = 0;

QP_P <<= 1;

}

if( CU_P->isLosslessCoded(PartPIdx) )

{

QP_P = 0;

QP_Q <<= 1;

}

QP_L = (QP_P + QP_Q + 1) >> 1;

Table 1 QPL for deblocking filtering the current edge between blocks P and Q as a function of lossless mode of both blocks

| Is block P in lossless CU? | Is block Q in lossless CU? | QPL for deblocking |
| --- | --- | --- |
| Yes | No | QPY,Q |
| No | Yes | QPY,P |
| No | No | ( QPY,P + QPY,Q + 1 ) / 2 |
| Yes | Yes | No deblocking of edge |

Fig. 2 illustrates the proposed deblocking filtering for an edge between a lossy coded region and a lossless coded region in a frame of the “PartyScene” sequence. It can be observed that the edge discontinuity between both regions is smoothed by the proposed derivation of the QP for determining deblocking strength.

| (a) | (b) |
| --- | --- |

Figure 2 (a) Boundary discontinuity between losslessly coded and lossy coded region is clearly visible. (b) The proposed method for determining the appropriate QP for deblocking filtering of the boundary dissolves the boundary. (PartyScene sequence)

# Objective Results

Lossless coding is not selected in the common test conditions described in JCTVC-H1100. In this experiment, the source code is evaluated and it is shown that the HM6.1 anchor is reproduced (Table 2). Encoding and decoding timing are performed in a cluster computing environment.

Table 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

# Conclusion

The visual quality of the deblocking filtering result for the boundary between a lossy and losslessly coded region benefits from assigning the QPY value of the CU on the lossy side of the boundary to the QPL quantity, which determines deblocking strength.

# References

[1]  B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, CA, USA, Feb. 2012.

# HEVC Draft Text Changes

The following HEVC draft text changes are based on the text provided with HM Text Ticket #492 on “Text cleanup of QP prediction / derivation”:

##### 8.7.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

If verticalEdgeFlag is equal to 1, the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = recPictureL[ xC + xB – i − 1, yC + yB + k ]		(8-2)

Otherwise (verticalEdgeFlag is equal to 0), the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +k, yC + yB + i ]		(8-3)

pi,k = recPictureL[ xC + xB +k, yC + yB – i − 1 ]		(8-4)

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	When pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- When pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

- If qpprime_y_zero_transquant_bypass_flag is equal to 1, the variables QPQ and QPP are further modified as follows:

- If QP’Y of the coding unit containing the sample q0,0 is equal to 0, QPQ is set equal to 0 and QPP is set equal to (QPP << 1);

- If QP’Y of the coding unit containing the sample p0,0 is equal to 0, QPP is set equal to 0 and QPQ is set equal to (QPQ << 1).

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )		(8-5)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

….

##### 8.7.1.4.3 Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is greater than 1.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

If verticalEdgeFlag is equal to 1, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-6)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-7)

Otherwise (verticalEdgeFlag is equal to 0), the following applies.

- The parameters ks and ke are derived as follows.

–	If xPOS is equal to 1, the parameters ks and ke are set to −1 and 2 respectively.

–	Otherwise if xPOS is equal to 2, the parameters ks and ke are set to 0 and 2 respectively.

–	Otherwise, ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-8)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-9)

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	When pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- When pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

If qpprime_y_zero_transquant_bypass_flag is equal to 1, the variables QPQ and QPP are further modified as follows:

- If QP’Y of the coding unit containing the sample q0,0 is equal to 0, QPQ is set equal to 0 and QPP is set equal to (QPP << 1);

- If QP’Y of the coding unit containing the sample p0,0 is equal to 0, QPP is set equal to 0 and QPQ is set equal to (QPQ << 1).

The variable QPC is specified in Error: Reference source not found based on an index qPI derived as follows:

qPI = ( ( QPQ + QPP + 1 ) >> 1 )		(8-10)

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) ) as input.

….

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).