| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0283_r1 |
| --- | --- |

| Title: | AHG6: Chroma QP Offsets and Chroma Deblocking Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cb_qp_offset and cr_qp_offset syntax elements are signalled in the PPS and specify the offset to the luma quantization parameter used for deriving the chroma QP values. The HEVC deblocking filter for the chroma components derives the chroma filtering strength without taking into account the cb_qp_offset and cr_qp_offset values, which can significantly change the chroma QP values for coding and, therefore, the filtering strength of chroma blocking artifact edges can be too weak or too strong. To resolve this issue, it is proposed to include the cb_qp_offset and cr_qp_offset values into the deblocking filtering process for chroma. The HM6.0 anchor is reproduced under common test conditions. The chroma deblocking strength correction is illustrated.

# Problem Statement

The HEVC Committee Draft (CD) [1] includes the signalling of offset values for the quantization parameters (QP) that are used for coding the chrominance components Cr and Cb. The cb_qp_offset and cr_qp_offset syntax elements are signaled in the PPS and their semantics are as follows:

“cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr, respectively.”

The HEVC deblocking filter [1] processes the reconstructed chrominance components. In particular, an edge between two blocks P and Q is processed in case at least one of these blocks is intra coded (boundary strength Bs=2).  The deblocking filter typically looks up the tC parameter in a table based on the QPC value that is derived as follows:

QPC = QpUV ( (QPY,P + QPY,Q + 1)/2 ),					(1)

with QpUV a table that converts the luma QPY into the chroma QPCr or QPCb values. The tC parameter is applied in the clipping operation of the chroma filter and determines the filtering strength. The QPC value is identical for both chroma components Cr and Cb.

The problem is that in equation (1) the chroma QP offset values, cb_qp_offset and cr_qp_offset, are not considered and since the range of these offset values is significant (-12 … 12), the deblocking filter may apply a deblocking strength to the chrominance components that is either too weak or too strong, which results in reduced chroma fidelity.

# Proposal

It is proposed to include the cr_qp_offset and cb_qp_offset values in the computation of the QPC values that are used to look up tC parameters for deblocking filtering the Cb and Cr components, as follows:

QPCr = QpUV ( (QPY,P + QPY,Q + 1)/2 + cr_qp_offset ),					(2)

QPCb = QpUV ( (QPY,P + QPY,Q + 1)/2 + cb_qp_offset ).					(3)

Subsequently, separate tC values for Cb and Cr are looked up in a table based on the computed QPCr and QPCb (constant 2 is added for Bs=2).

Fig.1 illustrates the quality improvement based on a test image with radial gradients. The image is coded according to the Intra-Main test conditions with QP=27 and cr_qp_offset = cb_qp_offset = 12. Fig. 1(a) is obtained with HM6.0 and Fig. 1(b) with the proposed correction for chroma deblocking. In Fig. 1(a) chroma blockiness is present while in Fig. 1(b) the chroma blockiness is reduced, because the QPC value 39 is used to determine filtering strength. Fig. 2 depicts a second example from the same test image.

| (a) | (b) |
| --- | --- |

Figure 1

| (a) | (b) |
| --- | --- |

Figure 2

# Objective Results

The proposal has no impact on BD-rate results under common test conditions (JCTVC-H1100), because configuration file parameters ChromaQpOffset and ChromaQpOffset2nd are equal to 0. In the performed test, the source code is checked against the anchor. The results are identical to the HM6.0 anchor, as illustrated in Table 1. Encoding and decoding times are measured in a cluster computing environment.

Table 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion

It iss proposed to include the cr_qp_offset and cb_qp_offset syntax element (PPS) values into the derivation of the QPC values that are used to configure the deblocking filtering strength for the chroma block boundaries. This way too weak or too strong chroma filtering is avoided in case cr_qp_offset or cb_qp_offset are nonzero.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, CA, USA, Feb. 2012

# HEVC Draft Text Changes

The following CD text changes are based on the document attached to HM Text Ticket #492.

##### 8.7.1.4.3 Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is greater than 1.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

If verticalEdgeFlag is equal to 1, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-2)

Otherwise (verticalEdgeFlag is equal to 0), the following applies.

- The parameters ks and ke are derived as follows.

–	If xPOS is equal to 1, the parameters ks and ke are set to −1 and 2 respectively.

–	Otherwise if xPOS is equal to 2, the parameters ks and ke are set to 0 and 2 respectively.

–	Otherwise, ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-3)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-4)

The variables QPQ and QPP are set equal to the QPY values of the coding units containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	When pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- When pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

The variable QPC is specified in Error: Reference source not found based on an index qPI derived as follows:

- If s’ represents the chroma sample array recPictureCb of the current picture:

- qPI = ( ( ( QPQ + QPP + 1 ) >> 1 ) + cb_qp_offset ).

- Otherwise, if s’ represents the chroma sample array recPictureCr of the current picture:

- qPI = ( ( ( QPQ + QPP + 1 ) >> 1 ) + cr_qp_offset ).

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) ) as input.

….

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).