| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27th April – 7th May, 2012 | Document: JCTVC-I0285-r1 |
| --- | --- |

| Title: | Elimination of clipping before dequantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Joel Sole Xiang Li Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In the 8th JCTVC meeting in San Jose, USA, a clipping was introduced before dequantization step to avoid 32-bit overflow in internal calculations. The clipping range is 16 bits or less depending on whether quantizer matrices are being used and the transform size. In this document, two methods to eliminate the need for clipping are proposed. In the first method, instead of normalizing the quantizer matrix entries by 16, normalization by 32 is proposed. This means that quantization matrix entry t effectively scales the quantizer step-size by a factor of (t/32) instead of (t/16). In the 2nd method, it is proposed to restrict the amount of left shift in the dequantization step when quantizer matrices are being used. Furthermore, modification to the decoding process for scaled transform coefficients is proposed to make the decoding process with or without quantization matrices similar.

# Introduction

In the HEVC committee draft, the level values for transform coefficients are clipped before dequantization step. When no quantization matrices are present, the clipping range is 16 bits. In presence of quantization matrices, the clipping range may be 16 bits or lower depending on the transform size. The meeting notes for 8th JCTVC meeting in San Jose, USA state the following in the discussion of [1]: “Adopt (text in H0312 [2]) as a placeholder to solve the problem, further study whether this is the ultimate solution in quantization AHG.” It is further mentioned in the meeting notes that this problem arises only when quantization matrix is being used. But the working draft imposes a clipping operation before dequantization step with or without quantization matrices. Thus, currently, there are 3 normative clips in the dequantization and transform operations on the decoder side, thereby making the design more complex. We propose that the clipping step before dequantization should eliminated. In this document we propose two different methods to accomplish this. We assume an encoder restriction is imposed so that for a compliant bitstream, the transform coefficient level values shall be restricted to 16 bits. First we introduce the current method in HEVC committee draft.

## Notation

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

N 	= transform size

M	= log2(N)

levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5

M[i][j]	= 8-bit unsigned scaling list matrix entries

Furthermore, let c[i][j] and d[i][j] be the quantized coefficient values and dequantized coefficient values respectively. In the HEVC working draft,

shift = (B + M – 9)

shiftScale = (B + M – 9 + 4 – (QP/6)).

When quantization matrices are not present,

y[i][j]=Clip3(−32768, 32768, c[i][j]),

d[i][j]= (( y[i][j] * levelScale[ QP%6] << ( QP/6 )) + ( 1 << ( shift - 1 ))  )>>  shift,

Otherwise if (shiftScale > 0)

y[i][j]=Clip3(−32768, 32768, c[i][j]),

d[i][j]=  ((y[i][j]*M[i][j]*levelScale[QP%6])+ (1<<(shiftScale-1)))>>shiftScale,

Otherwise

LevelLimit = 1 << Min( 15, 12 + B + M -(QP/6)) ,

y[i][j]=Clip3(−LevelLimit, LevelLimit-1, c[i][j]),

d[i][j]=(y[i][j]* M[ i ][ j ]*levelScale[ QP%6 ]) << ( -shiftScale ).

# Proposal

## Method 1

We propose a change in the normalization factor for the quantizer matrix entries from 16 to 32. This means that for a quantizer matrix entry t, the quantizer step-size is effectively scaled by (t/32) instead of (t/16). There are only two changes necessary in the decoding process. The first one is that the clip before quantization is removed for all cases (with or without quantization matrices). It should be stressed that we still assume that for a compliant bitstream, the transform coefficient level values should be restricted to 16 bits. The second one is that the derivation of shiftScale is changed to

shiftScale = (B + M – 9 + 5 – (QP/6)).

As noted in [1] and [2], for internal calculations, 33 bits are needed in the worst case when quantization matrices are used and transform coefficient level values are restricted to 16 bits. The above change brings the internal calculations back to 32 bits.

If it is desired to use the same quantization matrices as being used currently, the base QP can be increased by 6 to get exactly identical results. This is not possible only when the base QP has a value of 46 or higher. In such a case, instead of changing the base QP, the entries in the current quantization matrix can be doubled.

## Method 2

As noted in [1], when transform coefficient level values are restricted to 16 bits, 32-bit overflow for internal calculations in the dequantization step occurs when quantizer matrices are used and when shiftScale is less than 0. This happens when M=2, shiftScale = -3. The 32-bit overflow happens because of the left shift by –shiftScale in the “otherwise” clause. We propose restricting the left shift to a maximum of 2. This is accomplished be changing the “otherwise” clause to

d[i][j] = c[i][j] * M[i][j] * levelScale[QP%6] << min(2, -shiftScale)

Please note that in this case, the clipping before the dequantization step is eliminated. It is also necessary to modify the downshift after the first transform stage as follows.

gij = Clip3( −32768, 32767, ( eij + 64 ) >> downshift_stage1 )	,

where downshift_stage1 is derived as follows:

–	If scaling_list_present_flag is equal to 0 or shiftScale is greater than or equal to -2,

downshift_stage1 = 7,

–	Otherwise,

downshift_stage1 = 7 + (shiftScale + 2).

In the proposed method, one additional comparison is introduced when shiftScale is less than or equal to 0 and another comparison is introduced after the first transform stage. At the same time the comparison to determine whether the clipping before dequantization step should be to 16 bits or lower is eliminated. All these comparisons need to be performed once per TU. Additionally, the clipping before the dequantization step is eliminated. This saving is for every transform coefficient.

## Harmonization of decoding process with and without quantization matrices for Method 1

When quantization matrices are used, there are two paths for deriving the scaled transform coefficients depending on the value of shiftScale. But in the absence of quantization matrices, derivation of scaled transform coefficients involves a left shift as well as a right shift. We propose using similar processing for the two cases as follows:

The variable shift is derived as follows:

If scaling_list_present_flag is equal to 0,

shift = B + M – 9 – (QP/6).

If (shift > 0)

d[i][j]=  Clip3( -32768, 32767, ((c[i][j]*levelScale[QP%6]) + (1<<(shift-1)))>>shift),

Otherwise

d[i][j]=Clip3(−32768,32767,(c[i][j]* levelScale[ QP%6 ]) << ( -shift ))

Otherwise

Shift = B + M – 9 + 5 – (QP/6).

If (shift > 0)

d[i][j]= Clip3( -32768, 32767, ((c[i][j]* M[i][j] * levelScale[QP%6]) + (1<<(shift-1)))>>shift),

Otherwise

d[i][j]=Clip3( -32768, 32767, (c[i][j]* M[i][j] *levelScale[ QP%6 ]) << ( -shift ))

# Conclusions

We have proposed two different methods of removing the clipping before the dequantization step. In the first method, instead of normalizing the quantizer matrix entries by 16, normalization by 32 is proposed. This means that quantization matrix entry t effectively scales the quantizer step-size by a factor of (t/32) instead of (t/16). In the 2nd method, it is proposed to restrict the amount of left shift when quantizer matrices are being used. Furthermore, modification to the decoding process for scaled transform coefficients is proposed to make the decoding process with or without quantization matrices similar.

# References

[1] L. Kerofsky, A. Segall, “Non CE4: Limiting dynamic range when using a quantization weighting matrix”, JCTVC-H0541, San Jose, USA, February 2012.

[2] X. Li, X. Guo, S. Lei, “Non-CE4: Clipping of Transformed Coefficients before De-Quantization”, JCTVC-H0312, San Jose, USA, February 2012.

# Committee Draft text change

### 8.6.2 Scaling and transformation process

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij. It is a requirement of bitstream conformance that the values of cij fall within the range -32768 and 32767.

The variable log2TrSize is derived as follows:

log2TrSize = ( Log2( NW ) + Log2( NH ) ) >> 1		(8-1)

–	If cIdx is equal to 0,

BitDepth = BitDepthY

–	Otherwise,

BitDepth = BitDepthC

The variable shift is derived as follows:

–	If scaling_list_present_flag is equal to 0,

shift = BitDepth + log2TrSize − 9 – QP / 6 		(8-x)

–	Otherwise,

shift = BitDepth + log2TrSize − 9 + 5 – QP / 6 		(8-x)

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The elements of array M[ i ][ j ] with i = 0..nW − 1, j = 0..nH − 1 are set equal to ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error: Reference source not found and Equation Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-x)

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

If shift > 0

dij = Clip3( −32768, 32767,( ( cij * levelScale[ qP%6 ]) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-x)

Otherwise,

dij = Clip3( −32768, 32767, ( cij *  levelScale[ qP%6 ] ) << ( -shift ) )

–	Otherwise

If shift > 0

dij = Clip3( −32768, 32767,( ( cij * M[i][j] * levelScale[ qP%6 ]) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-x)

Otherwise,

dij = Clip3( −32768, 32767, ( cij M[i][j] * levelScale[ qP%6 ] ) << ( -shift ) )

Method 2:

### 8.6.3: Scaling process for transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij. It is a requirement of bitstream conformance that the values of cij fall within the range -32768 and 32767.

The variable log2TrSize is derived as follows:

log2TrSize = ( Log2( NW ) + Log2( NH ) ) >> 1		(8-2)

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = BitDepthY + log2TrSize − 9		(8-3)

levelLimit = 1 << Min( 15, 12 + BitDepthY + log2TrSize – QP / 6 ) 	(8-4)

–	Otherwise,

shift = BitDepthC + log2TrSize − 9		(8-5)

levelLimit = 1 << Min( 15, 12 + BitDepthC + log2TrSize – QP / 6 ) 	(8-6)

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The variable shiftScale is set equal to shift + 4 – QP/6.

The elements of array M[ i ][ j ] with i = 0..nW − 1, j = 0..nH − 1 are set equal to ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error: Reference source not found and Equation Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-7)

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

yij = Clip3( −32768, 32767, cij )		(8-8)

dij = Clip3( −32768, 32767,( ( cij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-9)

–	Otherwise, if shiftScale is greater than 0,

yij = Clip3( −32768, 32767, cij )		(8-10)

dij = Clip3( −32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] + 					( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-11)

–	Otherwise,

yij = Clip3( -LevelLimit, LevelLimit − 1, cij )		(8-12)

dij = Clip3( −32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( min(-shiftScale, 2) ) )	(8-13)

### 8.6.4Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i=0..nW−1, j=0..nH−1) is transformed to eij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found according to the width of the transform unit nW, with the (nW)x(nH) array d and the transform type variable horizTrType as the inputs and the output is the (nW)x(nH) array e.

- The intermediate sample values gij (i=0..nW−1, j=0..nH−1) are derived by

gij = Clip3( −32768, 32767, ( eij + 64 ) >> downshift_stage1 )		(8-14)

where downshift_stage1 is derived as follows:

–	If scaling_list_present_flag is equal to 0 or shiftScale is greater than or equal to -2,

downshift_stage1 = 7,

–	Otherwise,

downshift_stage1 = 7 + (shiftScale + 2).

- Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).