| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0286 |
| --- | --- |

| Title: | Cross-verification of Simplification of Multiple Sign Bit Hiding (JCTVC-I0156) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yong Yu Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-713-1345 yongyu@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JCTVC-I0156. Having a fixed threshold for the number of coefficients for the MBS process seems to reduce the complexity of the decoder and encoder, in both software and hardware design and verification. The proposed fixed threshold seems to be an optimal value across different categories of tests. The cross checker supports adopting the fixed threshold for number of coefficients in multiple sign bit hiding.

# Introduction

HM6.0 and committee draft of HEVC allows different threshold transmission for Multiple Sign Bit Hiding. JCTVC-I0156 proposed to make the threshold a fixed value. This contribution confirms that the software is implemented as the document describes and the experimental results match the cross checker’s result.

# Experimental results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  | 106% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 91% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 107% |  |  |
| Dec Time[%] | 89% |  |  | 90% |  |  |

# Note that the enc/dec time is not reliable. The tested data is generated from a cluster with processors of different configurations and possibly suspension of other higher priority jobs.

# Conclusion

Having a fixed threshold for the number of coefficients for MSB seems to reduce the complexity of the decoder and encoder, in both software and hardware design and verification. The proposed fixed threshold seems to be an optimal value across different categories of tests. The cross checker supports adopting the fixed threshold for number of coefficients in sign bit hiding.

# References

[1] JCTVC-I0156, “Simplification of Multiple Sign Bit Hiding”, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.